3GPP TSG-RAN WG4 Meeting # 112bis					R4-2415456

Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			On system parameter for WUR

Agenda item:		6.23.2.1

Document for:		Approval

# Introduction

In this paper, we present our overview on the system parameter for WUR RF requirement perspective.

# Discussion

In WF[1], the channel raster for WUR is not decided.

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS.

  - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

In TR 38.869, the WUS carrier/BW is defined in Figure 7.1.2.1-1 (quoted below). So it is the part of RB region within the MR transmission bandwidth. It can be observed that the WUS BW will occupy a range of the RB within the MR RB grid.

In latest RAN2 agreement, the LP-WUS will be configured in system information for RRC idle and inactive and this means UE can locate the LP-WUS when the SIB is obtained.

### Procedure and configuration of LP-WUS in RRC_IDLE INACTIVE

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. (e.g. dedicated signalling to enable/disable LP-WUS)

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	FFS on Entry/exit condition for LP-WUS monitoring (chair: RRC config is not a possible entry/exit condition, i.e. full coverage/OFDM unfriendly)

From the above observation, the WUR will not be configured with a channel and therefore channel raster concept will no apply to WUR. Network needs to assure the LP-WUS signal should be transmitted together with other NR signals using the same iFFT modulator, this also means the LP-WUS signal position should be referenced with the serving cell RB grid.

RAN2 agrees to configure LP-WUS in SIB.

Channel raster does not apply to WUR.

# Conclusions

In this contribution, we present our view on system parameter for WUR with below proposal:

Observation 1 RAN2 agrees to configure LP-WUS in SIB.

Proposal-1: Channel raster does not apply to WUR.

# References

- R4-2414309, WF on LP-WUS UE RF requirements, vivo