# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415509

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.19.3.2

Source:	Apple

Title:	On RRM Requirements for MIMO Phase 5

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112 the RRM requirements impact was discussed for MIMO phase 5 and WF [1] was approved. In this contribution we discuss RRM requirements impact for objectives other than UE initiated beam management.

# 2. Discussion

In RAN4#112 the RRM core requirements impact was agreed as follows:

In RAN#105 the WID for MIMO Ph5 was revised [2] and additional objectives were added. We also include the new objectives in the RRM requirements impact analysis.

CSI enhancements for up to 128 CSI-RS ports

For supporting > 32 TX ports RAN1 shall specify the following:

Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

It was already agreed that there was no impact to RRM requirements with introduction of enhancements to Type I, Type II codebook and extension of CRI to support up to 128 ports.

In [2] a new objective for SRS port grouping was introduced. The enhancement is to specify SRS port grouping for 2 CW for low complexity receiver. This enhancement is not expected to have impact on RRM requirements.

No impact to RRM requirements with SRS port grouping for 2CW with low complexity receiver.

CJT enhancements

Issue 2-3-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM core requirements impact. - Option 2: RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting. Issue 2-3-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM performance requirements impact. - Option 2: No accuracy requirements. Potential mapping tables are needed if any. - Option 3: Define accuracy requirements. - Option 4: Define accuracy requirements and report mapping tables.

The offset measurements would follow the reporting delay for aperiodic measurement report. Hence, we don’t see any RRM core requirements impact with CJT enhancements.

No impact to RRM core requirement impact with CJT calibration reporting enhancement.

For CJT deployments RAN1 is introducing CJT calibration reporting – delay offset, frequency offset, phase offset reporting. The reporting range for the offsets are:

Frequency Offset: [0 , AFO]; AFO={0.1ppm, 0,2ppm}

Delay Offset: [0 , AD]; AD={0.5CP, CP}

Phase: [0 , 2];

The reporting range and resolution for the reported quantities are very small. It would not be feasible to define any meaningful accuracy requirements for the newly introduced offset reporting for CJT. RAN4 can study the feasibility of defining accuracy requirements for phase, frequency, delay offset in performance part of the WI.

The delay, frequency and phase offset range and resolution are small to define any accuracy requirements.

- Study feasibility of defining accuracy requirements for frequency/delay/phase offset for CJT calibration in performance part of WI.

RAN4 could potentially define mapping tables for the delay quantities of not covered in RAN1 spec in performance part.

- RAN4 to consider defining mapping tables for frequency/delay/phase offset reporting in performance part of WI.

Non-coherent UL codebook for 3-antenna port transmission

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

For 3TX support RAN1 is introducing non-coherent UL codebook. Here is no RRM impact to introduction of 3TX codebook.

No impact to RRM core requirement impact with 3TX codebook enhancement.

In RAN#105 the WID was revised to include additional enhancements for 3T6R SRS antenna switching and additional UE capability for 3T3R antenna switching.

RAN4 has introduced requirements in R17 for SR antenna switching for xTyR when SRS is in the last 6 symbols of the slot.  Those should also be applicable to 3T6R case.

RAN4 has introduced requirements in Rel-17 for SRS antenna switching. The same requirements shall be applicable to 3T6R when SRS is in the last 6 symbols of the slot.

Asymmetric DL sTRP/UL mTRP

Revised objective in [2]:

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Open issues from [1] include:

Issue 2-5-3: Whether RRM impacts exist by 2TA enhancement?

<Way forward>

- FFS on whether there are RRM impacts by 2TA enhancement.

In Rel-18 RAN4 introduced timing requirements for 2TA for multi-DCI multi-TRP transmission scheme. Impacts for timing requirements for 2 TA with asymmetric DL sTRP/ UL mTRP needs to be further discussed in RAN4.

RAN4 has introduced timing requirements in Rel-18 for 2 TA with mDCI mTRP transmission scheme.

RAN4 should discuss impacts to timing requirements with 2 TA with asymmetric DL sTRP/ UL mTRP .

In Rel-18 with 2 TA there was a reference point for each TAG associated with the DLorJoint TCI state with same TAG as UL signal. With asymmetric DL sTRP/ UL mTRP there is only 1 DL TRP, hence only a common reference point is possible for both TAGs.

With asymmetric DL sTRP/ UL mTRP only 1 DL TRP and only common reference point is feasible for this case.

- RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

Since RAN1 has not discussed the framework for 2 TA with asymmetric DL sTRP/ UL mTRP, RAN4 should wait for further progress in RAN1 to understand if there is any other impact to requirements.

- RAN4 to discuss any other impacts to RRM requirements with 2 TA enhancement based on RAN1 progress

Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

<Way forward>

- Option 1: The legacy requirements can be reused.

- Option 2: The legacy requirements cannot be reused.

- Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios

To assess the impacts to TCI state switching requirements with asymmetric DL sTRP/ UL mTRP we need to wait for progress in RAN1/ RAN2 if any on the TCI state configuration. RAN4 to further discuss impact based on progress in RAN1.

- RAN4 to discuss any other impacts to TCI state switching requirements with asymmetric DL sTRP/ UL mTRP based on RAN1 progress

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

CSI enhancements for up to 128 CSI-RS ports

- No impact to RRM requirements with SRS port grouping for 2CW with low complexity receiver.

CJT enhancements

- No impact to RRM core requirement impact with CJT calibration reporting enhancement.

The delay, frequency and phase offset range and resolution are small to define any accuracy requirements.

- Study feasibility of defining accuracy requirements for frequency/delay/phase offset for CJT calibration in performance part of WI.

- RAN4 to consider defining mapping tables for frequency/delay/phase offset reporting in performance part of WI.

Non-coherent UL codebook for 3-antenna port transmissions

No impact to RRM core requirement impact with 3TX codebook enhancement.

RAN4 has introduced requirements in Rel-17 for SRS antenna switching. The same requirements shall be applicable to 3T6R when SRS is in the last 6 symbols of the slot.

Asymmetric DL sTRP/UL mTRP

RAN4 has introduced timing requirements in Rel-18 for 2 TA with mDCI mTRP transmission scheme.

RAN4 should discuss impacts to timing requirements with 2 TA with asymmetric DL sTRP/ UL mTRP .

With asymmetric DL sTRP/ UL mTRP only 1 DL TRP and only common reference point is feasible for this case.

- RAN4 to update the reference point definition for 2 TA with asymmetric DL sTRP/ UL mTRP.

- RAN4 to discuss any other impacts to RRM requirements with 2 TA enhancement based on RAN1 progress

- RAN4 to discuss any other impacts to TCI state switching requirements with asymmetric DL sTRP/ UL mTRP based on RAN1 progress

# Reference

- R4-2414035, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung.

- RP-242394, “Revised WID: NR MIMO Phase 5”, Samsung.