3GPP TSG-RAN WG4 Meeting #112bis                                 R4-2415522

Hefei, Anhui, China, 14 – 18 October 2024

Source: 	Apple

Title: 	RRM requirement for ATG

Agenda Item:	6.10.4

Document for:	Discussion

# Introduction

ATG RRM requirement was extensively discussed in the last RAN4 meeting, and the open issue are captured in WF [1]

This contribution presents our further views on the requirement impact due to introduction of CA for ATG.

# Discussion

## RRC_IDLE/ RRC_INACTIVE state mobility

The following open issues were captured in [1] and are for further study.

Issue 1-2-1: Cell re-selection Idle/Inactive mode CA measurements

- Proposal 1: RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA.

- Proposal 2: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

Issue 1-2-2: Idle/Inactive mode CA measurements (R16)

- Option 1: Introduce idle/inactive mode CA measurements requirements for ATG UE in Rel-19

- Option 2: No need to introduce idle/inactive mode CA measurements requirements in ATG CA.

Issue 1-2-3: Measurement report for fast CA/DC setup (R18)

- Option 1: RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline

- Option 2: No need to introduce Rel-18 NR CA EMR in ATG CA.

- Option 3: Consider whether to define the requirements of measurement report for fast CA setup in idle/inactive mode for R19 ATG UE

For cell re-selection Idle/Inactive mode CA measurements, Option 1 and Option 2 are not conflicts with each other. The legacy NR cell re-selection can be reused for ATG as we didn’t identify any specific aspects for ATG. At the same time, we just follow RF decision to update the side condition exception for ATG UE.

Proposal 1: Reuse legacy NR cell re-selection for ATG supporting CA and follow the RF agreement to update the exception of side condition for ATG UE supporting inter-band CA.

Issue 1-2-2 and Issue 1-2-3 are intended for fast CA/DC establishment after RRC connection setup for handheld UE. Regarding ATG UE, it is intended for in-flight eMBB service. There is little chance for ATG UE to go to RRC_IDLE mode since we can assume that there is always data connection demand from some passengers. It’s hard to see that all passengers go to seep even in red-eye flight. Further as we are considering co-located deployment only for ATG, network could derive coarse SCell quality by PCell quality and known frequency gap information as mentioned in [2].

Proposal 2: Idle/Inactive mode CA measurements and measurement report requirement (EMR feature) seems not necessary for ATG supporting CA.

## Timing requirements

Issue 1-3-1: MTTD

- Option 1: FFS whether MTTD requirement is needed or not

Issue 1-3-2: MRTD

Agreement:

- For ATG intra-band contiguous CA, not define MRTD requirement.

- For ATG inter-band CA, MRTD requirement is:

  - Option 1: A little larger than 3us, i.e., 3.545 µs

  - Option 2: 33us

  - Option 3: 3us

  - Option 4: further discuss

The following open issues were captured in [1] and are for further study.

Regarding MTTD, it is only specified for inter-band CA and intra-band non-contiguous CA for non-collocated scenario in section 7.5.4 for handheld UE. For ATG UE, only collocated scenario is considered for both intra-band contiguous CA_n79C and inter-band CA_n3-n39. We propose to skip MTTD requirement for intra-band case and only specify MTTD for inter-band CA. the derivation of MTTD requirement should be based on MRTD requirement with some time error budget from Rx to Tx.

Proposal 3: It is proposed to skip MTTD requirement for ATG intra-band contiguous CA and define MTTD requirement for ATG inter-band CA based on MRTD and addition timing error margin.

Regarding MRTD, although the current scenario is inter-band CA with collocated condition, we think it is still beneficial to define the requirement in more generic way to avoid too restrictive number given inter-band CA will anyway consider different RF chain to cover different band. Hence, we propose 33us as a generic MRTD requirement for ATG UE.

Proposal 4: It is proposed define MRTD requirement for ATG inter-band CA as 33us.

## Signaling characteristics

The below table summarized our view on signalling characteristics for ATG UE.

Table 2.3-1 signaling characteristics for ATG UE

| Sub-clause | Requirement | Comments |
| --- | --- | --- |
| 8 | Signaling characteristics |  |
| 8.1 | Radio link monitoring |  |
| 8.2 | Interruption |  |
| 8.2.2 | Introduction | Some adaption on the requirement applicability is needed depending on the supported feature by ATG UE. |
| 8.2.2.2.1 | Interruption at Scell addition/release | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.3 | Interruptions during measurements on deactivated SCC | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.4 | Interruptions at UL carrier RRC reconfiguration | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. SUL part should be deleted. |
| 8.2.2.2.5 | Interruptions due to Active BWP switching Requirement | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. |
| 8.2.2.2.6 | Interruptions at inter-frequency SFTD measurement | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. |
| 8.2.2.2.7 | Interruptions at SCell activation/deactivation with multiple downlink SCells | Not applicable to ATG UE due to single Scell in Rel-19. |
| 8.2.2.2.8 | Interruptions due to UE-specific CBW change | Can be completely reused for ATG UE. |
| 8.2.2.2.9 | Interruptions at NR SRS carrier based switching | Can be reused for ATG UE. |
| 8.2.2.2.10 | DL Interruptions at UE switching between two uplink carriers | UL carrier switching for handheld UE was motivated mainly by - Implementation difficulty to support too many chains - For some band combination (e.g. FDD in low band and TDD band in high band), it is beneficial to switch the chain in low band to a high band to achieve high data rate by using MIMO. This seems to be a solution for handheld UE with good tradeoff between implementation difficulty and performance gain. While it will introduce interruptions due to switching which is also not a best solution from performance perspective. While for ATG, it seems not a big issue to implement as many chains as needed so that the best performance without interruption can be achieved. Further for the example band of CA_n3-n39, there is little gain to do so as they are so close in frequency. Hence, we think it’s not necessary to consider this feature for ATG. |
| 8.2.2.2.10A | DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10B | DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10C | DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10D | DL Interruptions at UE switching across three or four uplink bands | Same comment as for 8.2.2.10. |
| 8.2.2.2.11 | Interruptions at direct SCell activation | Can be reused. |
| 8.2.2.2.12 | Interruptions due to SCell dormancy | Can be reused. |
| 8.2.2.2.13 | Interruptions at transitions between active and non-active during DRX | Not applicable |
| 8.2.2.2.14 | Interruptions when identifying CGI of an NR cell with autonomous gaps | Seems not useful for ATG UE. |
| 8.2.2.2.15 | Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps | Not applicable |
| 8.2.2.2.16 | Interruptions at NR SRS antenna port switching | Can be reused. |
| 8.2.2.2.17 | Interruptions at fast SCell activation | Can be reused. |
| 8.2.2.2.18 | Interruptions due to PUCCH SCell activation/deactivation | Can be reused. |
| 8.2.2.2.19 | Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18] | Can be reused for ATG. |
| 8.2.2.2.20 | Interruptions due to PDCCH ordered RACH on target LTM cell | ATG cell switching is not so frequent. And LTM feature is still in the process of optimizing and being enhanced. We think it’s not necessary to complicate the ATG mobility at this stage. We propose to skip this feature for the time being. If there is real demand for this feature, can be reconsidered in future release when LTM feature is stable for TN. |

# Summary

This contribution presented some analysis on ATG UE RRM requirements in the aspects of Idle Mode CA/DC, MRTD/MTTD, RRC_INACTIVE state CA/DC Measurement, interruptions for various features. The following proposals are concluded.

Proposal 1: Reuse legacy NR cell re-selection for ATG supporting CA and follow the RF agreement to update the exception of side condition for ATG UE supporting inter-band CA.

Proposal 2: Idle/Inactive mode CA measurements and measurement report requirement (EMR feature) seems not necessary for ATG supporting CA.

Proposal 3: It is proposed to skip MTTD requirement for ATG intra-band contiguous CA and define MTTD requirement for ATG inter-band CA based on MRTD and addition timing error margin.

Proposal 4: It is proposed define MRTD requirement for ATG inter-band CA as 33us.

Proposal 5: It is proposed to consider signalling characteristics as summarized in Table 2.3-1.

# References

[1] R4-2413959, WF on RRM requirements for NR_ATG_enh, CMCC, RAN4#112

[2] R4-2411758, (NR_ATG_enh-Core) Overview of RRM core requirements for R19 ATG, CMCC, RAN4#112