3GPP TSG-RAN WG4 Meeting #112bis	R4-2415561

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	On NTN UE OTA test

Source:	Samsung

Agenda item:	6.12.2.2

Document for:	Discussion

1.	Introduction

Usage scenarios and performance metric for NTN UE OTA test have been extensively discussed in previous meetings.

For usage scenario, some progress was made in last meeting [1, R4-2413603]

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE Agreements: - Test method development consider hand only and head+hand, RAN4 will discuss requirement applicability rules based on device type, NR-NTN/IoT-NTN support, or other criteria Prioritize hand only and head+hand - FFS down-selection of usage scenarios for requirements - Other usage scenarios are not precluded Issue 3-1-5: Usage scenarios for IoT-NTN handheld UE Agreements: - Same prioritized usage scenarios as NR-NTN handheld UE

For performance metric, companies’ views are still divergent.

In this contribution, we further discuss usage scenarios and performance metric.

2. 	Discussion

It was agreed that test method development will consider hand only and head+hand and RAN4 will discussion requirement applicability. Note that some NTN UE may be only applicable for hand only testing as it may only support message text or support voice call with lour speaker only. It is reasonable that such UEs are only required and tested with hand only mode.

Proposal 1:	NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode.

For performance metrics, there was no agreement in last meeting, so the candidate options remain as following [1, R4-2409931]

Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

Given the progress on user scenarios that both hand only and head+hand are considered in test method development, it is highly possible that a NTN handheld UE supporting voice call would be tested in both hand only and head+hand modes. It is very challenging to satisfy the two modes with same antenna performance for a certain direction, because different phantom will lead to different antenna radiation pattern and directivity.

Observation 1:	Given both hand only and head+hand considered in test method, only a full sphere metric is appropriate to meet the requirements for both modes at the same time.

On the other hand, the UE in field for FR1 actually does not receive signals from single direction. We should not simply consider only a single direction for UE to receive satellite signals, but should also consider reflected and scattered signals which may from any directions.

Moreover, the handheld NTN UE can be in various orientations. If directional requirements are specified, users have to always take the UE in specific orientation for better signal even in idle mode. It seems not possible for a portable devices like mobile phone.

Observation 2:	a handheld UE may receive satellite signals from any directions, and users may hold the UE in any orientation.

So the performance metric should consider all directions in full sphere. It is probably the upper hemisphere includes more incident satellite signal than the lower sphere, different weights for upper hemisphere and lower hemisphere can be considered.

Proposal 2:	Adopt full sphere TRP TRS as performance metric for NTN handheld UE, and larger weights and be put on the declared hemisphere, e.g., upper hemisphere, in the TRP TRS integration.

3. 	Conclusion

Proposal 1:	NTN UEs not supporting voice call or only supporting voice call with loud speaker etc. are only required and tested with hand only mode.

Observation 1:	Given both hand only and head+hand considered in test method, only a full sphere metric is appropriate to meet the requirements for both modes at the same time.

Observation 2:	a handheld UE may receive satellite signals from any directions, and users may hold the UE in any orientation.

Proposal 2:	Adopt full sphere TRP TRS as performance metric for NTN handheld UE, and larger weights and be put on the declared hemisphere, e.g., upper hemisphere, in the TRP TRS integration.

4. 	References

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo

- R4-2409931    “WF for [111][338] TRP_TRS_MIMO_OTA”, vivo