3GPP TSG-RAN WG4 Meeting #112-Bis	R4-2415723

Hefei, China, Oct. 2024

Source	:	Qualcomm Incorporated

Title	:	On UE Rx requirements for the LPWUR

Agenda item	:	6.23.2.2

Document for	:	Approval

# 1.	Introduction

In this contribution we first share our views on open items pertaining to the core requirements. We also identify development direction considering plenary confirmation about scope of WI and FR2. Finally, we share some preliminary SNR vs MDR simulation results while noting that RAN1 definition of LPWUS has not progressed sufficiently for RAN4 to complete threshold SNR.

# 2. 	Discussion

## 2.1	On core requirements and receiver topology

In a previous WF [4], it was agreed to…..:

Differentiate NF for OOK-based and OFDM-based receiver.

This differentiation will naturally lead to different performance expectations based on receiver topology. The functional underlying difference in the topologies is frequency selectivity – coherent receivers are well suited to rejecting close-by OFDM RBs, but typically, energy detectors must rely on filtering and perhaps, guard RBs. The effectiveness of an energy detector-based receiver to withstand adjacent RB jamming is therefore variable and cannot be presumed to be sufficient.

Observation 1: It is not easy to justify skipping ASCS requirements for envelope-detector-based LPWUR.

This argument can be extended to ACS and other jammers also. A more general conclusion is that overall performance is a strong function of receiver topology.

Proposal 1: Core requirements for LPWUS functionality are developed separately for envelope (OOK) detectors and coherent (OFDM) detectors, including ASCS.

## 2.2	Idle vs connected mode

Our studies [1] have shown that FR2 connected mode operation stands more to gain significant benefit from this feature relative to FR1 promise. Moreover, there is a pathway for further benefit if LPWUS functionality is enhanced to include more RRM functionality. Now, RAN4 UE RF requirements are expected to be invariant of idle or connected mode. Given the promise of better savings in connected mode, the side conditions may be better constructed to reflect both modes. The primary difference for connected mode UEs is the dependence on SSB rather than LPSS for time, frequency and AGC functionality. Both modes would be protected if the core requirement was specified in the presence of both LPSS as well as SSB.

Proposal 2: RAN4 core requirements are specified with both LPSS and SSB being provided to the UE.

## 2.3	SNR definition

Simulations to project required SNR based on signal details depend on the definition of SNR. The need to define SNR was captured in a WF [4]:

- Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first.

SNR definition is important to establish because for LPWUS there is potential for ambiguity due to OOK modulation of underlying OFDM. RAN1 makes no special ‘adjustment’ to account for the 50% duty cycle for Manchester coded bits, so it is evident that they are looking at the SNR in a long-term sense. This is translated to SNR over the chip pair that comprises a Manchester-coded bit. With Manchester coding, the noise power is accrued over both, the ‘ON’ and ‘OFF’ phases, but the signal power is only accrued over the ‘ON’ phase.

Another detail is reference plane for definition of SNR. RAN4 use a legacy formula of the form:

Sensitivity = -174dBm + 10*log(RxBW) + NF + SNR + IM

Since RAN4 derives the sensitivity expectation of a receiver by adding the impact of NF to some threshold SNR, it is self-evident the receiver is abstracted into an RF-part (NF impact) and a baseband part (SNR), in addition to an ‘implementation margin’. This baseband block is notionally everything after the last down-conversion in the agreed baseline topology [4]. For OFDM receivers it is common to include the impact of loss of orthogonality, like time and frequency adjustment into the SNR calculation.

Proposal 3: In context of legacy formula to derive sensitivity, the SNR of the FR1 LPWUR is measured at the baseband input.

Proposal 4: The SNR for the FR1 LPWUR is the ratio of the total signal energy in one ‘ON’ chip and one ‘OFF’ chip, to the total noise energy over the same period, in the BW occupied by the LPWUS, where the chip pair corresponds to a single Manchester coded bit.

## 2.4	FR2 applicability

### 2.4.1	WID scope

Clarification in the RAN#105 [3] confirms that FR2 is in scope:

- Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Moreover, there is expected to be strong overlap in LPWUS design between FR1 and FR2, and no extra time is deemed necessary. It is therefore prudent for RAN4 to start constructing link-level simulation assumptions for the FR2 scenario. Some details for FR2 are not yet certain like number of RBs and QCL relations, but some of these aspects also apply to 15k SCS in FR1 (for example, number of RBs). Moreover, RAN1 still has not finished defining the LPWUS even for FR1, for example what fraction of the LPWUS sequence is known to the UE and what fraction is information. These details can be left open in RAN4 with the understanding that parallel development in RAN1 will fill in these gaps, and a complete set of assumption will be available for RAN4#113.

Observation 2: Neither FR1 nor FR2 have sufficient LPWUS definition to allow detailed baseband SNR vs MDR simulations in RAN4.

### 2.4.2	FR1/FR2 coverage comparison

The appendix contains a comparative coverage analysis across FR1 and FR2, based on values recorded in TR38.830 (coverage enhancement TR). The main observation is that FR1 and FR2 bands are similar from an implementation margin perspective because their respective UL and DL coverages scale similarly enough. i.e. no special procedure is required for FR2

Observation 3: FR1 and FR2 are equivalent from the perspective of designing a LP-WUR with sufficient coverage, and no unique consideration is required for FR2.

### 2.4.3	FR2 receiver topology

Compared to FR1, FR2 channels are considered less scattering-rich, and this necessitated the use of a dual-pol Rx as the building block of any FR2 receiver. For legacy PDSCH reception cases, the dual pol architecture resulted in the UE being able to realize the full antenna gain in a pol. agnostic sense.

For coherent FR2 LPWUR, this serves as a good precedent. Specifically, for the sake of deriving requirements, the LPWUR can be assumed to be 2 Rx type, and arranged in dual pol fashion.

For OOK-based receivers, the benefit from combining across polarizations is highly dependent on implementation detail, and from a RAN4 point of view, without recent precedent. For deriving requirements for OOK-based receivers then, it may be better to assume a 1Rx type of receiver and rely on averaging sensitivity outcomes across polarizations per legacy procedure. This averaging would imply a 3 dB greater implementation loss for 1-Rx FR2 receivers compared to 2-Rx receivers.

A further open issue is whether the receiver uses a refined beam vs a single element beam. Given the similarity in FR1 and FR2 (section 2.4.2 and annex) with a single-element beam assumption for FR2, it seems reasonable to use the latter as a baseline

Proposal 5: The baseline FR2 coherent (OFDM) LPWUR has the following features:

- It comprises a 2-chain dual pol. receiver with legacy PDSCH cross-pol combining assumption

- Antenna gains are assumed to be those of a single element

Proposal 6: The baseline FR2 energy detection-based (OOK) LPWUR has the following features:

- It comprises a 1 Rx receiver

- Antenna gain is assumed to be that of a single element

### 2.4.4	FR2 SNR definition

Section 2.3 treated the FR1 case, which is agreed as 1Rx. FR1 treatment is easily extended to FR2 OOK or envelope-detector-based LPWUR, since they are also proposed above to be 1Rx. Since testing is in the radiated domain, some implementation margin may be necessary as described in 2.4.3.

Observation 4: For an FR2 LPWUR with 1 Rx, the SNR definition for FR1 LPWUR can be reused.

For an FR2 OFDM LPWUR per proposal 5, there are 2 baseband SNRs to resolve, one per polarization-specific chain. Legacy practice for PDSCH means separate demodulation and MRC across the chains. This process can be abstracted into a single LPWUR baseband SNR whose abstracted single RF front end is able to receive DL signal regardless of polarization mismatch.

Observation 5: For an FR2 OFDM receiver per proposal 5, the LPWUR can be abstracted into a 1Rx receiver with an antenna that is pol. agnostic.

Proposal 7: The SNR definition for FR1 LPWUR is reused for FR2. In case the FR2 LPWUR is 2Rx, the definition applies after abstraction into a 1Rx receiver using an agreed combining assumption.

### 2.4.5	FR2 LLS assumptions

A good starting point is the set of assumptions agreed for FR1 [2]. Some aspects need to be revisited, both to reflect progress in RAN1, as well as unique aspects to FR2. The FR2 set is shown below with highlighted parameters that change from FR1 agreements.

Proposal 8: RAN4 to adopt the following link level simulation assumptions as starting point for the FR2 use case:

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16 bits |  |  | (placeholder for RAN1 decision) |
| Channel structure |  |  |  | not needed |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | 0 (up to UE implementation) |  |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 5MHz (11 RB)-case 3 |  |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [16~17] dB NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | 7.68MHz |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | Not modelled |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional … |  |  | not needed anymore, see WF agreement [4] |

## 2.5	Preliminary link sim results for LPWUS sensitivity

### 2.5.1	FR1 SNR

As previously noted in the arguments to support obs. 2, the LPWUS definition is not yet complete in RAN1, and any MDR vs SNR performance projections can only be preliminary. The MDR curves presented below are for a coherent receiver in FR1and a target FAR of 1%, assuming the UE knows a-priori the entire sequence of length ‘B’ (number of bits). The FAR information is just for reference, ultimately this parameter is strictly up to UE implementation. Practical timing and frequency error are assumed. AWGN and TDLA-300 channel are compared – for TDLA-300, the SNR curves are not what manifest at the UE, but instead they are ‘apparent SNRs’ from the gNB’s perspective. They are plotted on the same axis to represent how much worse the UE appears from the gNB’s perspective due to additional fading in the channel.

Figure 2.5.1-1: FR1 MDR for FAR of 1%, coherent Rx

Some observations for a coherent LPWUR:

- Curves for M=1 and M = 4 show the expected 10*log10(M) relationship

- Comparing curves for different sequence length, the longer sequence length shows better sensitivity (can be decoded at lower SNR)

- Apparent SNR curves for TDLA300 show about 7 dB degradation at the target MDR due to channel fading, or a range reduction factor of 2.2

Considering that the SNR requirement for MCS4 is -1 dB, it is probably overkill for coherent LPWURs to be more sensitive (able to decode at lower SNR) than the MR’s PDSCH.

### 2.5.2	FR2 SNR

Figure 2.5.2-1 shows SNR projection for FR2, paralleling effort for FR1 in the previous subsection, but limited to AWGN channels.

Figure 2.4.2-1: FR2 MDR for FAR of 1%, coherent Rx

Observations largely mirror those in the FR1 exercise and for AWGN channels, reinforcing the similarity aspect.

# 3.	Conclusion

Observation 1: It is not easy to justify skipping ASCS requirements for envelope-detector-based LPWUR.

Proposal 1: Core requirements for LPWUS functionality are developed separately for envelope (OOK) detectors and coherent (OFDM) detectors, including ASCS.

Proposal 2: RAN4 core requirements are specified with both LPSS and SSB being provided to the UE.

Proposal 3: In context of legacy formula to derive sensitivity, the SNR of the FR1 LPWUR is measured at the baseband input.

Proposal 4: The SNR for FR1 LPWUR is the ratio of the total signal energy in one ‘ON’ chip and one ‘OFF’ chip, to the total noise energy over the same period, in the BW occupied by the LPWUS, where the chip pair corresponds to a single Manchester coded bit.

Observation 2: Neither FR1 nor FR2 have sufficient LPWUS definition to allow detailed baseband SNR vs MDR simulations in RAN4.

Observation 3: FR1 and FR2 are equivalent from the perspective of designing a LP-WUR with sufficient coverage, and no unique consideration is required for FR2.

Proposal 5: The baseline FR2 coherent (OFDM) LPWUR has the following features:

- It comprises a 2-chain dual pol. receiver with legacy PDSCH cross-pol combining assumption

- Antenna gains are assumed to be those of a single element

Proposal 6: The baseline FR2 energy detection-based (OOK) LPWUR has the following features:

- It comprises a 1 Rx receiver

- Antenna gain is assumed to be that of a single element

Observation 4: For an FR2 LPWUR with 1 Rx, the SNR definition for FR1 LPWUR can be reused

Observation 5: For an FR2 OFDM receiver per proposal 5, the LPWUR can be abstracted into a 1Rx receiver with an antenna that is pol. agnostic.

Proposal 7: The SNR definition for FR1 LPWUR is reused for FR2. In case the FR2 LPWUR is 2Rx, the definition applies after abstraction into a 1Rx receiver using an agreed combining assumption.

Proposal 8: RAN4 to adopt the following link level simulation assumptions as starting point for the FR2 use case:

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16 bits |  |  | (placeholder for RAN1 decision) |
| Channel structure |  |  |  | not needed |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | 0 (up to UE implementation) |  |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 5MHz (11 RB)-case 3 |  |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [16~17] dB NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | 7.68MHz |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | Not modelled |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% Providing the information whether the false alarm rate is considered or not |  |  | not needed anymore, see WF agreement [4] |

# 4.	References

[1] 	R4-2408032, ‘On LPWUR system considerations’, Qualcomm, RAN4#111, May 2024

[2] 	R4-2410569, ‘WF for [111][136] NR_LPWUS_UERF’, vivo, RAN4#111, May 2024

[3] 	RP-242361, ‘Way Forward on LP-WUS for FR2’, vivo, RAN#105, Sept. 2024

[4] 	R4-2414309, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#112, Aug 2024

[5] 	R4-2413223, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#112, Aug 2024

# 5.	Appendix: Coverage analysis (comparison b/w FR1 and FR2)

There have been some concerns regarding LP-WUS coverage in FR2, if supported. However, the requirement is to meet the msg-3 PUSCH coverage in the same carrier/FR. For FR2, msg-3 PUSCH coverage is smaller compared to FR1 in general.

Figure 15: LP-WUS and Msg-3 PUSCH coverages in different FRs

Below, we provide an analysis of LP-WUS coverage with respect to FR2. Firstly, let us consider analysis for FR1 (2.6GHz). As per TR38.830, following parameters are used. Occupied bandwidth is assumed to be 11 PRBs with SCS 30kHz for LP-WUS.

Table A-1.	Parameters to derive LP-WUS MIL for FR1 (2.6GHz)

| Parameter | Value |
| --- | --- |
| BS number of antenna elements | 192 |
| UE number of antenna elements | 1 |
| Power spectrum density | 33dBm per MHz |
| Occupied bandwidth | 3.96 MHz |
| System bandwidth | 95 MHz |
| Antenna element gain | 8dBi for BS Tx 5dBi for UE Rx |
| Thermal noise density | -174dBm/Hz |

From the parameters, we can derive EIRP, receiver gain, received signal level, as follows

EIRP=psdPerMhz+10⋅log10occBw+dBiTx+10⋅log10numTxEl=33+10⋅log104.32+8+10⋅log10192=70.2dBm

rxGain=sdBiRx+10⋅log10numRxEl=5+10⋅log101=5dB

sigLevRx=-174+NFDL+10⋅log10occBw+SNRsensitivityDL WUS=-174+NFDL+10⋅log103.96⋅106+SNRsensitivityDL WUS

where NFDL and SNRsensitivityDL WUS are noise figure and sensitivity of LP-WUS. From them, LP-WUS MIL can be derived as

MILLPWUS=EIRP+rxGain-sigLevRx-impLossDL=183.2-NFDL-SNRsensitivityDL WUS-impLossDL

where impLossDL is an implementation loss. The condition that LP-WUS coverage matches msg-3 PUSCH coverage can be expressed as

MILLPWUS≥MILMsg3

RAN1 agreed to use MILMsg3=153.51dB from TR38.830 Table 5.1.1.2-2 at the RAN1#116bis meeting. Putting these together, we can rewrite the condition as follows:

MILLPWUS-MILMsg3=29.7 dB-SNRsensitivityDL WUS+NFDL+impLossDL ≥0

SNRsensitivityDL WUS is the required SNR to achieve the target performance of LP-WUS and is dependent on LP-WUS design. NFDL and impLossDL are dependent on the LP-WUR implementation.

Next, we can do the same for FR2. As per TR38.830, following parameters are used. Occupied bandwidth is assumed to be 12 PRBs with SCS 120kHz for LP-WUS.

Table A-2.	Parameters to derive LP-WUS MIL for FR2

| Parameter | Value |
| --- | --- |
| BS number of antenna elements | 128 (per polarization) |
| UE number of antenna elements | 4 (per polarization) |
| Power spectrum density | 40dBm per system BW |
| Occupied bandwidth | 15.84MHz |
| System bandwidth | 95MHz |
| Antenna element gain | 8dBi for BS Tx 5dBi for UE Rx |
| Thermal noise density | -174dBm/Hz |

From the parameters, we can derive EIRP, Rx gain, Rx signal level, as follows

EIRP=psdSysBw+10⋅log10occBwsysBw+dBiTx+10⋅log10numTxEl=40+10⋅log1015.8495+8+10⋅log10128=61.3dBm

rxGain=sdBiRx+10⋅log10numRxEl=5+10⋅log104=11dB

sigLevRx=-174+NFDL+10⋅log10occBw+SNRsensitivityDL WUS=-174+NFDL+10⋅log1015.84⋅106+SNRsensitivityDL WUS

From them, LP-WUS MIL can be derived as

MILLPWUS=EIRP+rxGain-sigLevRx-impLossDL=174.3-NFDL-SNRsensitivityDL WUS-impLossDL

The condition that LP-WUS coverage matches msg-3 PUSCH coverage can be expressed as

MILLPWUS≥MILMsg3

Same as for FR1, MILMsg3 can refer to the value captured in TR38.830 Table 5.2.1.1-2, i.e., MILMsg3=139.72dB. From the above, we can rewrite the condition as follows:

MILLPWUS-MILMsg3=34.6 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0

Note here that the same beam shapes were assumed at the UE for receiving LP-WUS as well transmitting msg 3. A worse situation for WUS reception is if the UE uses less refined beams for it. In this case there would be a 6 dB reduction in FoM, calculated originally as 34.6 dB above

Table A-3

| FR | The condition that LP-WUS coverage matches or exceeds Msg-3 coverage |
| --- | --- |
| FR1 | 29.7 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0 |
| FR2 | 28.6 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0 (*) (*) assuming rough beam for LPWUS detection |

Observation: The coverage criteria (i.e. implementation challenges) looks very similar across FR1 and FR2