3GPP TSG-RAN WG4 Meeting #112bis		R4-2415776
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.12.2.2

Source:	vivo

Title:	Further discussions on IoT-NTN and NR-NTN OTA test method

Document for:	Approval

1	Introduction

This contribution provides our further views on NTN OTA test method and performance metric.

2	Discussion

RAN4 reach consensus on usage scenarios for NR-NTN and IoT-NTN devices and prioritized UE types, in WF [1]:

Issue 3-1-1: Usage scenarios for NR-NTN mobile handheld UE

Agreements:

  - Test method development consider hand only and head+hand, RAN4 will discuss requirement applicability rules based on device type, NR-NTN/IoT-NTN support, or other criteria Prioritize hand only and head+hand

    - FFS down-selection of usage scenarios for requirements

  - Other usage scenarios are not precluded

Issue 3-1-4: UE type for IoT-NTN

Agreements:

  - For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority.

Issue 3-1-5: Usage scenarios for IoT-NTN handheld UE

Agreements:

  - Same prioritized usage scenarios as NR-NTN handheld UE

However, the performance metric discussion for NR-NTN and IoT-NTN is still diverged. According to the discussions last meeting, given head+hand has been agreed as a key usage scenario, then the traditional TRP TRS performance metric should also be considered, especially for the UEs not support a beam tracking application/function.

Proposal 1: For hand only usage, partial sphere TRP/TRS or partial sphere CDF is sufficient. FFS the range of angles.

Proposal 2: For head+hand usage, additionally, full TRP/TRS should also be measured.

Then, the test method is nearly the same as traditional procedure for smartphone, the only difference would be how to perform the post-processing of all the EIRP/EIS value.

Observation 1: The test method and procedure for NTN is similar to smartphone, the key difference would be the data processing.

In last RAN4 meeting, the detailed test parameters for NR-NTN have been concluded. However, the IoT-NTN test parameters are still under discussion.

Issue 3-3-3: test parameters for IoT-NTN

Agreements:

  - RAN4 should further discuss test parameters for IoT-NTN OTA.

Issue 3-3-4: How to resolve UE antenna circular polarization issue

Agreements:

  - FFS RAN4 how to accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

For NB-IoT NTN OTA testing, the following parameters are conducted test cases in TS 36.521-4.

Table 6.2B.1.4.1-1 (NB MOP) in TS 36.521-4: Test Configuration Initial Conditions for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@3 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. NOTE 2: only applicable for low range. NOTE 3: only applicable for high range. |  |  |  |  |

Table 7.3B.4.1-1 (NB REFSENS) in TS 36.521-4: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | QPSK | 12 | BPSK | 1@0 | 15kHz |

According to typical configuration of 15kHz sub-carrier spacing for NB-IoT, QPSK with 1@0 Ntones (configuration ID 3) can be selected as test parameters to verify IoT-NTN TRP performance.

Proposal 3: For NB IoT-NTN, select QPSK with 1@0 Ntones as TRP-related test parameter, QPSK with 12 Subcarriers as TRS-related test parameters.

In addition, category M1 is also a typical configuration for IoT-NTN, RAN4 should also specify the test parameters for UE support cat M1 NTN.

Table 6.2A.1.4.1-1 (cat M1 NTN MOP): Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |
|  |  |  |  | HD-FDD |
| 1.4MHz |  |  | QPSK | 1 |
| 1.4MHz |  |  | QPSK | 2 |
| NOTE 1: Test Channel Bandwidth is checked separately for each E-UTRA satellite access band, the applicable channel bandwidths are specified in Table 5.3A-1. NOTE 2: The 1 RB allocation shall be tested at RB#0 with narrowband index 0 for low and mid range, RB #5 with max narrowband index for high range test frequency. NOTE 3: The RBstart of non-1RB allocation shall be RB #0 with narrowband index 0 for low and mid range, RB# (6 - RB allocation) with max narrowband index for high range test frequency. |  |  |  |  |

Table 7.3A.4.1-1 (cat M1 NTN REFSENS): Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |
|  |  | FDD |  |  | FDD and HD-FDD |
| 1.4MHz | QPSK | 4 |  | QPSK | 6 |

Proposal 4: For Cat M1 NTN, select QPSK with 1RB allocation as TRP-related test parameters, QPSK with 4 RB allocation as TRS-related test parameters.

3 Conclusion

In this paper, we provide our further views on NTN OTA.

Proposal 1: For hand only usage, partial sphere TRP/TRS or partial sphere CDF is sufficient. FFS the range of angles.

Proposal 2: For head+hand usage, additionally, full TRP/TRS should also be measured.

Observation 1: The test method and procedure for NTN is similar to smartphone, the key difference would be the data processing.

Proposal 3: For NB IoT-NTN, select QPSK with 1@0 Ntones as TRP-related test parameter, QPSK with 12 Subcarriers as TRS-related test parameters.

Proposal 4: For Cat M1 NTN, select QPSK with 1RB allocation as TRP-related test parameters, QPSK with 4 RB allocation as TRS-related test parameters.

4	References

- R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA, vivo, RAN meeting #111, May 2024.

- TS 38.508-1

- TS 36.521-4

- TS 38.521-5