3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415841

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on Measurements related enhancements for LTM

Source: 	Huawei, HiSilicon

Agenda item:	6.24.2.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on Measurements related enhancements for LTM.

Discussion

Inter-CU LTM

The first objective is to enable LTM operation between cells of different gNBs (i.e. inter-CU).  This objective is regarding high layer inter-CU interaction. One sub-objective is to support for subsequent LTM mobility procedures in inter-CU scenario.

The background is that subsequent LTM mobility procedure has been supported for intra-CU in RAN2. However no requirements are specified for subsequent LTM in R18 in RAN4.

From technical analysis perspective, referring to the requirements of Subsequent Conditional PSCell Change delay (in clause 8.11E.2 in TS38.133) specified in R18, the starting point of the delay is from the instance when UE transmits RRCReconfigurationcomplete message for the previous PSCell addition or change.  It means the RRC configuration and RRC processing time are not included in the subsequent conditional PSCell change delay. Likewise, inter-CU or intra-CU operation would be no impact on subsequent LTM cell switching delay. Regarding whether to specify requirements for subsequent LTM cell switching delay in R19, we are open to further discuss, although the function has been supported since R18 intra-CU scenario.

Proposal 1: RAN4 to discuss whether to define requirements for subsequent LTM cell switching delay in R19. The requirements would be the same for both intra-CU and inter-CU.

Event trigger reporting

In R18 LTM event triggering of L1 measurement results is not supported, so only periodic reporting, semi-persistent reporting and aperiodic reporting are specified in measurement reporting requirement. The R19 mobility enhancement is to support event triggered L1 measurement reporting. There are some progress in RAN2 during the last several meeting cycles.

Agreements on measurement enhancements for LTM:

1. Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility

It is straight forward that the “Event Triggered Reporting” would be introduced in section 9.14.3 and section 9.15.3 for LTM L1-RSRP measurement requirements in R19.

Proposal 2: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3.

It is agreed in RAN2 that it’s up to RAN1 whether the specified L1 filtering is needed or to leave it to UE implementation. In our understanding, as one shot sample is applied for CSI-RS based L1-RSRP in serving cell, to ensure the fairness between the RSRP level from candidate cells and that of serving cell, L1 filtering on neighbour cell is NOT expected. Moreover the gain from early L1 measurements is to enable network quick acquire the measurement results, introduction of L1 filter would vanish the gain. Anyway as the issue will be further discussed in RAN1, RAN4 can wait for more RAN1 progress.

Proposal 3: Wait for more RAN1 progress on L1 filtering.

As beam level measurement result is used and any beam in candidate RS configuration can be used for LTM event evaluation, the accuracy of beam level L1-RSRP from candidate cell shall be guaranteed. The existing CSI-RS based L1-RSRP accuracy in serving cell is specified in clause 10.1.19.2. As the beam level RSRP from serving cell and candidate cell will be compared as per Event LTM-x, the accuracy shall be the same for both sides. Therefore the same accuracy is supposed to be reused for CSI-RS based L1-RSRSP from candidate cell. Moreover to ensure the same accuracy, the CSI-RS bandwidth is not less than 48 PRBs and density is 3.

Unless otherwise specified, the requirements for absolute accuracy of CSI-RS based L1-RSRP in this clause apply to all CSI-RS resources of the serving cell configured for L1-RSRP measurement. The accuracy requirements in Table 10.1.19.2.1-1 are valid under the following conditions: - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. - Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS. - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

Proposal 4: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells.

Proposal 5: For CSI-RS based L1-RSRP measurement on neighbor cells, define the requirements for {D=3, PRB≧48}.

CSI-RS measurements for LTM procedures

In last meeting, the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement was discussed. However due to lack of information of L1 CSI-RS resource measurement configuration on neighbor cell from RAN1 and RAN2, the topic didn’t get any consensus.

The legacy L3 CSI-RS resources are included in MeasObjectNR, and the CSI-RS resources are defined as cell-specific. In details, in one MO, there are multiple cells which have the same SSB frequency and the same SCS. In each cell, the L3 CSI-RS resources have the same CSI-RS center frequency, bandwidth and density.

Observation 1: For L3 CSI-RS resources which are included in MeasObjectNR, in each cell, the L3 CSI-RS resources have the same CSI-RS center frequency, bandwidth and density.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

<omit unrelated>

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

CSI-RS-ResourceConfigMobility ::=   SEQUENCE {

subcarrierSpacing                   SubcarrierSpacing,

csi-RS-CellList-Mobility            SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-CellMobility,

...,

[[

refServCellIndex                    ServCellIndex                                                           OPTIONAL    -- Need S

]]

}

CSI-RS-CellMobility ::=             SEQUENCE {

cellId                              PhysCellId,

csi-rs-MeasurementBW                SEQUENCE {

nrofPRBs                            ENUMERATED { size24, size48, size96, size192, size264},

startPRB                            INTEGER(0..2169)

},

density                             ENUMERATED {d1,d3}                                                      OPTIONAL,   -- Need R

csi-rs-ResourceList-Mobility        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility

}

Based on L3 CSI-RS resource signalling design, the L3 CSI-RS based intra-frequency measurement is specified as below, considering the same center frequency, SCS and CP type between serving cell and neighbour cell.

A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

For R19 L1 CSI-RS resource measurement configuration on neighbor cell, the below agreements were achieved in the past August RAN2 meeting.

5.   R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

As per above RAN2 signalling design philosophy, the R19 L1 CSI-RS measurement resources of candidate cells is most likely to reuse existing NZP-CSI-RS-Resource structure in LTM-config rather than in MeasObjectNR. The NZP-CSI-RS-Resource is duplicated as below.

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

It is observed that one candidate neighbour cell can have multiple L1 CSI-RS resources and the center frequency and bandwidth of these CSI-RS resources can be different.

Observation 2: There may have more than one L1 CSI-RS resources configured in each candidate neighbour cell, and the center frequency and bandwidth of these L1 CSI-RS resources can be different.

The L1 CSI-RS resource configuration in serving cell uses NZP-CSI-RS-Resource. It can be observed that there may have also more than one NZP-CSI-RS resources configured in the serving cell. One thing needs to be pointed out the SCS of CSI-RS resources is not present in NZP-CSI-RS-Resource. The SCS of CSI-RS resource of serving cell refers to the SCS of the active BWP. It means that the center frequency of CSI-RS resources configured in the serving cell may change with the active BWP.

Observation 3: There may have more than one L1 CSI-RS resources configured in the serving cell and the center frequency, bandwidth and SCS of a L1 CSI-RS resource would change with the active BWP switching.

According to L3 CSI-RS intra/inter -frequency definition, the center frequency of CSI-RS resource of neigbhour cell is compared with the center frequency of CSI-RS resource of serving cell where the center frequency of each CSI-RS resource in serving cell is fixed. The advantage of the L3 CSI-RS intra/inter -frequency definition is that a measurement is always regarded as an intra-f or inter-f measurement, and does not change with active BWP switching.

The situation seems somewhat different for R19 L1 CSI-RS measurement. According to observation 3, the center frequency of CSI-RS resource of serving cell (which is the reference point in intra-f definition) is changing along with active BWP switching. Therefore the advantage of defining intra-f measurement based on center frequency is lost. Then we think alternative based on active BWP can be considered.

If BW of NZP-CSI-RS resource(s) of a candidate cell is within DL active BWP of the serving cell (e.g., case 1 and case 2), no gap is needed as well. If BW of NZP-CSI-RS resource(s) of a candidate cell is not within DL active BWP of the serving cell (e.g., case 3 and case 4), gap is needed. Regarding case 5, although BW of NZP-CSI-RS resource(s) of a candidate cell exceeds active BWP of serving cell, UE may can intercept parts of CSI-RS resource of candidate cell. RAN4 can further discuss the definition of case 5.

Figure 1. Intra-f/inter-f definition for L1 CSI-RS measurement on candidate cells

Proposal 6: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

From RAN2 perspective, the flexible resource configuration can work well, however from UE implementation perspective, some applicability conditions shall be added.

In R16 L3 CSI-RS based measurement, applicability condition is defined as below. From UE implementation perspective, multiple FFT sizes would be required if UE is required to simultaneously receive CSI-RS resources with different bandwidth. So the same csi-rs-MeasurementBW for all CSI-RS resources in the same MO is added as a applicability condition. Furthermore based on the intra-frequency definition of L3 CSI-RS measurement, the centre frequency of the CSI-RS resource is the same between neighbor cell and serving cell. Therefore for L3 CSI-RS intra-frequency, the center frequency of CSI-RS resource and bandwidth in one MO is the same.

The requirements in this clause apply, provided: - Only one MO is configured per CSI-RS frequency layer, and - all CSI-RS resources in the same MO are configured with the same csi-rs-MeasurementBW, and - associatedSSB is configured in CSI-RS-Resource-Mobility and detectable, and - all CSI-RS resources in the same MO are configured with the same periodicity, and - the associated SSB is QCLed with the corresponding CSI-RS resources in FR2, and - the number of CSI-RS resources in any duration that equals to the length of a slot is no larger than UE capability maxNumberCSI-RS-RRM-RS-SINR. - When there are mixed numerologies, the length of a slot is defined based on the smallest SCS

For L1 CSI-RS measurement on candidate cell, to simply the requirements definition, the periodicity of CSI-RS resources are supposed to be the same. RAN4 can further discuss whether to limit the CSI-RS resource to be measured is the same as active BWP bandwidth.

Proposal 7: The applicability conditions are: all CSI-RS resources on candidate cells are configured with the same periodicity; FFS on whether to limit the CSI-RS resource to be measured is the same as active BWP bandwidth.

After RAN4 reach conclusion on the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, the supported measurement types can be discussed. If the intra-f definition follows proposal 6, it is straight forward that intra-frequency measure is performed without gap and inter-frequency is performed with gap.

Proposal 8: Prioritize intra-frequency without gap and inter-frequency with gap for CSI-RS based L1 measurement on neighbour cell.

Regarding the objective of supporting for CSI-RS measurements for LTM procedures, we think RAN4 shall first have an unified understanding on how CSI-RS based L1 measurement of candidate cells works.

UE first performs SSB based L3 measurement on the neighbour cells. Through SSB based L3 measurement, the coarse timing is obtained, and then UE can correctly receive configured CSI-RS resource of candidate neighbour cells.

Proposal 9: UE shall first perform SSB based L3 measurement on candidate neighbour cells.

For FR1, the procedure is straight forward. After UE correctly received configured CSI-RS resource of candidate neighbour cells, UE can directly measure on configured CSI-RS based L1-RSRP resources, as UE in FR1 has omnidirectional RX reception.

Proposal 10: In FR1, after UE correctly received configured CSI-RS resource of candidate neighbour cells, UE can directly measure on configured CSI-RS based L1-RSRP resources, as UE in FR1 has omnidirectional RX reception.

In FR2, it is a bit complicated. In last RAN# 102 meeting, two possible alternatives on how to perform CSI-RS based L1 measurement on neighbour cells were proposed. The comments from companies are preliminarily collected. Herein we would like to further share our views in the following.

Alternative 1: Firstly UE performs SSB based L3 measurements and acquire SSB index information on the candidate cells. Network and UE can acquire SSB index through SSB L3 measurement procedure. Then, UE perform L1 CSI-RS measurement with RX beam sweeping.

In our understanding, it is most likely that CSI-RS resources for LTM measurement are to be associated with SSB. It shall be noted that the associated SSB index can be obtained in UE side through L3 SSB measurement. Whether L1 CSI-RS resource is linked to the associated SSB needs further consideration. RAN1 may be responsible for the configuration. If the configuration is feasible, UE can sweep Rx beam with CSI-RS resources within a limited angle (within the associated SSB beam range). If so, the CSI-RS level finer Rx beam gain can be achieved. However if the associated SSB is not indicated (optional), the RX beam sweeping for CSI-RS measurement would be performed with a larger range. Then the RX beam gain will decrease.

In summary, alternative 1 of L1 CSI-RS measurement on candidate neighbour cell procedure is:

Step 1: L3 SSB based measurement Step 2: L1 CSI-RS measurement with RX beam sweeping - if associatedSSB/QCL-D is indicated for CSI-RS resource, refiner RX beam sweeping performs within SSB beam range, - otherwise Rx beam sweeping may be performed within larger range.

Although the alternative 1 seems straight forward, some drawbacks are observed:

1. With RX beam sweeping on CSI-RS resource of neighbour cell, the measurement delay of neighbour cell would be quite longer (N times) than that of serving cell where typical L1 measurement periodicity is 20ms for serving cell. As we know, in FR2 environment, the channel changes quickly. If UE is moving to the cell edge, the RSRP on serving cell would react channel degradation faster than that on neighbour cell. It is most likely that UE will switch to neighbour cell. However when “neighbour cell” become serving cell, RSRP fluctuation of the current serving cell will be intense, and UE may quickly switch back. In a word, different measurement periodicity between neighbour cell and serving cell may result in “ping-pong” handover.

2. As we know, there is no RX beam sweeping for L1 CSI-RS measurement on serving cell when repetition is OFF. The QCL-D in qcl-InfoPeriodicCSI-RS is SSB. UE uses SSB level RX beam to perform CSI-RS measurement. Compared to L1 CSI-RS measurement on neighbour cell with RX beam sweeping, the RX beam gain on serving cell and on neighbour cell is different. Therefore the RSRP comparison between serving cell and neighbor cell will be somewhat unfair.

Alternative 2: Another option is inherited from R15 CSI-RS based L1 measurement on serving cell. In R15, Rx beam sweeping is not applied to CSI-RS based L1 measurement for beam management when repetition is OFF. So there is no scaling factor (i.e., N=1) and UE RX beam uses SSB-level beam.

TS38.133 section 9.5.4.2 For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

If the above principle is followed, UE would not sweep the RX beam for CSI-RS resources on candidate cell. The procedure of CSI-RS based L1 measurement on candidate neighbour cell is that UE performs SSB based L3 measurement and acquires the SSB index information, and then UE performs SSB based L1 measurement to refine the RX beam. It shall be noted that the SSB based L1 measurement on candidate cells is the same as R18 LTM where UE sweeps RX beam based on SSB. Afterwards UE perform measurements on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping.

In summary, alternative 2 of L1 CSI-RS measurement on candidate neighbour cell procedure is:

Step 1: L3 SSB based measurement Step 2: L1 SSB measurement with RX beam sweeping (i.e., R18 LTM) Step 3: L1 CSI-RS measurement without RX beam sweeping (like R15 L1 CSI-RS measurement)

At first glance, alternative 2 has three steps. However the solution can ensure that the CSI-RS based L1 measurements on neighbour cell and serving cell are compared fairly. And these disadvantages observed from alternative 1 can be avoided.

As the above two alternatives lead to different UE implementation and requirements definition, RAN4 shall decide which direction would go with.

Proposal 11: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss the below alternatives:

Alternative 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and performs L1 CSI-RS measurement with Rx beam sweeping on candidate cells.

Step 1: L3 SSB based measurement Step 2: L1 CSI-RS measurement with RX beam sweeping - if associatedSSB/QCL-D is indicated for CSI-RS resource, refiner RX beam sweeping performs within SSB beam range, - otherwise Rx beam sweeping may be performed within larger range.

Alternative 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE performs measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement.

Step 1: L3 SSB based measurement Step 2: L1 SSB measurement with RX beam sweeping (i.e., R18 LTM) Step 3: L1 CSI-RS measurement without RX beam sweeping (like R15 L1 CSI-RS measurement)

Proposal 12: For alternative 1, some unbalanced factors between candidate neighbor cell and serving cell are introduced:

-due to RX beam scaling factor, the L1 CSI-RS measurement periodicity on neighbor cell would be far longer than that of serving cell,

-due to RX beam scaling factor, the RX beam gain on neigbor cell would be larger than that of serving cell.

Proposal 13: Alternative 2 can ensure that the CSI-RS based L1 measurements on neighbour cell and serving cell are compared fairly.

Conclusion

The contribution provides analysis on Measurements related enhancements for LTM. The following proposals are proposed:

Proposal 1: RAN4 to discuss whether to define requirements for subsequent LTM cell switching delay in R19. The requirements would be the same for both intra-CU and inter-CU.

Proposal 2: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3.

Proposal 3: Wait for more RAN1 progress on L1 filtering.

Proposal 4: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells.

Proposal 5: For CSI-RS based L1-RSRP measurement on neighbor cells, define the requirements for {D=3, PRB≧48}.

Observation 1: For L3 CSI-RS resources which are included in MeasObjectNR, in each cell, the L3 CSI-RS resources have the same CSI-RS center frequency, bandwidth and density.

Observation 2: There may have more than one L1 CSI-RS resources configured in each candidate neighbour cell, and the center frequency and bandwidth of these L1 CSI-RS resources can be different.

Observation 3: There may have more than one L1 CSI-RS resources configured in the serving cell and the center frequency, bandwidth and SCS of a L1 CSI-RS resource would change with the active BWP switching.

Proposal 6: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

Proposal 7: The applicability conditions are: all CSI-RS resources on candidate cells are configured with the same periodicity; FFS on whether to limit the CSI-RS resource to be measured is the same as active BWP bandwidth.

Proposal 8: Prioritize intra-frequency without gap and inter-frequency with gap for CSI-RS based L1 measurement on neighbour cell.

Proposal 9: UE shall first perform SSB based L3 measurement on candidate neighbour cells.

Proposal 10: In FR1, after UE correctly received configured CSI-RS resource of candidate neighbour cells, UE can directly measure on configured CSI-RS based L1-RSRP resources, as UE in FR1 has omnidirectional RX reception.

Proposal 11: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss the below alternatives:

Alternative 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and performs L1 CSI-RS measurement with Rx beam sweeping on candidate cells.

Step 1: L3 SSB based measurement Step 2: L1 CSI-RS measurement with RX beam sweeping - if associatedSSB/QCL-D is indicated for CSI-RS resource, refiner RX beam sweeping performs within SSB beam range, - otherwise Rx beam sweeping may be performed within larger range.

Alternative 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE performs measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement.

Step 1: L3 SSB based measurement Step 2: L1 SSB measurement with RX beam sweeping (i.e., R18 LTM) Step 3: L1 CSI-RS measurement without RX beam sweeping (like R15 L1 CSI-RS measurement)

Proposal 12: For alternative 1, some unbalanced factors between candidate neighbor cell and serving cell are introduced:

-due to RX beam scaling factor, the L1 CSI-RS measurement periodicity on neighbor cell would be far longer than that of serving cell,

-due to RX beam scaling factor, the RX beam gain on neigbor cell would be larger than that of serving cell.

Proposal 13: Alternative 2 can ensure that the CSI-RS based L1 measurements on neighbour cell and serving cell are compared fairly.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2406502, WF on NR mobility enhancements (part 1), MediaTek

- R4-2406513, Draft Big CR to TS 38.133 on core requirement maintenance for Further NR mobility enhancements, MediaTek Inc, Apple