3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415878

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for ATG CA operation

Agenda item:	6.10.4

Document for:	Discussion

Introduction

In the last RAN4 meeting, general scenario for ATG DL CA was discussed and there were some agreements [1]. In this contribution, we provide our views on RRM coer requirements for ATG with CA operation.

Discussion

General

UE antenna type

RAN4 had agreement for UE antenna types for intra-/inter-band DL CA.

- For intra-band contiguous CA, same antenna type should be applied on each carrier. - For inter-band CA, either different or same antenna type can be applied on different carriers. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on both PCell and SCell. - 2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell. - 3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell. - [4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel.] - 5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels. - Note: If some of the cases will be precluded in RF discussion, they will be removed for RRM requirement discussion.

For inter-band CA, baseline RF architecture is two separate antenna chain. In the last meeting, RF session agreed that there is no new capability for UE antenna type for ATG CA case, so it means that the baseline RF architecture for inter-band DL CA would be the same for ATG UE. Therefore, if ATG UE with an antenna array support inter-band DL CA, the UE should have separate antenna panels. So, case 4 in above should be precluded.

- Proposal 1: RAN4 to preclude the following UE antenna type for inter-band CA

  - ATG UE with an antenna array on both PCell and SCell with only one antenna panel.

Co-location signaling

- Proposal 1: FFS whether co-location information in inter-band CA configuration should be provided to UE.

In current RAN4 specification, non co-location deployment for inter-band CA is assuemed, and MRTD/MTTD requirements has been defiend based on the assumption. In ATG CA, since only co-located deployement scenario for both intra- and inter-band DL CA is considered, RAN4 needs to futher discuss whether co-location information should be provided depending on the conclusion of MRTD requirments for ATG inter-band CA.

- Proposal 2: FFS whether co-location information in inter-band CA configuration should be provided depending on the conclusion of MRTD requirments for ATG inter-band CA

Specification structure

- Option 1: The CA related requirements can be directly added in the sections defined for R18 ATG UE - Other Options are not precluded.

Basically, the CA related RRM requirements could be added in the existing ATG sections as Option 1, and newly introduced requirements such as interruption and SCell activation/deactivation delay could be added new sub-cluases with suffix D. However, RAN4 needs to discuss which RRM requirements will be introduced first. The overall specification structure for ATG CA can be further discussed after RAN4 has agreed on which RRM requirements will be defined.

- Proposal 3: The overall specification structure for ATG CA can be further discussed after RAN4 has agreed on which RRM requirements will be defined.

RRC_IDLE/ RRC_INACTIVE state mobility

Cell re-selection Idle/Inactive mode CA measurements

- Proposal 1: RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA. - Proposal 2: For cell re-selection, define the exceptions of side conditions for UE supporting CA in FR1. The legacy methodology exception applicability defined in Clause 4.2 and Clause B.3.2.1 can be reused with update of the reference clause of ΔRIB,c.

In RF session, the delta RIB for ATG CA (band n3A-n39A as baseline) was agreed as 0. So, RAN4 can reuse NR cell re-selection as a baseline for ATG cell re-selection for UE supporting CA without exceptions of side conditions for CA.

- Proposal 4: RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA without exceptions of side conditions for CA.

Idle/Inactive mode CA measurements (R16) / Measurement report for fast CA/DC setup (R18)

Idle/Inactive mode CA measurements (R16) - Option 1: Introduce idle/inactive mode CA measurements requirements for ATG UE in Rel-19 - Option 2: No need to introduce idle/inactive mode CA measurements requirements in ATG CA.

Measurement report for fast CA/DC setup (R18) - Option 1: RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline - Option 2: No need to introduce Rel-18 NR CA EMR in ATG CA. - Option 3: Consider whether to define the requirements of measurement report for fast CA setup in idle/inactive mode for R19 ATG UE

For Rel-19 ATG CA, RAN4 agreed to consider only single SCell, and only co-loacated DL CA deployment is considered. The network would not configure early measurement reporting to UE since SCell is always co-located with PCell. Therefore, the requirements for Idle/Inactive mode CA measurements (R16) and Measurement report for fast CA/DC setup (R18) are not necessary.

- Proposal 5: No need to introduce requirements for idle/inactive mode CA measurements and Measurement report for fast CA/DC setup in ATG CA

Timing

MTTD

- Option 1: FFS whether MTTD requirement is needed or not

For MTTD requirement, UE shall be capable of handling a relative transmission timing difference among the closest slot timing boundaries of different carriers to be agreegated. However, since Rel-19 ATG UE only consider signal UL transmission, MTTD requirement is not needed.

- Proposal 6: RAN4 not to define MTTD requirement for Rel-19 ATG

MRTD

- For ATG intra-band contiguous CA, not define MRTD requirement. - For ATG inter-band CA, MRTD requirement is: - Option 1: A little larger than 3us, i.e., 3.545 µs - Option 2: 33us - Option 3: 3us - Option 4: further discuss

Existing 33us MRTD requirement for inter-band CA has been defined under assumption that 30us propagation delay difference and 3 us TAE/cell phase synchronization accuracy since non co-located CA deploymenet is considered. However, for ATG inter-band DL CA, since only co-located deployment is considered, MRTD does not need to account for propagation delay differences. Therefore, existing MRTD for intra-band non-contiguous CA as 3us can be reused for ATG intra-band DL CA.

- Proposal 7: For ATG inter-band DL CA, existing MRTD for intra-band non-contigous CA as 3us can be reusded.

Signalling characteristics

SCell Activation and Deactivation Delay / Interruption

For SCell activation/deactivation delay, it can reuse the principle from the existing SCell activation / deactivation delay requirements as baseline for ATG. Since co-located based CA operation is assumed, only known SCell and scellWithoutSSB capability could be considered.

- Proposal 8: Reuse the principle from existing SCell activation / deactivation delay requirements as baseline for ATG, and only known SCell and scellWithoutSSB capability can be considered.

For interruption requirements, it can reuse the principle from the existing interruption requirements for SCell activation/deactivation and during measurement on deactivated SCell as baseline for ATG. The interruption requirements could be defined based on the identified SCell activation and deacation delay requirements. And considering only single uplink configuration in ATG, two uplink carrier related interruption requirements should be precluded.

- Proposal 9: Reuse the principle from exsiting interruption requirements for SCell activation/deactivation and during measurement on deactivated SCell as baseline for ATG.

Measurement procedure

General measurement requirements

In General, the principle from existing requirements for measurement procedure could be resued for ATG CA such as measurement gap and CSSF. For deactivated SCell for cell identification and measurement period requirements, it can also reuse the principle from the existing cell identification and measurement requirements with measCycleScell as baseline for ATG. For example, measurement period for intra-frequency measurements without gaps for deactivated SCell can be defined below.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | ceil( 5 x Kp x N1 Note 1 x Klayer1_measurement) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | ceil(5 x Kp x N1 Note 1 x Klayer1_measurement) x max(measCycleSCell, 1.5 x DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp x N1 Note 2 x Klayer1_measurement) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: For ATG UE with the antenna array, N1 = 3 when network assistance on ATG cells reference locations is provided, otherwise N1 = 4. Otherwise, N1 = 1. |  |

- Proposal 10: Reuse the principle from the existing requirements for measurement procedure such as measurement gap, CSSF, deactivated SCell identification and measurement requirements with measCycleScell as baseline for ATG CA

Scheduling Availability/Restrictions

For intra-band DL CA, since the same antenna type for each carrier is considered, the existing scheduling restriction could be applied as:

- When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

For inter-band DL CA, even if different antenna type of each carrier is assumed, the exsiting scheduling restriction for inter-band CA can be applied as:

- When TDD inter-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols, if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

- Proposal 11: Reuse the existing scheduling restriction for intra-/inter-band CA regardless of antenna types for ATG CA

Conclusion

In this contribution, we provide views on RRM requireiments for ATG with CA operation, and we propose

- Proposal 1: RAN4 to preclude the following UE antenna type for inter-band CA

  - ATG UE with an antenna array on both PCell and SCell with only one antenna panel.

- Proposal 2: FFS whether co-location information in inter-band CA configuration should be provided depending on the conclusion of MRTD requirments for ATG inter-band CA

- Proposal 3: The overall specification structure for ATG CA can be further discussed after RAN4 has agreed on which RRM requirements will be defined.

- Proposal 4: RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection to support CA without exceptions of side conditions for CA.

- Proposal 5: No need to introduce requirements for idle/inactive mode CA measurements and Measurement report for fast CA/DC setup in ATG CA

- Proposal 6: RAN4 not to define MTTD requirement for Rel-19 ATG

- Proposal 7: For ATG inter-band DL CA, existing MRTD for intra-band non-contigous CA as 3us can be reusded.

- Proposal 8: Reuse the principle from existing SCell activation / deactivation delay requirements as baseline for ATG, and only known SCell and scellWithoutSSB capability can be considered.

- Proposal 9: Reuse the principle from exsiting interruption requirements for SCell activation/deactivation and during measurement on deactivated SCell as baseline for ATG.

- Proposal 10: Reuse the principle from the existing requirements for measurement procedure such as measurement gap, CSSF, deactivated SCell identification and measurement requirements with measCycleScell as baseline for ATG CA

- Proposal 11: Reuse the existing scheduling restriction for intra-/inter-band CA regardless of antenna types for ATG CA

Reference

[1] R4-2413959, “WF on RRM requirements for NR_ATG_enh,” Moderator (CMCC)