3GPP TSG-RAN WG4 Meeting #112bis			 R4-2415895

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.17.2.2

Source: 	Moderator(Qualcomm Incorporated)

Title: 	Summary of e-mail discussion on data sharing for AI/ML

Document for:	Information

# Introduction

RAN4 is currently conducting a feasibility for testing of CSI compression. It was agreed that this feasibility study will be performed over multiple steps and data sharing (ML models of encoder or decoder or both, and/or datasets for training/testing/inference) will be required [1]. Data sharing is a new process for 3GPP and several logistical issues will need to be solved.

This email thread will discuss logistical issues regarding data sharing for AI/ML. Before RAN4#112 there was another brief discussion on this topic in which companies provided inputs regarding data size and possible formats to be used for ML model sharing. A brief summary will be provided.

# Discussion

## Data size

Based on company inputs provided so far(inputs provided in the email discussion before RAN4#112, the data size needs can be summarized as follows:

### Training data

- Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance)

- Total space needed would depend on the number of contributing companies and meetings

- Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies

  - Data sharing might not be needed in every meeting

Companies are invited to provide any further comments as needed. No need for any inputs if companies just agree with this.

| Company | Comments |
| --- | --- |
| Intel | The summary above is a good initial estimate for the CSI compression use case. Same time the size of the training dataset may depend on RAN4 assumptions on the specific dataset used for training including at least the number of samples, and whether reference encoder shall be trained based on decoder input or output. The respective aspects were not concluded and further RAN4 decisions may be required to provide a more detailed estimate. In addition, our understanding is that the discussion is focused on the CSI compression use case and potentially training data sharing may be required for other use cases. We would suggest to clearly state that the assumptions are relevant to CSI compression use case only. |
| Nokia | The assumptions above are reasonable for the currently considered use-case (i.e., CSI compression in frequency-space domains). However, if we are thinking about more future proof solutions. Potentially, even larger dataset sizes might be considered. For example, the space-frequency-time CSI compression datasets will include multiple time sample, that would further increase the data-set size x10-100 times. Thus, we need a clarification, that The values are provided specifically for CSI compression use-case study in RAN4. |
| Apple | We agree with the assumptions above. There is a possibility of increased dataset sizes if we consider diverse datasets across generalization scenarios (speed, SNR, propagation model) as well as scalability scenarios (rank, number of ports, etc). We may need to come up with all possible generalization/scalability scenarios we envision to consider for future study. |
| ZTE | We are agree with the assumptions above. Different cases may have different datasets. If we introduce more scenarios in the future, we need to clarify the above assumptions that is only for some specific cases from RAN4 perspective. |
| Ericsson | Similar to other companies, the estimates are OK, but we may need to be flexible to other cases in the future. For now, there should be enough for companies to share up to 600-800k training set sizes for CSI compression. |
| Xiaomi | We are fine with the estimations for current discussed use case for CSI compression. When other scenarios are considered, we may need further discussion. |
| CMCC | The values are OK. Share similar view with other companies that it is better to clarify that this is only for feasibility study for testing of CSI compression use-case in RAN4. |
| Samsung | For this discussion, the estimation is okay for us, just similar to other companies’ comments. We are also okay to emphasize the estimation should be given in a case-by-case manner, which makes the clarification sentence from Nokia is also reasonable to us. |
| OPPO | In general, we agree with the assumptions above. A clarification question: “~60GB per company per year” means that a company can receive 60GB of space annually?, for example, 60GB available in 2025 and 60GB+60GB available in 2026? Seems we only need a fixed space for a given case. |
| Huawei, Hisilicon | The values are reasonable. Agree with Intel, Nokia, ZTE, CMCC and Samsung to clarify that the values are provided specifically for feasibility verification of potential testing options in AI CSI compression in RAN4. Regarding to generalization/scalability, we are not sure whether the criteria of feasibility in Option 3 should take account of generalization/scalability. If generalization/scalability is considered, combining the dataset from all companies is one way to generate the generalized data. In this case, no more space is needed. However, if we have to consider generalization/scalability across different simulation assumptions, then the space should be enlarged. |
| vivo | Consideration on data size We think that companies’ assumptions are reasonable. The listed data size is valid for current CSI compression in frequency-space domain. The data size would be 10~100 times larger for CSI compression in time-space-frequency domain, since multiple channels are included in one sample. Also, the data size would be larger if the study of generalization/scalability begins. Thus, we may start the data sharing with the listed data size for current study of CSI compression in frequency-space domain, and discuss the needed data size when new study is needed. Consideration on data upload/download speed The data upload/download speed would be ensured to be a proper level. Training data size of one company is about 1GB up to 10GB, which would be larger for future study. For example, if the size of one dataset is 10GB and the data upload/download speed is 1MBps, it would take about 3 hours to upload/download one dataset. Breakpoint resume should be supported, since there may be network fluctuation or interruption during the dataset upload/download, which may take several hours. |

### AI/ML Model Storage/Sharing

- Model size: ~7MB – 100MB

- Total space needed would depend on the number of companies contributing and meetings

- Upper bound should be 600MB per company per year -> ~10GB per year

Companies are invited to provide any further comments as needed. No need for any inputs if companies just agree with this.

| Company | Comments |
| --- | --- |
| Nokia | A similar clarification as above is needed: The values are provided specifically based on CSI compression use-case study in RAN4. |
| Apple | A similar agreement on the number of potential AI/ML models needed across different generalization and scalability scenarios should be reached. |
| Samsung | Nokia’s clarification is also okay for us. |
| OPPO | Similar issue on “per year”, how to understand it? |
| vivo | Similar to data size, we may start the model sharing with the listed model size for current study of CSI compression in frequency-space domain, and discuss the needed model size when new study is needed. |

### Testing and inference data

Testing and inference data sizes were not explicitly discussed but these should have smaller size compared to the training data and similar format.

Companies are invited to provide comments on the size of the data for testing and inference.

| Company | Comments |
| --- | --- |
| Intel | The dataset size can be in the range 10-25% of the training data set size |
| Qualcomm | The testing dataset size can be 12.5% of the training data set size. Additional 12.5% of the training data set size can be reserved for validation. |
| Nokia | It was no specific discussion of testing/inference datasets before. Conceptually, except for the size, consideration of testing/inference datasets should be the same as for the training dataset. Hence, we think that it will be enough to keep only a single dataset discussion, also considering that training dataset provides the upper bound from the size point of view. |
| Apple | We should limit the discussion on the training dataset size. Training data set size will include data used for training, validation, and testing. Partitioning could be agreed as a separate discussion. |
| ZTE | From our understanding, testing data comes from the the training dataset. Generally, We should pay attention to the training data, striving to avoid both overfitting and underfitting. In cases where the training data size if sufficient, a smaller proportion of the test set might be used. |
| Ericsson | We have yet to discuss how wide testing needs to be. Anyhow, it is reasonable to discuss simply total space needed for all dataset sharing. |
| Xiaomi | Training data set will include test data set and inference data set. We can further discuss the ratio for each data set respectively. Generally, we are fine that the ratio is around 10%~15%. |
| Samsung | We should be clarified about the purpose of “testing and inference data” here, and our understanding is if it is for model training, the dataset size discussion in 2.1.1 already contains that part. If it is for validation of trained model for different companies, such as validation of generalization in different condition, we may need additional storage, but it should be much smaller than the one discussion in 2.1.1. Therefore, we consider this “testing and inference dataset” is not needed to be explicitly calculated, by assuming the additional size is much smaller than training data, especially if RAN4 assume “Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies”, which already include rooms. |
| CATT | From our understanding, these testing and inference data are used during training stage, i.e., for validation and/or test, and they should be a part of training data which means their sizes are already considered in Section 2.1.1. Therefor it is not necessary to discuss this issue separately. Regarding the ratios of validation and test data, 10%~20% in total is reasonable, e.g., 9:1:1 of the datasets discussed in 2.1.1 for training, validation, and test respectively. |
| OPPO | The size of the training set is much larger than that of the test/validation data set. Usually around 10% is sufficient. So if we're just considering data storage space, the space left for the training set now is enough to cover the test and validation sets. In addition, the test set and the validation set themselves can also be obtained by partitioning from the training set. |
| Huawei, Hisilicon | Since we are trying to align on a model, dataset for validation/testing is needed for cross-checking. The validation/testing dataset is part of the training dataset. No more room is needed. |
| vivo | For simplicity, we suggest that the uploaded training dataset includes training data and testing data. For example, the first 90% samples in the uploaded dataset is training data (including validation data), and the last 10% samples is testing data. |

## Data formats

### Training/Testing/Inference data

Data to be share will have to be in a pre-defined format such that it can be processed by all interested companies. The format will have to be discussed, agreed and documented.

Companies are invited to provide comments and concrete examples on the data format below. RAN4 is currently discussing a specific use case (CSI compression), it could be useful to discuss whether the format should be easily adaptable to other use cases/training assumptions or not.

| Company | Comments | Concrete example or other notes |
| --- | --- | --- |
| Intel | The discussions shall focus on CSI compression use case and formats for other cases should be discussed separately. Furthermore, we do not think that there should be a strong requirement that the format should be easily adaptable to other use cases/training assumptions. Using Matlab '-v7.3' file format is preferred. The specific details of the data format (including FP precision, ) are recommended to be discussed in RAN4 #112bis based on companies inputs. |  |
| Qualcomm | Matlab or NumPy file formats are preferred. It should be checked if 3GPP allows Matlab file formats since Matlab is not an open-source software. |  |
| Nokia | The following aspects of data files should be considered when sharing data for the feasibility study of the CSI compression test decoder: - Dataset file format of the data files: We prefer using the open-source Parquet file format for sharing the dataset due to its advantages, including efficient storage, faster query performance, broad compatibility with data processing tools, embedded metadata, and support for complex data types. - Dataset file description/metadata: Since Parquet (and potentially other formats) is a columnar storage file format, pre- and post-processing steps will be required when interfacing the Parquet files with company-specific model training and simulation systems. To ensure fast and efficient data usage across different companies, a detailed file template must be agreed or attached to the file. - The data description can include, for example, the following parameters of the eigenvectors: - Real/imaginary parts of the complex numbers - Number of Antenna Ports - Number of Sub-bands - Number of Layers - Number of Samples - Content of the dataset files: The shared dataset could include the following elements: - Encoder Input (Eigenvectors and eigenvalues, potentially, up to 4 layers but now just 1 layer is considered for now) - Encoder Output (Compressed CSI) - New variables can be further added if deemed necessary. |  |
| Apple | We also prefer Matlab '-v7.3' or Numpy file formats. We will need to agree if we store the raw channel H or the eigenvectors. If one company trains the AI/ML with a specific pre-processing of raw channel to eigenvector, but the stored eigenvectors have been pre-processed in a different way, could this cause mismatches? If we store eigenvectors we can’t make comparisons with etype-II. |  |
| ZTE | Dataset files depend on the platform adopted by each companies. Matlab or python also needs to be considered. We are agree with Nokia’s proposal about dataset file description. All uploaded data needs to have the same pre-processing procedure or detailed description about pre-processing and follow the assumptions of RAN4. What’e more, different pre-processing may have different simulation results. It’s better to clarify in the simulation assumptions. |  |
| Ericsson | Numpy is a reasonable way to share data. We will need to discuss details for storage/extraction and make those processes as efficient as possible. |  |
| Xiaomi | For data formats, we prefer NumPy file formats due to their faster data reading performance. In addition to choosing the data format, it is crucial to align the datasets between companies. This includes standardizing the raw data generation methods, such as how to generate raw channel H. Furthermore, if pre-processing is involved, the procedures for obtaining eigenvectors must also be clearly defined. |  |
| CMCC | We prefer Numpy file formats. As for to store the raw channel H or the eigenvectors, raw channel H is preferred. |  |
| Samsung | Both .mat (Matlab) and .npy (Numpy) are both okay to us. In practice, it is no difference, but just a format conversion procedure. Both can be used with minimal efforts only. We don’t see it is an issue at all. We also see no big value for mandating “aligning” this, and we can allow both if dataset is agreed and intended to be shared. |  |
| CATT | Either format is okay. Besides, we agree with Nokia. The corresponding dataset file descriptions containing sufficient information, i.e., scenario/configurations, should be provided when sharing a dataset. Regarding the data content, either raw channel H or eigenvectors is okay to us. If eigenvectors are shared, any used pre-processing should be clarified as well. |  |
| OPPO | Both .mat (Matlab) and .npy (Numpy) are OK |  |
| Huawei, Hisilicon | We are OK with Numpy. Both raw channel H and eigenvectors are needed. When there is a mismatch using eigenvectors, we can check if there is any mismatch of the pre-processing. |  |
| vivo | Numpy is preferred to share data, since Numpy is very widely used in Python and Matlab may cause legal concerns. |  |

### AI/ML models

For the AI/ML model format, companies already provided some proposals which were briefly discussed. Please provide comments(such as pros/cons for different formats) and preferred format. Based on the preliminary input from MCC, any open source format can be used but this has to be further checked.

AI/ML model formats proposed so far: PyTorch, TensorFlow, ONNX

| Company | Comments | Preferred format |
| --- | --- | --- |
| Intel | The details of the AI-/ML model format are recommended to be discussed in RAN4 #112bis. In particular, recommend to further investigate and select the approach to share AI-ML models for CSI Decoder including - Option 1: Model parameters in PyTorch or TensorFlow formats and source codes - Option 2: Entire models in PyTorch or TensorFlow formats (no source codes) - Option 3: Models in ONNX format (no source codes) It is encouraged that companies check whether the respective models can be used as a part of training framework. |  |
| Qualcomm | Some companies use PyTorch and some other companies use TensorFlow formats to represent AI/ML model. ONNX is needed to convert between PyTorch and TensorFlow. Companies should, at least, provide AI/ML models in ONNX format. In addition, companies can provide AI/ML models in PyTorch/TensorFlow formats, too. Companies should also mention the version of ONNX/PyTorch/TensorFlow that was used to generate the AI/ML model. If Pytorch gets selected as one of the AI/ML model format, RAN4 should down-select between different types of Pytorch files (e.g., between .pt and .t7 files) |  |
| Nokia | Possible formats of the shared AI/ML models should be limited to the ones developed in popular frameworks (e.g., TensorFlow and PyTorch) with support for intermediary sharing platforms (e.g., ONNX). We also support the proposal from Intel that we additionally need to consider and analyze the possibility to use shared model (e.g., decoder) for training (i.e., of the encoder). |  |
| Apple | We also support the proposal from Intel that we additionally need to consider and analyze the possibility to use shared model (e.g., decoder) for training (i.e., of the encoder). That could be used for feasibility analysis: Each company trains its preferred encoder with the selected test decoder. Each company to provide AI/ML models in open standard format (ONNX) that captures the computation graph from the native PyTorch/Tensorflow model. Each company can also provide AI/ML models in PyTorch/TensorFlow formats |  |
| ZTE | Considering different platforms are used for different companies. It’s better to transfer frameworks before upload. Regarding ONNX, RAN4 needs to check the feasibility. |  |
| Ericsson | Regarding the model format, we should take care to differentiate between short term model sharing and eventual capturing in a specification. Either way though, in our understanding, ONNX can vary from version to version and may not always be backward compatible, thus it might be better to fix the ONNX version. Quite probably there is more to agree than “use ONNX”. Also, if we agree on the ML architecture, is there some other way to share the actual parameters that we could agree in RAN4 and may lead to less risk to different formats companies use ? |  |
| Xiaomi | We are open to further discuss model format. For ONNX, it’s a solution when company use different model types. There are some possible issues due to ONNX which may need further study. For example, there may be compatibility issues. ONNX may not support all operations from PyTorch or TensorFlow, especially custom or recent operations. Need to ensure compatible versions of ONNX, PyTorch, and TensorFlow to avoid issues. Besides, When converted to ONNX, advanced features or specific layer implementations may not be translated perfectly, affecting performance or behavior. Testing may be needed to ensure the converted model's behavior matches the original on different platforms. |  |
| CMCC | PyTorch/Tensorflow can be considered. And both entire models and source codes can be shared, with the consideration that shared model can be used for encoder training and source codes can be used for cross check. |  |
| Samsung | As we proposed before, for simplicity, our best guess is most of companies may use PyTorch, so directly using PyTorch could be easier for us? ONNX is also okay for us. |  |
| CATT | To make models widely usable, intermediate format is necessary. Therefore companies need provide intermediate model files, e.g., ONNX. Furthermore, considering most of companies use pytorch or tensorflow and the size of models files is very limited, pytorch or tensorflow model files can also be shared voluntarily by companies together with ONNX files. |  |
| OPPO | TensorFlow and PyTorch are the two most prevalent and widely utilized formats. Participating companies can provide reference models using TensorFlow or PyTorch. If any company faces difficulties in providing reference models in these two formats, we can further consider what are the challenges they encounter. In addition, if a company needs to use another model format, they can generate it by ONNX or other methods based on their needs. |  |
| Huawei, Hisilicon | It will be helpful if both ONNX and pytorch/tensorflow are shared. For companies which use the same platform, there is no need to convert. For companies which use a different platform, there’s still a chance to cross check. |  |
| vivo | ONNX is suggested. Some companies may use Tensorflow and may not support PyTorch, and some other companies may use PyTorch and may not support Tensorflow. And then, it would cause problem to use Tensorflow/PyTorch. ONNX would be used for both Tensorflow based model and PyTorch based model. Also, we may further study the details of using the ONNX/Tensorflow/PyTorch model without source code for training (if needed) and inference. |  |

## Other issues

During the previous discussion on data sharing other issues than data size and formats were brought up such as how long the data needs to stored, need for documentation(where will the data formats be documented), need for scripts to check on data integrity/contents, etc.

Companies are invited to bring up any other issues that should be discussed regarding AI/ML data sharing.

| Company | Comments | Other notes |
| --- | --- | --- |
| Intel | We do see the value in documentation of the agreements on the logistics questions (data formats, etc) and respective information can be added to TR 38.843. For the initial stage we do not see a strong need for scripts to check data integrity / contents and it can be handled at a later stage, if issues are identified. |  |
| Nokia | In our understanding, the current discussion is applied only to RAN4 study and not to potential dataset/models to be standardized. If we consider standardized models/datasets, then many other aspects need to be considered, including security, integrity, versioning, but this discussion can take place later on. At the current stage, we would like to highlight the following additional aspects: - As we commented above, dataset metadata/description will be needed so that the yare usable by other companies. - An API can be considered for dataset sharing. At least it will be future-proof solutions, i.e., usable not only temporary for the study but in a longer term. - We can limit the length of storage of large datasets, especially with intermediate data used in the RAN4 study. |  |
| Ericsson | We agree with Nokia that the current scope of the model format discussion should be simulation only, not eventual capture in a specification. Regarding dataset sharing, apart from the format and integrity, maybe we need to consider “alignment” of the datasets. If they are all supposed to come from the same system simulation assumptions, maybe statistics should be checked as the datasets should be similar. If they come from elsewhere, maybe some way of assessing that the datasets are comparable and appropriate for the study. Otherwise there is a risk of mixing different datasets that are potentially not really aligned. |  |
| CMCC | Share similar view with other companies that no need to capture in the specification. If companies prefer to capture in the specification, it is necessaryto clarify that this is only for feasibility study for testing of CSI compression use-case in RAN4. |  |
| Huawei, Hisilicon | Since the data sharing is only for study, there is no need for documentation of the data formats. For checking data integrity/contents, if it is hard to derive a criterion or a verification procedure, we can leave it up to companies’ cross checking. |  |

# Conclusion

Based on the companies inputs, the summary of the discussion is given below. This has to be taken into account when discussing further details on data sharing in RAN4 for AI/ML CSI compression testability study.

2.1 Data size

2.1.1 Training data:

- Companies agree that the current assumptions and estimates for the CSI compression use case are reasonable (Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance)

- These assumptions are specific to the CSI compression use case in RAN4 and for the current simulation assumptions.

- Larger datasets could be needed/used in future studies depending on the use case. It would be good to maintain some flexibility

- Importance of ensuring proper data upload/download speeds and supporting breakpoint resume due to potential network interruptions – to be discussed with MCC

2.1.2 AI/ML Model Storage/Sharing:

- Companies agree that the current assumptions and estimates for the CSI compression use case are reasonable (Model size: ~7MB – 100MB, Upper bound should be 600MB per company per year -> ~10GB per year)

- These assumptions are specific to the CSI compression use case study in RAN4

- The upper bound is assuming that a company contributes in each meeting within a year (6 WG meetings)

2.1.3 Testing and inference data:

- Most companies commented that testing and inference data should be a subset of the training dataset so there would be no need to consider the data size separately

- Testing data set should be 10-25% of the training dataset (most companies showed 10-12.5%)

- Other details on partitioning or how to use the datasets would have to be discussed separately

2.2 Data Formats:

2.2.1 Training/Testing/Inference data:

- Possible formats to be used for the dataset which brought up are: NumPy, Matlab and Parquet

- Formats will have to be discussed with MCC to see which ones can be used in 3GPP.

- Most companies prefer NumPy, some where ok also with Matlab

- The details of the data formatting (how data is represented, what is contained in each field, etc) will have to be discussed and agreed after the file format will be decided

2.2.2 AI/ML models:

- Possible formats to be used for the AI/ML models are ONNX, PyTorch and TensorFlow

- Formats will have to be discussed with MCC to see which ones can be used in 3GPP

- There are pros/cons for each format, most companies seem to favor ONNX as it is open source, however, more discussion is needed to agree on a format

- Further details on the format (contents of the files and what information needs to be provided) will have to discussed/agreed after deciding on the format

2.3 Other Issues

- How to document the file formats will have to be further discussed

  - Some level of documentation will be needed such that there is a common reference and anyone can process the data

- Different use cases will likely have different needs and all formats discussed here apply for the CSI compression use case

- If there will be a need to specify datasets or models, the level of documentation required might be different(more details might be needed, etc)

- The need for an API for data sharing needs to be discussed

- The “lifetime” of the data needs to be further discussed, some data used in intermediate steps in the RAN4 study might not have to be stored for a long time

# References:

R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, RAN4#112