3GPP TSG-RAN WG4 Meeting #112bis	R4-2415934

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.23.4.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

As summarized in WF [1], there are still many open discussion points. This document proposes RRM core requirements for LP-WUS/WUR based on the WFs.

- Discussion

  - RRM relaxation and serving cell measurement for case 3

During previous meeting, serving cell measurement for case 3 was discussed. The WF is copied as below.

- For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

Whether to combine the measurements across MR and WUR may be related to RAN1 LS (R4-2414909). According to the LS, RAN1 has the common understanding that UE may not support LP-WUS reception on all the bands supported by the UE. If this UE assumption is agreed, the measurement on MR and WUR can be combined in the cases which MR and WUR can cover same frequency bands.

Observation 1: The measurement on MR and WUR can be combined in the cases which MR and WUR can cover same frequency bands.

Proposal 1: It is better to wait for WUR supporting band assumptions to determine whether to combine the measurements across MR and WUR.

  - MR RRM relaxation

In accordance with previous agreement, MR relaxation requirements for both serving/neighbouring cell are needed for case #3. The existing relaxation for scaling factor can be the starting point for MR RRM relaxation.

Proposal 2: MR relaxation requirements for both serving/neighbouring cell are needed and the existing relaxation for scaling factor can be the starting point for MR RRM relaxation.

  - Interruption requirement

RAN1 is now discussing the wake-up delay between LP-WUS reception and MR to start PDCCH monitoring as follows:

  - Wake-up delay

    - Definition: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

    - UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

      - FFS: X is 2, 3, 4

RRM requirements for interruption should be based on the RAN1 agreements for wake-up delay.

Proposal 3: RRM requirements for interruption should be based on the RAN1 agreements for wake-up delay.

- Conclusion

In this contribution, we propose possible RRM core requirements for LP-WUS/WUR

Observation 1: The measurement on MR and WUR can be combined in the cases which MR and WUR can cover same frequency bands.

Proposal 1: It is better to wait for WUR supporting band assumptions to determine whether to combine the measurements across MR and WUR.

Proposal 2: MR relaxation requirements for both serving/neighbouring cell are needed and the existing relaxation for scaling factor can be the starting point for MR RRM relaxation.

Proposal 3: RRM requirements for interruption should be based on the RAN1 agreements for wake-up delay.

References

[1] 3GPP, R4-2413889, “WF for RRM core requirements for LP-WUSWUR”, vivo