# 3GPP TSG-RAN WG4 Meeting #112bis                            R4-2415939

# Hefei, China, 14th-19th Oct, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on receiver characteristics for LP-WUR

# Agenda Item:   6.23.2.3

# Document for:  Approval

- Introduction

In this contribution, we provided our view on receiver characteristics for LP-WUR.

- Discussion

## 2.1 IBB and OBB

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies specified for IBB and OBB in TS38.101-1. The left issues in RAN4#112 meeting on IBB and OBB for LP-WUR are listed as follows[1].

Issue 3-1-1: IBB and OBB assumption Agreement: - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR. - With assumption that Rx dynamic range for LR may not be as good as MR Issue 3-1-2: IBB and OBB requirements Agreement: - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS IBB and OBB requirements

LP-WUR an MR may share the same antenna and baseband processor, which means that LP-WUR and MR will suffer the same level of interference. In our understanding, same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Proposal 1: Same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Regarding to test conditions, the test parameters that are needed to consider include MR channel bandwidth, power level and guard RBs. Similar to ACS test, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier. As for whether guard RBs are needed, we can further discuss it after we have preliminary agreements on the test conditions for ACS.

Proposal 2: For IBB and OBB test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs for IBB and OBB.

Proposal 3: Consider the test parameters for IBB of LP-WUR in Table 1, Table 2 and the test parameters for OBB of LP-WUR in Table 3, Table 4. For these test parameters, the missed detection rate shall be ≤1%.

Table 1  In-band blocking parameters for LP-WUR

| RX parameter | Units | WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB (for 30kHz SCS) |
| LP-WUS Power | dBm | REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | [7.5+guard RB] |
| FIoffset case 2 | MHz | [12.5+guard RB] |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 2  In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -56 | -44 |
| Finterferer (offset to LP-WUS)1 | MHz | -FIoffset, case 1 and FIoffset, case 1 | ≤ -FIoffset, case 2 and ≥ FIoffset, case 2 |
| NOTE 1: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). |  |  |  |

Table 3  Out-of-band blocking parameters for LP-WUR

| WUS transmission bandwidth configuration (RB) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11RB (for 30kHz SCS) | REFSENS + 6.0 dB |

Table 4  Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f - FDL_low < -15 or 15 < f - FDL_high < 60 | -85 < f - FDL_low ≤ -60 or 60 ≤ f -FDL_high < 85 | 1 ≤ f ≤ FDL_low -85 or FDL_high + 85 ≤ f ≤ 12750 |

## 2.2 Intermodulation Requirements

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal. The main issue for intermodulation requirements for LP-WUR is the detailed test condition[1].

Issue 3-1-4: Intermodulation requirements Agreement: - RAN4 further check and discuss detailed test condition for Intermodulation requirements, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS Intermodulation requirements

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively in TS38.101-1[2]. The missed detection rate shall be ≤1% under these assumptions. Similar to the discussion in Section 2.1, bandwidth of the interferer should be 5MHz and WUS should be located at edge of the NR carrier. As for whether guard RBs are needed, we can further discuss it after we have preliminary agreements on the test conditions for ACS.

Proposal 4: For intermodulation test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs for intermodulation requirements.

Proposal 5: Consider the test parameters for intermodulation of LP-WUR in Table 5. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Wide band intermodulation parameters for LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB(for 30kHz SCS) |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset to LP-WUS)2 | MHz | [-7.5-guard RB] / [+7.5+guard RB] |
| FInterferer 2 (Offset to LP-WUS)2 | MHz | [-15-guard RB] / [+15+guard RB] |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). |  |  |

## 2.3 Spurious Response

Spurious response is also used to evaluate the receiver capability when there is presence of an unwanted CW interfering signal on the frequency where out-of-band blocking limit is not met. Spurious response requirement is needed for LP-WUR.

Proposal 6: Consider the test parameters for spurious response of LP-WUR in Table 6 and Table 7. For these test parameters, the missed detection rate shall be ≤1%.

Table 6  Spurious response parameters for LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
|  |  | 11RB(for 30kHz SCS) |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENS + 6 dB |

Table 7  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

- Conclusion

Proposals:

Proposal 1: Same interference level of IBB and OBB as MR can be assumed for LP-WUR.

Proposal 2: For IBB and OBB test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs for IBB and OBB.

Proposal 3: Consider the test parameters for IBB of LP-WUR in Table 1, Table 2 and the test parameters for OBB of LP-WUR in Table 3, Table 4. For these test parameters, the missed detection rate shall be ≤1%.

Table 1  In-band blocking parameters for LP-WUR

| RX parameter | Units | WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB (for 30kHz SCS) |
| LP-WUS Power | dBm | REFSENS + 6 dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | [7.5+guard RB] |
| FIoffset case 2 | MHz | [12.5+guard RB] |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. |  |  |

Table 2  In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | -56 | -44 |
| Finterferer (offset to LP-WUS)1 | MHz | -FIoffset, case 1 and FIoffset, case 1 | ≤ -FIoffset, case 2 and ≥ FIoffset, case 2 |
| NOTE 1: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). |  |  |  |

Table 3  Out-of-band blocking parameters for LP-WUR

| WUS transmission bandwidth configuration (RB) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11RB (for 30kHz SCS) | REFSENS + 6.0 dB |

Table 4  Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | -44 | -30 | -15 |
| Finterferer (CW) | MHz | -60 < f - FDL_low < -15 or 15 < f - FDL_high < 60 | -85 < f - FDL_low ≤ -60 or 60 ≤ f -FDL_high < 85 | 1 ≤ f ≤ FDL_low -85 or FDL_high + 85 ≤ f ≤ 12750 |

Proposal 4: For intermodulation test, bandwidth of the interferer should be 5MHz and LP-WUS should be located at edge of the NR carrier. FFS guard RBs for intermodulation requirements.

Proposal 5: Consider the test parameters for intermodulation of LP-WUR in Table 5. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Wide band intermodulation parameters for LP-WUR

| Rx parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB(for 30kHz SCS) |
| Pw in LP-WUS Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 6 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | 5 |
| FInterferer 1 (Offset to LP-WUS)2 | MHz | [-7.5-guard RB] / [+7.5+guard RB] |
| FInterferer 2 (Offset to LP-WUS)2 | MHz | [-15-guard RB] / [+15+guard RB] |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). |  |  |

Proposal 6: Consider the test parameters for spurious response of LP-WUR in Table 6 and Table 7. For these test parameters, the missed detection rate shall be ≤1%.

Table 6  Spurious response parameters for LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
|  |  | 11RB(for 30kHz SCS) |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENS + 6 dB |

Table 7  Spurious response for LP-WUR

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

- Reference

- R4-2414309 WF for LP-WUS UE RF, RAN4#112, Maastricht, Netherlands, Aug. 2024.

- TS38.101-1, v18.6.0