3GPP TSG-RAN WG4 Meeting #112bis                           R4-2416116

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:              General discussion on the NES common signal adaptation

Agenda item:			6.22.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some initial discussions on NES common signal adaptation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update

The latest progress for NES common signal/channel adaptation is duplicated as below

<PRACH adaptation> Agreement For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 1-1: Scaled/adjusted PRACH configuration period - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Opt 1-3: Muting/masking ROs - Opt 1-4: additional timing offset(s) - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources, - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g. - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Opt 2-2: Additional timing offset(s) - FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) - FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset) Agreement Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1 - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs RAN1#117 Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement For SSB-RO mapping rule for additional PRACH resources for Case 2. - Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping. Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated. - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available. - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period - FFS: details <SSB adaptation> Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact [FFS point regarding SSB adaptation captured in R1-2407409] - Proposal 2.1.1c-rev2 Study further the following for Rel-19 NES-capable UE in idle/inactive mode - [Option 1: Adaptation of CD-SSB] - Option 2: Adaptation of CD-SSB at least for a cell not providing initial cell selection - FFS: whether R19 NES-capable UE can use the cell for initial cell selection - Option 3: Adaptation of SSB that is not CD-SSB at least for a cell that has large periodicity CD-SSB (e.g. periodicity greater than 20 msec) - No adaptation on CD-SSB - Option 4: No additional SSB adaptation mechanism is specified - Proposal 2.1.2a-rev1 For adaptation of SSB in time-domain, support SSB adaptation based on two SSB configurations where at least one configuration is active - At least SSB periodicity can be same or different between the SSB configurations - ssb-PositionsInBurst" and "ss-PBCH-BlockPower" are same between the SSB configurations - FFS: whether ssb-PositionsInBurst" and "ss-PBCH-BlockPower" can be different between the SSB configurations is supported - FFS: other settings between the SSB configurations - FFS: whether more than one SSB configuration can be active <Conclusions regarding paging adaptation captured in R1-2407409> ## Moderator suggestion for RAN1#118: wait for RAN2 progress.

From the agreements achieved in RAN1 summarized as above, we can observe that:

- RAN1 paid more attention on PRACH adaptation

- SSB-RO mapping rule for 3 cases. Extend the RAN1#117 agreement on SSB-RO mapping rule

- DCI indication. DCI based adaptation was supported with the existing DCI format, while how to use DCI to indicate additional PRACH resources is still suspending

- Additional PRACH resources configuration. How gNB can configure additional PRACH resources for NES capable UE were discussed without conclusions yet

- SSB adaptation

- PCell and/or SCell (s). Rel-19 NES-capable UE’s PCell CD-SSB on sync raster is not supported. Whether NCD-SSB or SSB not on sync raster should be supported is need to be further discussed. Rel-19 NES-capable UE’s SCell NCD-SSBs are supported, CD-SSB with > 20 ms periodicity is FFS

- SSB adaptation mechanism. Cell DTX extension to SSBs not on sync-raster for connected mode UEs was extensively discussed

- Rel-19 NES-capable UE in idle/inactive mode. It is somewhat possible to support the scenario of UEs in idle/inactive mode, but majority companies have concerns on whether to support it truly due to the huge impacts brought by CD-SSB adaptation

- Adaptation based on two SSB configurations where up to two configurations can be active. Whether the configuration of the two SSBs (i.e., SSB-burst transmission pattern) can be same or not is suspending

Observation 1: Based on RAN1 agreements on SSB adaptation.

- PCell and/or SCell (s). Rel-19 NES-capable UE’s PCell CD-SSB on sync raster is not supported, FFS: NCD-SSB or SSB not on sync raster; Rel-19 NES-capable UE’s SCell NCD-SSBs are supported, FFS: CD-SSB with > 20 ms periodicity

- FFS: Whether to support Rel-19 NES-capable UE in idle/inactive mode

- FFS: Additional SSB resources configuration/SSB transmission pattern for different adaptation mechanism

Observation 2: Based on RAN1 agreements on PRACH adaptation.

- DCI-based adaptation for additional PRACH resources. FFS: The details

- Extend the RAN1#117 agreement on SSB-RO mapping rule to no time-domain overlap and time-domain overlap but no overlap in frequency domain cases

- FFS: Additional PRACH resources configurations

Since not too much concrete conclusions obtained in this meeting for PRACH adaption, in this paper, we mainly focus on SSB adaptation discussions.

3 Discussion

3.1 SSB adaptation

    - SSB adaptation impact in RRM requirement

In connect mode, a lot of RRM requirements in relation to SSB based measurement, theoretically from our understanding, at least the following requirements (DL related only, TRP specific is not considered) may be impacted:

- Section 6: RRC_CONNECTED state mobility. 6.1.1 Handover; 6.2 RRC Connection Mobility Control

- Section 8: Signalling characteristics. 8.1.2 Requirements for SSB based radio link monitoring; 8.3 SCell Activation and Deactivation Delay; 8.5.2 Requirements for SSB based beam failure detection; 8.5.5 Requirements for SSB based candidate beam detection; 8.9 NR-DC: PSCell Addition and Release Delay; 8.10 Active TCI state switching delay; 8.11 PSCell Change;

- Section 9: Measurement Procedure. 9.2 NR intra-frequency measurements (including cell identification, measurement);9.3 NR inter-frequency measurements (including cell identification, measurement); 9.5 L1-RSRP measurements for Reporting; 9.8 L1-SINR measurements for Reporting; 9.13 L1-RSRP measurements for a cell with different PCI from serving cell [inter-cell, mTRP].

Based on the agreements in RAN1, we can be sure that at current stage, the Rel-19 NES-capable UE’s SCell NCD-SSBs are supported. That means the NW can adapt NCD-SSB transmission in the SCell for L1/L3 measurements including L3/L1/RLM measurements, and SCell activation when the UE is configured with CA. From this, the SSB adaptation impact in RRM requirements related to SCell based on NCD-SSBs can be discussed.

Proposal 1: Rel-19 NES-capable UE’s SCell (s) are supported for adaptation of NCD-SSB in time-domain

Proposal 2: In connected mode, the impact on the following RRM requirements can be studied on adaptation of SSB in time domain with high priority under the applicable scenario

- L1 measurements including L1-RSRP/BFD/CBD/L1-SINR

- L3 measurements including intra-frequency and inter-frequency

  - Measurement requirement of SCC with active SCell and the measurement requirement of SCC with deactivated SCell

  - FFS: Known and unknown SCell

- SCell activation delay

Meanwhile, based on the agreements and discussions above mentioned, so far, although RAN1 reached alignment that the SCell scenarios shall be considered, they cannot reach consensus on whether PCell shall be considered. From this,

Proposal 3: RAN4 to wait for RAN1 progress whether: PCell are supported for adaptation mechanisms of SSB in time-domain

    - SCell activation and deactivation delay impact

From our understanding, SSB on-demand transmission is one kind of SSB adaptation transmission mechanisms from the rationale perspective, which mainly focus on SCell operation and is most likely triggered by NW with an on-demand SSB request. The NW will trigger the on-demand denser SSB transmission (shorter SSB periodicity) for a certain duration for SCell activation and measurements. Actually, if we compare the configuration of on-demand SSB for SCell operation to the configuration of adapted SSB, we can find that 1) On-demand SSB case 2 assumption is somewhat identical to SSB adaptation configuration option 2 when more than one on-demand SSB configurations are configured for the cell to UE, as long as we take A. the default SSB in option 2 as the always-on SSB and B. the adapted SSB and the on-demand SSB as the additional SSB resources; 2) At the same time for option 1, there is a possibility that the SSB adaptation can be done with the indication of on-demand SSB transmission, as long as the indication content contains not only the transmission availability but also the denser periodicity to be used.

Besides, based on the following SCell activation figures, we can find that in practice, for SSB adaptation SCell operation, the UE can receive the adaptation indication at #1 Before SCell activation command reception (SCell deactivate state) #2 Along with MAC-CE activation command; #3 During SCell activation procedure but after MAC-CE activation command reception; #4 After/When SCell activation is completed and SCell is activated. The similar procedure as on-demand SSB based SCell activation

Fig.1 SSB adaptation SCell operation

Furthermore, it was agreed in RAN1#117 and RAN1 #118 that MAC CE based signaling to indicate on-demand SSB transmission on the cell.

Considering all the commonality discussed above, we think the MAC CE based SCell activation case can also be considered as the starting point for SSB adaptation SCell operation.

However, how to indicate SSB adaptation, by RRC/MAC-CE/DCI is not decided yet in RAN1. From our understanding, at least for SCell operation perspective, maybe an unified MAC-CE framework for on-demand SSB and adaptation SSB is feasible and more efficient, since they have a communal goal: to fast SCell activation procedure with denser SSB periodicity.

While, some companies raised that it will likely be able to use new SSB burst with compact SSB pattern mechanism to reduce the SCell activation delay, but it is still under discussion

Observation 3: The following issues were discussed but without any clear conclusions in RAN1

- How NW to indicate SSB adaptation, by RRC/MAC-CE/DCI

- How to adapt SSB transmission to reduce the SCell activation delay

- Unified MAC-CE framework for on-demand SSB and adaptation SSB

From this, we have

Proposal 4: For Rel-19 NES-capable UE’s SCell

- RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

- RAN4 to study the SCell activation and deactivation delay impact.

  - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

    - SSB adaptation scenario in time domain

The two options come from the agreements achieved in RAN1 #117, copied as below

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

From our understanding, from the whole procedure perspective, both of the two options are the same: the NES-capable UE gets the additional SSB configuration from RRC, and then gets the adaptation indication from L1/L2 signalling. The main difference lies on what solution to do the adaptation. For example,

- For pure option 1, one solution is that one configuration is provided to NES-capable UE initially, and DCI/MAC-CE may carry the indication/configuration of which SSB configuration subset to be used/not to be used to perform the adaptation. In this sense, maybe additional processing time is needed

- For pure option 2, one solution is that multiple configurations are provided to NES-capable UE through RRCReconfiguration initially, one configuration is taken as the default one. When it comes to adaptation, gNB activate/deactivate the additional configuration. From this, seems that option 2 can enable more scheduling flexibility, better UE experience and more NW energy saving gain.

- As the RAN1 agreement says, Option 1 and option 2 are not completely separated, option 2 can be deemed one of the solutions/implementations to realize option 1 for some functionalities/in some scenarios.

Observation 4：The whole procedure of the two options are the same, that is, the NES-capable UE gets the additional SSB configuration from RRC, and then gets the adaptation indication from L1/L2 signalling

Observation 5: Option 2 can enable more scheduling flexibility, better UE experience and more NW energy saving gain, and it is one of the solutions to realize option 1 for some functionalities

Since some issues are under discussion in RAN1, including: option 1 applicable scenarios, Cell DTX for SSB adaptation, SSB transmission pattern of the two SSB configurations, we proposal

Proposal 5: Suggest to wait for RAN1 progresseses and agreements on the applicable scenarios and functionalities of option 1 and option 2 for SSB adaptation

    - SSB burst adaptation periodicity

In last meeting, some companies mentioned whether to introduce longer SSB periodicity (> 160 ms) for SSB adaptation. Actually, this issue is also discussing in RAN1. Based on the simulation results provided in TR 38.864, longer SSB periodicity can bring significant BS energy saving gains, compared to 20ms SSB transmission periodicity baseline under most cases. From this, proponent think the longer periodicity (>160 ms) should be adapted based on the traffic load to maximize the saving gain, but no conclusion achieved yet.

From RAN4 perspective, in our opinion, longer SSB periodicity may bring interesting RRM requirements impacts, e.g., measurement period expansion, SCell activation delay enlargement, etc, depending on the modified TSSB, TSMTC_X related to new upper bound of transmission period of SSB. In this sense, the corresponding impacts on RRM requirements based on the introduced longer SSB adaptation periodicity shall be further discussed.

Proposal 6:

- Suggest RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements

- For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported

    - The transition period of SSB burst adaptation

From our understanding, the change of the SSB periodicity will present the transition of measurement procedure/state. After X ms period, UE can ready to receive the adapted/legacy SSB to measure. The corresponding RRM requirements including transition delay and/or interrupts at state transitions shall be discussed, accordingly,

Proposal 7:

- RAN4 to discuss the transition period requirements by SSB periodicity adaptation

  - FFS: Requirements on interrupts at state transitions

3.1.6 The triggered mechanism of SSB adaptation in time-domain

Based on the agreements achieved in RAN1 #116-bis, below

<#116-bis> Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

we can find that from RAN1 side, SSB adaptation triggered by NW was already taken as a preferred solution. Just, how to indicate the SSB adaptation by which mechanism (s) for Rel-19 NES-capable connected/idle mode UEs is not decided yet

For connected mode UE, the backup solution can be: 1) RRC-reconfiguration. Per-UE manner; 2) L1/L2 signalling

Furthermore, the indication via dynamic signalling in idle mode is considered as a promising way and extensively discussed.

Observation 6: Based on RAN1 agreements, SSB adaptation triggered by NW is taken as a preferred solution no matter for idle mode UE or connected mode UE

- For connected mode UE, the Alt.s can be RRC-reconfiguration, MAC-CE and DCI

- For idle mode UE, the Alt.s can be SIB and DCI

Proposal 8: RAN4 to focus on NW controlled SSB adaptation as starting point

3.1.7 Miscellaneous in SSB adaptation

Before going deeper into the analysis of RRM requirements impact, we do feel compelled to provide some discussions on the deployment (intra-cell/inter-cell) assumption, and which applicable case(s) can be determined to define RRM requirements.

- Deployment scenario 1: Intra-cell

Based on this, from our understanding, the intra-cell can be splitted into two applicable cases, only single TRP is, show as below

- Serving cell with only Rel-19 NES-capable UE   (b)  Serving cell with both legacy and Rel-19 NES-capable UE

Fig.2 Applicable cases for intra-cell deployment assumption

- Applicable case 1-1: NES only cell case. Only Rel-19 NES-capable UE is served in the cell area. The legacy UEs can be barred based on NW choice or are forbidden to camp on the NES cell via other ways. In such case, the new NES-specific requirements (if any) can be defined following the same basic logic considered for legacy UE.

- Applicable case 1-2: NES cell case. Both legacy and Rel-19 NES-capable UEs are served in the cell area. Adaptation based on configuration of additional SSB/Paging/PRACH resources for Rel-19 NES-capable UE in addition to the resources for legacy UE. That is the Rel-19 NES-capable UE can “see” both adaptation resource configurations and legacy resource configurations (i.e., the resources for legacy UE), while the legacy UE can only “see” legacy resource configurations. In such applicable case, since two kinds of resources the NES-capble UE can receive, the new UE behavior, miscellaneous factors and new measurement rules will be introduced case by case. Snce up to two configurations can be active, the new occasion/resources overlapping issues (e.g., PRACH) may arise if the additional adaptation resources overlap with legacy resources.

Observation 7: Based on RAN1 agreements, there are two applicable cases for intra-cell deployment. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE

- Deployment scenario 2: Inter-cell

From our understanding, there could be an applicable case for inter-cell deployment is that Rel-19 NES (only) cell within the coverage of sync normal cell 1, show as below. In such case,

- If the cell 2 is a NES only cell, the Rel-19 NES-capable UE can derive adaptation resources configurations of cell 2, meanwhile, they can also derive legacy resources configurations of normal cell 1.

- If the cell 2 is a NES cell, the Rel-19 NES-capable UE can derive both additional adaptation resources and legacy resources configurations of cell 2, meanwhile, they can also derive legacy resources configurations of normal cell 1.

Fig.3 Applicable cases for inter-cell deployment assumption

In summary, considering different deployments and applicable cases may bring different/new impacts on RRM requirements, we prefer that

Proposal 9: For adaptation of common signal/channel transmissions, RAN4 to discuss:

- Which deployment assumption should be considered to define the new requirements (if any)

  - Intra-cell and/or inter-cell deployment

- For the deployment assumption, which applicable case should be considered to define the new requirements (if any)

  - Serving cell with only Rel-19 NES-capable UE and/or serving cell with both legacy and Rel-19 NES-capable UE

3.1.8 UE behaviour in NES cell which transmits both legacy SSBs and NES-based SSB adaptation

From our understanding, if NES only cell case (i.e., Serving cell with only Rel-19 NES-capable UE cell) is considered, there is no need to consider the corresponding UE behaviour, NES-capable UE shall measure the NES-based SSB resources always. If NES cell case (i.e., Serving cell with both legacy and Rel-19 NES-capable UE). The UE behaviours can be:

- NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

- Both SSB resources (legacy SSBs and adapted SSBs) the NES-capble UEs can to measure. While, the new measurement rules, the combination rules of measurement results based on legacy SSBs and adapted SSBs shall be further discussed in RAN4

- Based on UE implementation

Proposal 10: Regarding UE behavior under NES scenario.

- If NES only cell case is considered. The NES-capable UE shall measure the NES-based SSB resources always

- If NES cell case is considered. The candidate UE behaviours can be

  - NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

  - Both SSB resources (legacy SSBs and adapted SSBs) the NES-capble UEs can to measure. New measurement rules and the measurement results combination rules shall be discussed in RAN4

  - Based on UE implementation

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on common signals/channels adaptation. The following observations and proposals are obtained:

Observation 1: Based on RAN1 agreements on SSB adaptation.

- PCell and/or SCell (s). Rel-19 NES-capable UE’s PCell CD-SSB on sync raster is not supported, FFS: NCD-SSB or SSB not on sync raster; Rel-19 NES-capable UE’s SCell NCD-SSBs are supported, FFS: CD-SSB with > 20 ms periodicity

- FFS: Whether to support Rel-19 NES-capable UE in idle/inactive mode

- FFS: Additional SSB resources configuration/SSB transmission pattern for different adaptation mechanism

Observation 2: Based on RAN1 agreements on PRACH adaptation.

- DCI-based adaptation for additional PRACH resources. FFS: The details

- Extend the RAN1#117 agreement on SSB-RO mapping rule to no time-domain overlap and time-domain overlap but no overlap in frequency domain cases

- FFS: Additional PRACH resources configurations

Observation 3: The following issues were discussed but without any clear conclusions in RAN1

- How NW to indicate SSB adaptation, by RRC/MAC-CE/DCI

- How to adapt SSB transmission to reduce the SCell activation delay

- Unified MAC-CE framework for on-demand SSB and adaptation SSB

Observation 4：The whole procedure of the two options are the same, that is, the NES-capable UE gets the additional SSB configuration from RRC, and then gets the adaptation indication from L1/L2 signalling

Observation 5: Option 2 can enable more scheduling flexibility, better UE experience and more NW energy saving gain, and it is one of the solutions to realize option 1 for some functionalities

Observation 6: Based on RAN1 agreements, SSB adaptation triggered by NW is taken as a preferred solution no matter for idle mode UE or connected mode UE

- For connected mode UE, the Alt.s can be RRC-reconfiguration, MAC-CE and DCI

- For idle mode UE, the Alt.s can be SIB and DCI

Observation 7: Based on RAN1 agreements, there are two applicable cases for intra-cell deployment. 1) Serving cell with only Rel-19 NES-capable UE; 2) Serving cell with both legacy and Rel-19 NES-capable UE

Proposal 1: Rel-19 NES-capable UE’s SCell (s) are supported for adaptation of NCD-SSB in time-domain

Proposal 2: In connected mode, the impact on the following RRM requirements can be studied on adaptation of SSB in time domain with high priority under the applicable scenario

- L1 measurements including L1-RSRP/BFD/CBD/L1-SINR

- L3 measurements including intra-frequency and inter-frequency

  - Measurement requirement of SCC with active SCell and the measurement requirement of SCC with deactivated SCell

  - FFS: Known and unknown SCell

- SCell activation delay

Proposal 3: RAN4 to wait for RAN1 progress whether: PCell are supported for adaptation mechanisms of SSB in time-domain

Proposal 4: For Rel-19 NES-capable UE’s SCell

- RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

- RAN4 to study the SCell activation and deactivation delay impact.

  - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements

Proposal 5: Suggest to wait for RAN1 progresseses and agreements on the applicable scenarios and functionalities of option 1 and option 2 for SSB adaptation

Proposal 6:

- Suggest RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements

- For discussion on RRM requirements impacts based on longer SSB adaptation periodicity can wait for RAN1 progress on whether longer SSB burst adaptation periodicity can be supported

Proposal 7:

- RAN4 to discuss the transition period requirements by SSB periodicity adaptation

  - FFS: Requirements on interrupts at state transitions

Proposal 8: RAN4 to focus on NW controlled SSB adaptation as starting point

Proposal 9: For adaptation of common signal/channel transmissions, RAN4 to discuss:

- Which deployment assumption should be considered to define the new requirements (if any)

  - Intra-cell and/or inter-cell deployment

- For the deployment assumption, which applicable case should be considered to define the new requirements (if any)

  - Serving cell with only Rel-19 NES-capable UE and/or serving cell with both legacy and Rel-19 NES-capable UE

Proposal 10: Regarding UE behavior under NES scenario.

- If NES only cell case is considered. The NES-capable UE shall measure the NES-based SSB resources always

- If NES cell case is considered. The candidate UE behaviours can be

  - NES-capable UEs to measure these additional NES-based SSB resources with the highest priority

  - Both SSB resources (legacy SSBs and adapted SSBs) the NES-capble UEs can to measure. New measurement rules and the measurement results combination rules shall be discussed in RAN4

  - Based on UE implementation

4 References

[1] R4-2413885, “WF on Netw_Energy_NR_enh”, Ericsson