# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416142

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:			Ericsson

Title:			NTN Ku-band – Regulatory aspects and band(s) definition

Agenda item:		6.9.1

Document for:		Discussion

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to define NTN bands to cover the different frequency ranges proposed in the WI.

In last RAN4#112 meeting, we submitted a first contribution ([2]) which mainly focused on CEPT regulation. And a first Ku-band was then agreed for CEPT countries ([3]). In this contribution, we are investigating additional worldwide regulations and check which band(s) could be defined based on them. Note that other regulations should be further investigated to conclude on the band plan for the NTN Ku-band.

# Discussion

## WI objectives

The NTN Ku band WI lists the following objectives related to Ku-band definition:

From the WI objectives, reading the 2nd and 4th bullet, the WI gives the appearance that US was excluded from the Ku-band(s) definition in the scope of this WI. But, after further checking, it seems US should still be considered for the 10.7-12.7 GHz (S-to-E) / 13.75-14.50 GHz (E-to-S) frequency ranges defined by ITU for Region 2.

Proposal1: Confirm that USA is in the scope of Ku-band(s) definition for the 10.7-12.7 GHz (S-to-E) / 13.75-14.50 GHz (E-to-S) frequency range.

## Region 2

Looking at ITU Radio Regulation, the 12.2-12.7 GHz frequency range for Region 2 is allocated to Broadcasting-satellite service and not fixed-satellite, which might question the relevancy of this frequency range for NTN operation.

Nevertheless, the footnote 5.492 mentions that this frequency range could still be used for fixed-satellite service with the condition this doesn’t create more interference, nor require more protection.

Observation: According to ITU Radio Regulations, the 12.2-12.7 frequency range is allocated to Broadcasting-Satellite. Still, the footnote 5.492 states that “may also be used for transmissions in the fixed-satellite service (space-to-Earth), provided that such transmissions do not cause more interference, or require more protection from interference”.

### USA

In US, the Ku-band and extended Ku-band are allocated to Fixed Satellite /Broadcast Satellite and partly to Fixed service (10.7-11.7 GHz and 12.2-12.75 GHz) as primary, as shown on following Figure 1.

Figure 1: USA Frequency Allocation - Ku-band

Also, the 13.75-14 GHz frequency range is submitted to additional restrictions on minimum antenna diameter, minimum and maximum EIRP.

ESIM operations are allowed in the following frequency ranges (CFR 47 25.202):

- 10.7-11.7 GHz (s-to-E) for both GSO and NGSO.

- 11.7-12.2 GHz (s-to-E) for both GSO and NGSO.

- 14.0-14.5 GHz (E-to-s) for both GSO and NGSO.

But under the condition of footnote NG527A, which stipulates that ESIM operation in 10.7-11.7 GHz frequency range may be authorized under the conditions that those earth stations don’t claim protection from fixed service.

### Canada

In Canada, the Ku-band is allocated to Fixed Satellite /Broadcast Satellite and partly to Fixed service (10.7-11.7 GHz and 12.2-12.75 GHz) as primary, as shown on following Figure 2.

Figure 2: Canada Frequency Allocation - Ku-band

Technical Requirements for Fixed Earth Stations operating above 1 GHz in Space Radiocommunication Services and Earth Stations in Motion (ESIMs) Operating in the Fixed-Satellite Service are specified in SRSP-101.

ISED also published its Decision on Updates to the Licensing and Fee Framework for Earth Stations and Space Stations in Canada (SMSE-008-22) in which the list of bands eligible for generic licensing is specified in table D1.

As it can be observed, Earth Station shall not create interference nor claim for any protection to/from the other licensed services in the frequency range 10.7 – 10.95 GHz (S-to-E).

Also, land ESIM is not allowed in that 10.7 – 10.95 GHz frequency range, nor in the 11.2-11.45 GHz frequency range. It should be noted that land ESIM is the type operation which is covered in the coexistence study, similar to what was done for bands n511 and n510.

### Initial band(s) proposal for Region 2

ESIM operation is allowed in the 10.7-11.7 GHz frequency range, but it shall no claim for any protection nor create harmful interference in the 10.7-11.7 GHz (USA) or 10.7-10.95 GHz (Canada) frequency range.

It might be possible to consider a single band: 10.7-12.7 GHz (s-to-E) and 14.0-14.5 GHz (e-to-S).

Except the last 50MHz in s-to-E spectrum, this would be aligned with the band previously agreed for CEPT countries. Still, no protection for ESIM operation should be claimed in the 10.7-11.7 GHz (USA) or 10.7-10.95 GHz (Canada), and no land ESIM operation is allowed in the 10.7 – 10.95 GHz and 11.2-11.45 GHz frequency ranges (Canada).

## Region 3

### Japan

In Japan, the Ku-band is allocated to Fixed Satellite /Broadcast Satellite and partly to Fixed service (10.7-11.7 GHz and 12.2-12.75 GHz) as primary, as shown on following Figure 3.

Figure 3: Japan Frequency Allocation - Ku-band

According to J204, the 11.7-12.2 GHz frequency range is only allocated for NGSO operation.

In the 12.2-12.5 GHz frequency range, VSAT shall not claim any protection of fixed service operating in the frequency band (J208):

### Australia

In Japan, the Ku-band is allocated to Fixed Satellite /Broadcast Satellite and partly to Fixed service (10.7-11.7 GHz) as primary, as shown on following Figure 4.

Figure 4: Australia Frequency Allocation - Ku-band

Also, according to ACMA ([4]), “under the Communications with Space Objects Class Licence, the operation of earth station receivers is authorised on the basis of no protection is afforded from interference caused by a point-to-point station in the frequency bands 10.7-11.7 GHz, 18.2-18.8 GHz and 19.3-19.7 GHz”.

### Initial band(s) proposal for Region 3

It might be possible to consider a single band: 10.7-12.75 GHz (s-to-E) and 14.0-14.5 GHz (e-to-S).

This would be aligned with the band previously agreed for CEPT countries, but VSAT shall not claim protection in the 12.2-12.5 GHz (Japan) or 10.7-11.7 GHz (Australia).

## Summary on band(s) proposal

Proposal2: Consider the band 10.7-12.7 GHz (s-to-E) and 14.0-14.5 GHz (e-to-S) for North America, and reflect in the specifications that no protection should be claimed for ESIM operation in part of the 10.7-11.7 GHz frequency range and that land ESIM could not be authorized in the 10.7 – 10.95 GHz and 11.2-11.45 GHz frequency range.

Proposal3: Consider the band 10.7-12.75 GHz (s-to-E) and 14.0-14.5 GHz (e-to-S) for Japan and Australia, reflecting in the specifications that no protection should be claimed in the 10.7-11.7 GHz and the 12.2-12.5 GHz frequency range.

Proposal4:  Further investigate the possible band plan for the NTN Ku-band based on other regulations and consider if the same band plans and conditions apply at a regional level.

# Conclusion

In this contribution, we investigated the Regulations in Region 2 (USA and Canada) and in Region 3 (Japan and Australia) related to the NTN Ku-band. We made the following proposals and observations:

Proposal1: Confirm that USA is in the scope of Ku-band(s) definition for the 10.7-12.7 GHz (S-to-E) / 13.75-14.50 GHz (E-to-S) frequency range.

Observation: According to ITU Radio Regulations, the 12.2-12.7 frequency range is allocated to Broadcasting-Satellite. Still, the footnote 5.492 states that “may also be used for transmissions in the fixed-satellite service (space-to-Earth), provided that such transmissions do not cause more interference, or require more protection from interference”.

Proposal2: Consider the band 10.7-12.7 GHz (s-to-E) and 14.0-14.5 GHz (e-to-S) for North America, and reflect in the specifications that no protection should be claimed for ESIM operation in part of the 10.7-11.7 GHz frequency range and that land ESIM could not be authorized in the 10.7 – 10.95 GHz and 11.2-11.45 GHz frequency range.

Proposal3: Consider the band 10.7-12.75 GHz (s-to-E) and 14.0-14.5 GHz (e-to-S) for Japan and Australia, reflecting in the specifications that no protection should be claimed in the 10.7-11.7 GHz and the 12.2-12.5 GHz frequency range.

Proposal4:  Further investigate the possible band plan for the NTN Ku-band based on other regulations and consider if the same band plans and conditions apply at a regional level.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- R4-2411190, NTN Ku-band – Regulatory aspects and band(s) definition, Ericsson

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General

- Business operating procedure Submission and processing of applications for space and space receive apparatus licences, DECEMBER 2023