3GPP TSG-RAN WG4 Meeting #112bis	R4-2416191

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Defining RRM scope of work for less than 5 MHz

Agenda item:	6.8.3.4

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

The initial discussion was initiated in the last meeting and a WF was proposed in [2]. In this contribution we discuss some aspects related to the work of specifying less than 5 MHz operation for NTN and provide our thoughts regarding current open issues on this WI.

Discussion

In the past meeting contributions and discussions, it seems to be a consensus among the group that the RRM requirements to be most impacted by the operation with less than 5 MHz in NTN are the same impacted by the less than 5 MHz in terrestrial networks, which are:

- Index detection in connected mode mobility (handovers)

- RLM/BFD monitoring  (due to reduced bandwidth)

- HO delay requirements for unknown target cell in connected mode mobility

When we look at the WF [2] from previous meeting, there is a table which tends to evaluate which sections of the RRM specification are  to be impacted in this WI.

Issue 1-2-3: On top of which requirements in TS38.133 for NTN less than 5MHz Moderator proposal: Companies to discuss the potential impacts based on the content in the following table. RRM impacts summary due to spectrum less than 5MHz RRM requirements; NTN requirements in TS38.133 v18.5.0; Possible impacts if BW below 5MHz. IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access; TBD. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access; TBD. Handover; 6.1C Handover for SAN; TBD. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C; TBD. RLM; 8.1C Radio Link Monitoring for Satellite Access 8.5C Link Recovery Procedures for Satellite Access; TBD. Others, if any.

In order to optimize the work flow and to make the WI more productive and efficient, we propose that RAN4 narrows down the focus of the work tor the requirements which some impact is already known, which would not preclude that other requirements are also impacted if an impact is identified:

Prioritize the discussion of the following requirements for less than 5 MHz operation in NTN.

  - Index detection in connected mode measurements (intra and inter-frequency measurements)

  - RLM/BFD monitoring

  - TΔ in HO delay requirements

Index detection requirements

Concerning the requirements for index detection in operation with less than 5 MHz in NTN (clauses 9.2C and 9.3C in TS 38.133), the principles adopted for terrestrial operations shall be maintained. As discussed in details in [3] and [4], the main change in the index detection of neighbor cells is caused by PBCH puncturing. Because of the puncturing, the UE is allowed more time to complete cell index detection and therefore the time for detection has been enlarged by considering the UE might need more samples to complete the measurements. The following agreement was made for the less than 5MHz operation in terrestrial networks:

Agreement: - SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

For NTN, the same principle applies: because of the PBCH puncturing, when 12 PRBs are used, the number of samples need to be enlarged.

Therefore, we propose:

For measurements towards a target cell with 12-PRBs SSB, the time period for time index detection shall follow TN agreements, i.e.:

  - Unknown intra-frequency cell [3+4] samples

  - Unknown inter-frequency cell [3+3] samples

One practical aspect of the proposal above is that for most cases, in TN, several tables were implemented for updating this requirement for different scenarios. This has created a large number of tables to be maintained, and also makes it difficult to combine features. For example, if a new table is crated for Time index indetification for less than 5 MHz in NTN, then it might make more difficult to utilize 5 MHz operation with other features in the future (which would require new different tables, one for each combination). The specification practices in RAN4 has also been under scrutiny and review, as RAN4 tries to improve the quality of its specification to avoid too much repetition and unnecessary enlarging of the specification document. Based on this, we propose that:

For time index identification (intra and inter frequency) utilize the existing tables for NTN requirements to introduce the new requirements for less than 5 MHz.

RLM/BFD monitoring

Another issue indicated in the WF and discussed by many companies is the impact in the RLM/BFD monitoring that is caused by a reduced transmitting bandwidth configuration (Tx BW). With the reduced BW configuration, there needs to be an adaptation of the PDCCH parameters (for example the CCE to REG mapping or the aggregation level) such that the entirety of the PDCCH can be accommodated in the bandwidth allocated for Tx Transmission. With such modified parameters, the levels of Qin and Qout used for link monitoring will be different in practice compared to legacy operation (above 5 MHz).

There is no further impact in the PDCCH decoding caused by NTN implementation or by NTN features, therefore, the definitions for less than 5MHz operation in terrestrial networks might be applied directly to NTN.

Adopt the same set of PDCCH parameters for hypothetical PDCCH BLER for RLM and BFD requirements used in terrestrial networks for the less than 5 MHz operation.

Conclusion

The paper has analyzed the initial scope of the work for adopting the less than 5 MHz for the NTN UEs in FR1 and the following proposals were made.

Proposal 1: Prioritize the discussion of the following requirements for less than 5 MHz operation in NTN.

a.	Index detection in connected mode measurements (intra and inter-frequency measurements)

b.	RLM/BFD monitoring

c.	TΔ in HO delay requirements

Proposal 2: For measurements towards a target cell with 12-PRBs SSB, the time period for time index detection shall follow TN agreements, i.e.:

a.	Unknown intra-frequency cell [3+4] samples

b.	Unknown inter-frequency cell [3+3] samples

Proposal 3: For time index identification (intra and inter frequency) utilize the existing tables for NTN requirements to introduce the new requirements for less than 5 MHz.

Proposal 4: Adopt the same set of PDCCH parameters for hypothetical PDCCH BLER for RLM and BFD requirements used in terrestrial networks for the less than 5 MHz operation.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2413974, "WF for RRM of NTN with channel BW less than 5MHz", Moderator (Xiaomi), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- R4-2413189, “Support of less than 5MHz for NR NTN”, Qualcomm, RAN4 #112, Maastricht, Netherlands, 19th – 23rd August, 2024

- R4-2413043, “Discussion on RRM requirements for NTN enhancement”, ZTE Corporation, Sanechips, RAN4 #112, Maastricht, Netherlands, 19th – 23rd August, 2024