3GPP TSG-RAN WG4 Meeting #112bis	R4- 2416435

Hefei, China, October 14 – 18, 2024

Source:	Nokia

Title:	RRM performance requirements for NR Carrier Phase Positioning

Agenda item:	4.2.2.6

Document for:	Discussion

Introduction

The specification of NR carrier phase positioning (NR CPP) is one of the objectives in the Rel-18 WID on expanded and improved positioning [1]. RRM performance part of the NR_pos_enh2 WI was closed at RAN4 #112.Nevertheless, some aspects still need to be finalized.

This contribution provides our view on open issues for RRM performance requirement for NR CPP [2].

Discussion

RAN4 agreements and open issues on performance requirements

RAN4 agreements and list of open issues from RAN4 #112 are captured below [2]. More details are available in [3].

## Sub-topic 2-2 Carrier Phase Positioning Performance Requirements (agenda 6.1.2.6) ### Issue 2-2-1: Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs Agreements: - For the CPP test case, both RSTD/Rx-Tx measurement and CPP reporting are configured, and TE check the success rate for CPP reporting. - If UE supports CPP measurement, UE shall also pass the test case for RSTD/Rx-Tx measurement. ### Issue 2-2-2: Additional margins due to frequency drift and RF calibration Agreements: - RAN4 to clarify that the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols, which can be controlled at least by the window configuration. - Companies can bring proposals on other scenarios in WI performance maintenance phase.

  - Verification of legacy measurements for CP measurements

Regarding issue 2-2-1, it is noted that the agreement is not yet reflected in the big CR [4] submitted to RAN #105. Thus, following changes are required for FR1 in clause A.6.7.19.1 for RSCPD with RSTD and clause A.6.7.20.1 for DL RSCP with UE Rx-Tx time difference and corresponding changes for FR2.

### A.6.7.19	RSCPD Measurements

#### A.6.7.19.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_CONNECTED

##### A.6.7.19.1.1	Test purpose and environment

The purpose of the test is to verify that RSCPD with RSTD measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.Y1.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.7.19.1.1-1.

Table A.6.7.19.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation with nr-DL-PRS-RSCPD-Request from LMF via LPP [34] as defined in TS 37.355 [34, clause 6.5.12.] to enable UE to perform and report RSCPD in RRC CONNECTED, shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

##### A.6.7.19.1.3	Test requirements

The RSCPD reported together with RSTD fulfils RSCPD with RSTD measurement accuracy specified in clause 10.1.Y1.2 for both Cell 1 and Cell 2.

The UE shall also pass the applicable RSTD measurement accuracy test, i.e. the test in clause A.6.7.13.1 for single PFL, clause A.6.7.13.2 for dual PFL, clause A.6.7.13.3 for reduced number of samples for single PFL or clause A.6.7.13.4 with Rx TEG, respectively.

### A.6.7.20	RSCP Measurements

#### A.6.7.20.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA

##### A.6.7.20.1.1	Test purpose and environment

The purpose of the test is to verify that DL RSCP with UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.Z1.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.7.20.1.1-1.

Table A.6.7.20.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData , NR-Multi-RTT-RequestLocationInformation with nr-DL-PRS-RSCP-Request from LMF via LPP [34] and NR-Multi-RTT-MeasurementCapability as defined in TS 37.355 [34, clause 6.5.12.] to enable UE to perform and report RSCP in RRC CONNECTED, shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.20.1.3	Test requirements

The DL RSCP with UE Rx-Tx time difference measurement fulfils DL RSCP with UE Rx-Tx measurement accuracy specified in clause 10.1.Z1.2 for both Cell 1 and Cell 2.

The UE shall also pass the applicable UE Rx-Tx time difference measurement accuracy test, i.e. the test in clause A.6.7.15.1 for single PFL, clause A.6.7.15.2 for reduced number of samples for single PFL or clause A.6.7.15.3 with RxTx TEG, respectively.

The above revisions are included in the accompanying CR [5].

Agree the changes in R4-2416436 related to additions on the verification of the legacy measurement accuracy in case of CPP.

  - Accuracy performance for additional PRS configurations

It was observed at RAN4 #112 that accuracy requirements for CP measurements (RSCPD/DL RSCP) for some PRS configurations were missed out in the simulations. In particular, accuracy requirements for the following highlighted configurations in Table 1 need to be determined to update the big CR [4].

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  |  | 132 | 1 | 32 |
|  | 60, FR1 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1 |  |  |  |
| Es/Iot for three cells (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13)dB for NSample = 1; (-3, -6, -6)dB for NSample = 1. (-3, -13, -13)dB for NSample = 1; (0, -6, -6)dB for NSample = 1. |  |  |  |
| PRS periodicity | 40 ms |  |  |  |
| Network synchronization | Synchronous with time shifts <0, 0, 3 us> |  |  |  |

Table 1: PRS configurations used for NR RSCPD study for SCS = 15kHz, 30kHz, 60kHz and 120 kHz (FR1 and FR2).

For these additional PRS configurations, we provide our simulation results in [6].

This yields to following proposal.

Take into account simulation results in R4-2416437 for the additional PRS configurations to determine remaining CP measurement accuracy requirements in the big CR.

Additional RF margin for frequency drift and RF calibration

Regarding issue 2-2-2, the impact of CFO has been discussed for a number of meetings. Yet, there is no evidence in our view that this impact is negligible. According to our investigation in [6], which is based on residual CFO assumptions in TR 38.859, as investigated by RAN1 during the study of NR CPP, we observe a non-negligible impact to the measurement accuracy at target UE, which may be mitigated to some degree by double differential carrier phase estimation at LMF, but there is a significant impact to the accuracy of the measurements reported by target UE or PRU, respectively. In particular, the impact depends on the residual CFO magnitude, as depicted in [6]. We propose to add an additional RF margin based on the worst residual CFO setting (30Hz).

Results related to CFO impact from [6] are duplicated below.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | Carrier frequency offset Uniform distribution within [-CFO, +CFO] (Hz) | RSCPD (cell-1, cell-2) (-3, -6) dB, 0 μs |  | RSCPD (cell-1, cell-3) (-3, -6) dB, 3 μs |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | 0 | -3.43 | 3.1465 | -3.3991 | 3.2851 |
|  |  |  |  |  |  | 10 | -13.098 | 13.0452 | -13.1672 | 13.0822 |
|  |  |  |  |  |  | 30 | -38.0144 | 38.124 | -37.9715 | 37.7174 |
|  |  | 52 | 128 | 1 | 1 | 0 | -5.4249 | 5.176 | -4.8311 | 5.1138 |
|  |  |  |  |  |  | 10 | -25.8386 | 25.9488 | -25.7816 | 25.6056 |
|  |  |  |  |  |  | 30 | -76.2626 | 75.4297 | -76.1049 | 76.1096 |
|  |  | 104 | 64 | 1 | 1 | 0 | -3.785 | 3.6766 | -3.6592 | 3.7185 |
|  |  |  |  |  |  | 10 | -50.3115 | 50.9978 | -50.9346 | 50.5281 |
|  |  |  |  |  |  | 30 | -150.3907 | 152.645 | -152.2091 | 151.3301 |
|  | 4 | 24 | 256 | 4 | 1 | 0 | -3.3039 | 3.3657 | -3.4974 | 3.3261 |
|  |  |  |  |  |  | 10 | -13.2597 | 13.0465 | -12.9865 | 13.1022 |
|  |  |  |  |  |  | 30 | -37.7498 | 37.9398 | -37.876 | 38.2247 |
|  |  | 52 | 128 | 1 | 1 | 0 | -5.3824 | 5.0907 | -5.2903 | 5.1927 |
|  |  |  |  |  |  | 10 | -25.9525 | 25.6621 | -25.8651 | 25.7856 |
|  |  |  |  |  |  | 30 | -76.3308 | 75.7829 | -75.7985 | 76.1889 |
|  |  | 104 | 64 | 1 | 1 | 0 | -3.5021 | 3.9081 | -3.5395 | 3.7459 |
|  |  |  |  |  |  | 10 | -50.4416 | 50.7178 | -51.0415 | 50.6541 |
|  |  |  |  |  |  | 30 | -151.7979 | 152.3772 | -150.685 | 152.6669 |
| 30 | 2 | 24 | 128 | 4 | 1 | 0 | -3.3571 | 3.2634 | -3.4436 | 3.1603 |
|  |  |  |  |  |  | 10 | -25.4747 | 25.2721 | -25.476 | 25.3186 |
|  |  |  |  |  |  | 30 | -76.135 | 75.7619 | -76.4754 | 75.9004 |
|  |  | 48 | 64 | 1 | 1 | 0 | -4.9102 | 5.21 | -5.0967 | 5.4025 |
|  |  |  |  |  |  | 10 | -50.5251 | 50.6343 | -50.448 | 50.6174 |
|  |  |  |  |  |  | 30 | -152.4604 | 151.9876 | -152.7231 | 151.0814 |
|  | 4 | 24 | 128 | 4 | 1 | 0 | -3.43 | 3.1465 | -3.2591 | 3.1655 |
|  |  |  |  |  |  | 10 | -25.5483 | 25.5584 | -25.1741 | 25.4391 |
|  |  |  |  |  |  | 30 | -76.3208 | 76.1638 | -75.9138 | 74.9953 |
|  |  | 48 | 64 | 1 | 1 | 0 | -5.4639 | 5.5631 | -5.5513 | 5.5624 |
|  |  |  |  |  |  | 10 | -50.7366 | 50.8635 | -50.6201 | 50.9179 |
|  |  |  |  |  |  | 30 | -152.2208 | 151.5496 | -152.0633 | 151.7557 |
| 60 | 2 | 24 | 64 | 4 | 1 | 0 | -3.3982 | 3.3662 | -3.5209 | 3.3504 |
|  |  |  |  |  |  | 10 | -50.5787 | 50.2356 | -51.0181 | 50.1422 |
|  |  |  |  |  |  | 30 | -152.5238 | 151.3629 | -151.6489 | 152.3591 |
|  | 4 | 24 | 64 | 4 | 1 | 0 | -3.3571 | 3.2634 | -3.3858 | 3.2271 |
|  |  |  |  |  |  | 10 | -50.9792 | 50.9464 | -50.6128 | 50.394 |
|  |  |  |  |  |  | 30 | -151.5781 | 152.1275 | -151.4249 | 151.529 |

Table 19: DL RSCPD error for AWGN channel in degree with CFO and for Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.[6]

The results indicate that the error in timing accuracy depends on the magnitude of the residual CFO, the number of PRBs, whilst the carrier frequency impact in FR1 is negligible.

This yields to following proposal.

RAN4 to further investigate how to take into account the significant impact of CFO onto CPP accuracy (i.e. about 35 to 148 degrees in FR1) in the measurement accuracy requirements for RSCPD and DL RSCP, where the RF margin currently is set to TBD in following tables of the agreed (although not approved) big CR [4]:

Table 10.1.43.2-5: Margin for DL RSCPD measurement accuracy in FR1.

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | [TBD] |
| ≥ 52 | ≥ 24 | N/A | [TBD] |
| ≥ 104 | ≥ 48 | ≥ 24 | [TBD] |
| N/A | ≥ 132 | ≥ 64 | [TBD] |
| N/A | N/A | ≥ 132 | [TBD] |

Table 10.1.43.2-6: Margin for DL RSCPD measurement accuracy in FR2.

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | [TBD] |
| ≥ 64 | ≥ 32 | [TBD] |
| ≥ 132 | ≥ 64 | [TBD] |
| N/A | ≥ 128 | [TBD] |

Table 10.1.44.2-5: Margin for relative DL RSCP measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | [TBD] |
| ≥ 52 | ≥ 24 | N/A | [TBD] |
| ≥ 104 | ≥ 48 | ≥ 24 | [TBD] |
| N/A | ≥ 132 | ≥ 64 | [TBD] |
| N/A | N/A | ≥ 132 | [TBD] |

Table 10.1.44.2-6: Margin for relative DL RSCP measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | [TBD] |
| ≥ 64 | ≥ 32 | [TBD] |
| ≥ 132 | ≥ 64 | [TBD] |
| N/A | ≥ 128 | [TBD] |

Conclusion

Remaining open issues for RRM performance requirements for NR carrier phase positioning (NR CPP) being one of the objectives in the Rel-18 WID on expanded and improved positioning [1], as depicted in [2] and [4] are discussed in this contribution and corresponding proposals are made.

The following proposals are made.

- Agree the changes in R4-2416436 related to additions on the verification of the legacy measurement accuracy in case of CPP.

- Take into account simulation results in R4-2416437 for the additional PRS configurations to determine remaining CP measurement accuracy requirements in the big CR.

- RAN4 to further investigate how to take into account the significant impact of CFO onto CPP accuracy (i.e. about 35 to 148 degrees in FR1) in the measurement accuracy requirements for RSCPD and DL RSCP, where the RF margin currently is set to TBD in following tables of the agreed (although not approved) big CR [4]:

Table 10.1.43.2-5: Margin for DL RSCPD measurement accuracy in FR1.

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | [TBD] |
| ≥ 52 | ≥ 24 | N/A | [TBD] |
| ≥ 104 | ≥ 48 | ≥ 24 | [TBD] |
| N/A | ≥ 132 | ≥ 64 | [TBD] |
| N/A | N/A | ≥ 132 | [TBD] |

Table 10.1.43.2-6: Margin for DL RSCPD measurement accuracy in FR2.

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | [TBD] |
| ≥ 64 | ≥ 32 | [TBD] |
| ≥ 132 | ≥ 64 | [TBD] |
| N/A | ≥ 128 | [TBD] |

Table 10.1.44.2-5: Margin for relative DL RSCP measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (degree) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | [TBD] |
| ≥ 52 | ≥ 24 | N/A | [TBD] |
| ≥ 104 | ≥ 48 | ≥ 24 | [TBD] |
| N/A | ≥ 132 | ≥ 64 | [TBD] |
| N/A | N/A | ≥ 132 | [TBD] |

Table 10.1.44.2-6: Margin for relative DL RSCP measurement accuracy in FR2

| PRS BW (RB number) |  | Margin (degree) |
| --- | --- | --- |
| SCS=60kHz | SCS=120kHz |  |
| ≥ 24 | N/A | [TBD] |
| ≥ 64 | ≥ 32 | [TBD] |
| ≥ 132 | ≥ 64 | [TBD] |
| N/A | ≥ 128 | [TBD] |

- References

- RP-233382, “Revised WID on Expanded and Improved NR positioning”, 3GPP TSG RAN#102

- R4-2414033, “WF on SL positioning and carrier phase positioning”, CATT

- R4-2411806, “Topic summary for [112][211] NR_pos_enh2_part2”, Moderator (CATT)

- R4-2413983, “Big CR to 38.133 for RRM performance part for expanded and improved NR positioning”, Ericsson

- R4-2416436,”CR 38.133 Introduction of TCs on CPP measurement accuracy”, Nokia

- R4-2416437, “Simulation results for DL RSCPD and DL RSCP”, Nokia