3GPP TSG RAN WG4 #112bis													     R4-2416481

Hefei, CN, October 14th – 18th, 2024

Agenda Item:		6.8.3.1

Source:				Inmarsat, Viasat

Title:	Operator input for NR NTN Less than 5 MHz

Document for:	Discussion and Decision

# Background

Issue 1-2-3: Synchronization raster

- Agreement

  - Reuse TN sync raster for 3MHz channel bandwidth and extend to 3000MHz for NTN FR1 3MHz channel bandwidth as below table:

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |

  - Include note1:

      - NOTE 1:	Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.

During RAN4#112, there was a lengthy discussion on the 12 RB support and respective sync rasters, and the following agreement was reached.

However, this agreement, in  particular the text of “NOTE 1” does not fully reflect the ongoing discussion both during online and offline sessions, and the original intentions of this WI objective, where NTN operators clearly expressed the need and priority to support 12 PRB configuration.  We thereby propose that the note should be revisited, or at least clarified.

# Discussion

The rationale for support of NR NTN Less than 5 MHz was extensively discussed in several RAN Plenary meetings and ultimately approved at RAN#103, with the following rationale [1]:

Less than 5MHz for NTN

In Rel-18, the feature to support less than 5MHz for NR is defined for TN to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1” by extending support for NTN.

This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. This feature should be built on the work done in the Rel-18, i.e., WI of NR support for dedicated spectrum less than 5 MHz for FR, as much as possible by extending support for NTN.

Furthermore, this is a snippet of the NTN operators’ contribution at RAN4#103 [2], capturing the rationale and key target objectives for this work:

A few important things have to be noted, further from the above:

- Initial NR NTN deployments will have to coexist with existing non-3GPP satellite services for a certain period of time.

- In certain geographical areas, there might be other constraints which limit the amount of contiguous spectrum initially available.

- In some cases, and specific geographic coverage areas, whilst the spectrum may be available, it may be interrupted by legacy services or other allocations that cannot be immediately refarmed, and thus may not be sufficiently large to support a contiguous 5 MHz channel, or even schedule the main 5G NR DL control signals, such as SSB and CORESET#0.

- In such scenarios, channel BW of 5 MHz with full 20 PRB SSB might not be possible, and instead a smaller allocation may be required, in order to deploy an NR NTN cell, also accounting for possible Doppler shift

- 12 PRB SSB/CORESET#0 would provide a significant improvement by reducing the minimum spectrum occupancy requirements enough to be able to deploy a 5G NR NTN cell.

  - Additional spectrum chunks could then be handled by allocating them together with the 12 PRB SSB/CORESET in a non-contiguous manner

- Satellites are indeed power limited, largely EIRP-limited, however, a smaller transmission BW can improve the power spectral density (PSD), for a given EIRP

Observation 1: Reduction of the minimum BW would help deploying NR NTN services in specific geographic areas where there are existing legacy services or other services interrupting the spectrum allocation, such that 5 MHz of contiguous BW might not be immediately available

It is understood that the PBCH puncturing can lead to a link budget loss for PBCH.  However, the reduction in BW allows to achieve a higher PSD for the same EIRP, or alternatively increase the EIRP, which in turn would allow to compensate for the link budget loss.

In addition, for the scenarios that are spectrum limited, the satellite DL EIRP is not the limiting factor, allowing thus to deploy sufficient power boosting to fully recover (and exceed) the link budget losses from puncturing.

Observation 2: For a given satellite DL EIRP, the reduced BW and resulting increase in the Power Spectral Density (PSD) would be sufficient to compensate for the DL link budget loss due to PBCH puncturing.

Observation 3: The NR NTN scenario is different compared to the specific rail scenario supported by bands such as n100, therefore the same constraints do not apply.

Figure 1 - Targeted configurations for less than 5 MHz with 12 RB SSB within larger channel BW and 3 MHz channel BW

For the reasons discussed above, we want to reiterate that the support of  12 PRB configurations is the primary target of this WI objective, and should be supported, both for 3 MHz channel BW, as well as within a larger channel BW configuration.

Proposal 1: Clarify that 12 PRB configuration of SSB/CORESET and data is supported with the default 3 MHz channel BW and GSCN

Proposal 2: Clarify that 12 PRB configuration of SSB/CORESET and data can be used at least with the standard larger-than 3 MHz GSCN within larger nominal channel BW (i.e. 5 MHz and above)

Lastly, we can also note that a reduction of the minimum BW required can facilitate with easier frequency resource management and spectrum reuse, by allowing a given frequency carrier to be partitioned in smaller chunks, also including the common control channels (e.g. FRF3 with 3 MHz CHBW only requires 9 MHz, vs 15 MHz with 5 MHz CHBW). Even further, if the 12 PRB carriers are allowed to partially overlap and deploy in a contiguous fashion, it is possible to achieve higher FRF, and thus higher C/I, within smaller spectrum allocations, greatly improving power and spectrum usagae efficiency.

Observation 4: Contiguous deployment of multiple 12 PRB carriers can allow support of FRF=4 in adjacent beams with a significantly smaller spectrum allocation.

Figure 2 - Possible advantage of 4x 12 PRB contiguous configurations for NR NTN, allowing FRF=4 in less than 10 MHz of spectrum

Proposal 3: Consider if the case where multiple contiguous 12 PRB cells can be supported by suitable GSCN offsets.

# Conclusions

Observation 1: Reduction of the minimum BW would help deploying NR NTN services in specific geographic areas where there are existing legacy services or other services interrupting the spectrum allocation, such that 5 MHz of contiguous BW might not be immediately available

Observation 2: For a given satellite DL EIRP, the reduced BW and resulting increase in the Power Spectral Density (PSD) would be sufficient to compensate for the DL link budget loss due to PBCH puncturing.

Observation 3: The NR NTN scenario is different compared to the specific rail scenario supported by bands such as n100, therefore the same constraints do not apply.

Proposal 1: Clarify that 12 PRB configuration of SSB/CORESET and data is supported with the default 3 MHz channel BW and GSCN

Proposal 2: Clarify that 12 PRB configuration of SSB/CORESET and data can be used at least with the standard larger-than 3 MHz GSCN within larger nominal channel BW (i.e. 5 MHz and above)

Observation 4: Contiguous deployment of multiple 12 PRB carriers can allow support of FRF=4 in adjacent beams with a significantly smaller spectrum allocation.

Proposal 3: Consider if the case where multiple contiguous 12 PRB cells can be supported by suitable GSCN offsets.

# References

- RP-241985 - Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN    Samsung

- RP-240506 - MSS Satellite operator RAN4-led non-spectrum priorities for NTN Evolution in Release 19     Inmarsat, Viasat, Omnispace, Hughes/EchoStar, Terrestar Solutions, Ligado Networks