3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416492

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Further discussion on beam management core requirements

Agenda item:		6.17.4.1

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2].

Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2], as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In RAN4#110, there were discussions on core requirements for AI beam management including, measurement accuracy for beam measurement reporting, metrics/KPIs and other testability issues for AI-BM requirements and tests, while there is no agreement achieved yet [4]. In RAN4#110bis, for the metrics/KPIs for Beam Management requirements/tests, the following agreement is achieved:

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

In RAN4#111, for reported measurements and ground truth, the following agreements are achieved:

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

And the following agreement is achieved in RAN4#112:

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms

In this contribution, we would like to discuss on core requirement for AI-based beam management use case.

# 2. General handling for AI-BM core requirement

After several meeting’s discussion, some progresses has been achieved on AI-beam management, but most of the progresses are related to the ground truth of predicted RSRP and the testing environment for the beam prediction performance. Unfortunately, there is no big progress for specifying AI-BM core requirement, which is required to be completed by Q3 2025.

Considering there are some progresses for AI-BM case 1 and case 2 in RAN1/2, we see the necessity of starting core requirement discussion in RAN4, at least to confirm the scope of impacted RAN4 requirement. Furthermore, the core requirement is also the basis for testability discussion, in which the test setup discussion strongly depends on the new RAN4 core requirement and how the corresponding core requirement shall be tested. As proposed in previous meeting, For AI-BM core requirement, RAN4 shall firstly agree on the expected RAN4 requirement impacts from the perspectives of (1) NW and UE-sided model; (2) different sub-features introduced in RAN1/2, such as: Data collection, Inference, Perf. monitoring, LCM and additional assistance information (for UE-sided model only).

Proposal 1: For AI-BM core requirement, RAN4 shall firstly agree on the expected RAN4 requirement impacts from the perspectives of (1) NW and UE-sided model; (2) different sub-features introduced in RAN1/2, such as: Data collection, Inference, Perf. monitoring, LCM and additional assistance information (for UE-sided model only).

Proposal 2: RAN4 use the below table as the template for collecting views from companies, on RAN4 core requirement impact for AI-BM use case.

|  | New Sub-Feature introduced in RAN1/2 | Whether RAN4 requirement impact is expected? | Clause in TS 38.133 impacted, if any |
| --- | --- | --- | --- |
| NW-sided model | Data collection (for inference and/or training) | e.g., Yes or No | e.g., new clause or clause x.x or no impact |
|  | Inference |  |  |
|  | Perf. monitoring |  |  |
|  | LCM |  |  |
| UE- sided model | Data collection for training |  |  |
|  | Inference (including additional assistance information) |  |  |
|  | Perf. monitoring |  |  |
|  | LCM |  |  |

After providing the analysis, an example of the above table is provided with our views on the expected RAN4 core requirement impact for Rel-19 AI-BM use case.

# 3. RAN4 Core Requirement Impact for NW-Sided Model

For NW-sided model for AI-BM, we provide our analysis on RAN4 core requirement impact for NW-sided model, for the procedure of data collection, inference, performance monitoring and LCM.

## 3.1 Data collection for NW-sided model (for training and inference)

In RAN1#116bis, RAN1 agree to using existing CSI framework for configuration of Set A and B as starting point. From L1-RSRP reporting content perspective, as provided in TR 38.843 [1], regarding data collection for NW-side AI/ML model, options for the contents of collected data are provided by considering the different reported contents and larger number of beams. Particularly for model inference, for data collection for NW-sided model inference, the enhancement on UE beam reporting (i.e., the report of more than 4 beam related information) in L1 signaling could be introduced for Rel-19 AI-BM.

On the other hand, for data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the potential approaches for overhead reduction are provided in TR 38.843 [1], while the following agreement in RAN1#117 could be considered as the way for overhead reduction: since the beam indexes are given accordingly to the configured measurement report, only the differential RSRP are needed.

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

Observation 1: For NW-sided model, for inference report, enhancement on beam reporting in L1 signaling is agreed to be introduce to accommodate Top-M (where M can be larger than 4) beams with corresponding beam information.

From signaling perspective, both L1 signaling and higher-layer signaling are considered to report the collected data are considered in TR 38.843.

Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following reporting signalling for beam-specific aspects maybe applicable: - L1 signalling to report the collected data - Higher-layer signalling to report the collected data - At least not applicable to AI/ML model inference - Note1: higher layer signalling design is up to RAN2 - Note2: Whether each signalling applicable to each LCM purpose is a separate discussion - Note3: The legacy signalling principle (e.g. RSRP reporting for L1) can be re-used

And from RAN2#125bis, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW-side AI/ML model inference, and RRC-based measurement and reporting for NW-sided training:

Agreements from RAN2#125bis:

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

And for NW-sided data collection for training, RAN2 has achieved the following agreements:

RAN2#127 discussed NW-side data collection and made the following agreements: - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases. - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network. - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported. - FFS on event based data collection/logging. - On-demand request from the network is supported. FFS details on signalling. - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements) - Data collection report will not be transmitted over SRB1. FFS which SRB is used.

We can observe that for data collection for NW-side AI/ML model (for BM-Case1 & 2), L1 measurement framework will be used for model inference, and MDT-based measurement/reporting will be used for training.

Based on the above analysis, RAN4 shall introduce the necessary core requirement on supporting the data collection for NW-sided model.

Proposal 3: RAN4 shall introduce the necessary L1-RSRP reporting requirement on supporting data collection for NW-side AI/ML model inference/training (for both BM-Case1 & 2), by considering:

- L1 measurement/reporting for inference: e.g., Top-M (where M can be larger than 4) beams with corresponding beam information;

Proposal 4: No RAN4 impact is expected for NW-side data collection.

Another issue related is whether or not measurement accuracy needs to be improved to enable better AI/ML performance. In RAN4#110, the following options were given:

Issue 2-2: Measurement accuracy - Proposals - Option 1: RAN4 should consider tighter accuracy requirements for existing measurements (e.g. RSRP) if they are used/reported for AI/ML (e.g. training, inference, etc) - Option 2: RAN4 should study the impact of measurement accuracy on performance before discussing any possible tightening - Option 3: Accuracy requirements cannot be tightened - Option 4: Others

From system design perspective for L1 beam reporting accuracy for NW-sided model, the tradeoff between higher achievable system performance (i.e., the necessity of introducing higher measurement accuracy requirement) and implementation feasibility shall be balanced, while RAN1 may focus on the former aspect and the latter part shall be within RAN4 scope/expertise.

In existing RAN4 specification, FR2 RSRP measurement absolute/relative accuracy requirements differ under normal/extreme conditions, and under different input level conditions: e.g., 6dB is assumed for SS-RSRP intra-frequency accuracy requirement for normal condition without Noc. Additionally, the absolute accuracy test requirement is further relaxed by adding Gmin and Gmax (i.e,. the minimum and maximum UE gain values from Table B.2.1.5.1-1 in TS38.133), which is Gmin = -10dB and Gmax = +20dB for FR2 PC3 UE.

On the other hand, from the system performance perspective (from the impact of different beam measurement accuracy), it is agreed to further studied by considering the following aspects in RAN4#112:

Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

During Rel-19 RAN1 study, the L1-RSRP measurement error has been introduced in addition to the baseline evaluation (without any measurement error), and the corresponding results are captured in section “Performance with measurement error” in clause 6.3.2.3. It should be noted that measurement error can be modelled as a combined one for BB and RF errors, or differentiated BB and RF errors separately. Most of the evaluation shows the degradation introduced by measurement errors, while the impacts can’t be quantitively identified because different AI/ML model may be sensitive to the level of measurement errors differently. As a general take-out, the higher level of measurement error, the lower prediction accuracy is expected for both DL TX beam predication and TX-RX beam pair prediction, but we can’t see a clear alignment among companies’ results.

Observation 2: RAN1 has studied the impact of measurement error on prediction accuracy (1) by modeling BB and RF factors as combined or differentiated errors; (2) by studying the different level of relative measurement error; and (3) by studying the impact for DL TX beam predication and TX-RX beam pair prediction.

Observation 3: Lower prediction accuracy is expected for both DL TX beam predication and TX-RX beam pair prediction, but no clear alignment is achieved among companies’ results from RAN1 evaluation.

Considering there is no clear evidence which shows that the existing relative L1-RSRP measurement accuracy required by Rel-15 RAN4 requirement will impairment the AI/ML model from working well, we see no reason to further enhance the accuracy requirement, unless the feasibility of tightened measurement accuracy is demonstrated.

Proposal 5: For measurement accuracy requirement for AI-BM NW-sided model:

- Unless the feasibility of tightened measurement accuracy is demonstrated, RAN4 shall reuse the accuracy requirement from Rel-15 L1-RSRP for both absolute and relative RSRP reporting.

## 3.2 Inference for NW-sided model

For the model inference for BM-Case1 and BM-Case2 with a NW-side AI/ML model, the following potential specification impacts from physical layer aspects are given in TR 38.843 [1]:

For BM-Case1 and BM-Case2 with a NW-side AI/ML model: - L1 beam reporting enhancement for AI/ML model inference: - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered For BM-Case 2: - Reporting information about measurements of multiple past time instances in one reporting instance. Notes: Only applicable to NW-side AI/ML model. The potential performance gains of measurement reporting should be justified by considering UCI payload overhead.

Accordingly, we see no RAN4 impact is expected for model inference (except the relevant data collection as provided in Section 2.1.2), because the model inference is implemented in gNB, in which the AI/ML inference results are also used by gNB to conduct beam management.

From the below RAN2 agreement, it can be observed that there is no new signaling introduced for gNB-sided model inference (depending on further RAN1 input).

Agreements from RAN2#125bis:

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

It should be noted that the potential enhancement of “measurements of multiple past time instances in one reporting instance” shall also be categorized as data collection for inference, and even if this mechanism is introduced in Rel-19, it should also not introduce any new RAN4 requirement for model inference.

Proposal 6: No RAN4 impact is expected for NW-side AI/ML model inference for BM-Case1 & 2, except data collection for model inference.

## 3.3 Performance monitoring for NW-sided model

Similar to model inference with a NW-side AI/ML model, the performance monitoring shall also be gNB implementation-based, i.e., the monitoring can be configured by the NW based on the adopted performance metric. Till now, there is no clear conclusion that there will be RAN2 specification impact for gNB-side model monitoring, i.e., the following agreement from RAN2#126:

Agreements for NW-sided model for Beam Management in RAN2#126:

1	For the network-side model, required network side additional condition is left up to the network implementation

2	RAN2 will wait for RAN1 for any required UE side additional conditions.

3	For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

Proposal 7: FFS RAN4 impact from NW-side AI/ML model performance monitoring for BM-Case1 & 2 (if RAN1/2 introduce any mechanism for UE side additional conditions)

## 3.4 LCM for NW-sided model

In WID [2], it is expected that RAN2 will introduce the necessary LCM related aspects, enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback. However, as given in RAN2#125bis, it has confirmed the roles of UE in the NW-sided LCM procedures: i.e., For NW-sided model for AI-BM, UE will neither be informed about any LCM decision, nor be involved in any LCM decision making, except being configured for providing the required measurement/data.

Agreements from RAN2#125bis

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

Considering RAN4 requirement shall focus on UE-relevant signaling procedure, there will be no RAN4 impact expected for LCM for NW-sided model.

Proposal 8: No RAN4 impact is expected for NW-side AI/ML model LCM for BM-Case1 & 2.

## 3.5 Summary of requirement impact for NW-sided model

Here based on the table in Proposal 2, we would like to provide the summary on the expected RAN4 requirement impact for Rel-19 AI-BM NW-sided model.

Proposal 9: Based on corresponding proposals, the expected RAN4 requirement impacts from AI-BM NW-sided model are summarized as:

|  | New Sub-Feature introduced in RAN1/2 | Whether RAN4 requirement impact is expected? | Clause in TS 38.133 impacted, if any | Related Proposal(s) |
| --- | --- | --- | --- | --- |
| NW-sided model | Data collection (for inference and/or training) | Yes: for L1-RSRP reporting No: for NW-side data collection | • Clause 9.5 L1-RSRP measurements for reporting (or similar new requirement for L1-RSRP measurement for reporting for AI-BM) | Proposal 3,4,5 |
|  | Inference | No |  | Proposal 6 |
|  | Perf. monitoring | FFS RAN4 impact |  | Proposal 7 |
|  | LCM | No |  | Proposal 8 |

# 4. RAN4 Core Requirement for Supporting UE-Sided Model

For UE-sided model for AI-BM, we provide our analysis on RAN4 core requirement impact for UE-sided model, for the procedure of data collection, additional assistance information, inference, performance monitoring and LCM.

## 4.1 Data collection for UE-sided model

For data collection for UE-sided model, the following solutions are provided [1]. (Note: OTT server is also called as “server for UE-side data collection” based on RAN2 discussion.)

| 1a) | 1b) |
| --- | --- |
| 2. Data collection via CN | 3. Data collection via OAM |

Based on the above contents from TR 38.843, the potential interaction(s) between UE and NW for data collection for UE-sided model could be: (1) UE reporting of the supported/preferred DL RS configuration; (2) trigger/initiating data collection; (3) assistance information from NW to UE for data collection.

According to the below RAN4 agreement, the requirements for data collection for training could only be introduced if the training procedure is defined in 3GPP specifications. RAN2 has evaluated the different options of data collection for model training to understand the potential specification impact as follows [R2-2407807].

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

It has been shown Option 1 (including 1a and 1b) and Option 2 involve application layer and NAS layer respectively, and their corresponding RAN4 impact can be easily precluded. For Option 3, the data transfer will be firstly terminated in OAM, and the corresponding mechanism is also not appropriately tested by RAN4, based on normal RAN4 practice. Therefore, we would like to confirm no RAN4 impact is expected for data collection for UE-sided model training.

Proposal 10: No RAN4 impact is expected from data collection for UE-sided model training.

Regarding the data collection mechanisms for inference and performance monitoring, we treat them as the part of procedures for model inference and performance monitoring, which will be analyzed in the following sections.

## 4.2 Inference for UE-sided model

For UE-sided model for AI-BM, UE shall report the prediction result to NW based on the output of a UE-side AI/ML model. In RAN1#116-bis and RAN1#117, the following agreement is achieved for content in the report of inference results, for UE-sided AI/ML model inference for BM-Case1 and Case2. In RAN1#118, the quantization of a RSRP value support the differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting.

Agreement from RAN1#116-bis For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement from RAN1#116-bis For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement from RAN1#117 For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output Agreement from RAN1#118 For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details

On the other hand, RAN1 also discussed how the CSI-ReportConfig is used for the configuration of inference results reporting, with the following latest agreement achieved in RAN1#118:

Agreement from RAN1#116-bis For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

Accordingly, RAN4 requirement for model inference with UE-side AI/ML model needs to be specified for both AI-BM Case1 and Case 2, in which the above RAN1 conclusion and future progress shall be considered.

Proposal 11: RAN4 requirement of UE-side AI/ML model inference shall be specified.

Proposal 12: RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) the accuracy of inference results accordingly to the agreed KPI; (2) the delay of reporting inference results (containing the inference latency).

##### 4.2.1 Beam prediction KPI

For the part of beam prediction KPI, we also present our analysis in our accompanying paper for testability:

In previous RAN4 discussion, For the metrics/KPI discussion, the following options are given in the Rel-18 study item and captured in TR [1]:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Here, RAN4 requirement shall be set up by considering the expected behavior from the good UE implementation. For beam prediction accuracy, it is straightforward that NW will directly follow the UE’s prediction results by TCI switching to the Top-1 predicted beam, which make the 2nd and other strongest beam predicted not that related. By combining the merits from Option 2 and Option 3, we propose the compromised option by merging Option 2 (Top-1/K) and 3:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

  - FFS: K = 2 and x = 3 as baseline

##### 4.2.2 Input to AI/ML network

Another aspect to be discussed is the input to AI/ML. From Rel-18 RAN1 study, model inputs are described as “measurements based on a) Set B of beams (BM-Case 1) and b) Set B of beams at historic time instance(s) (BM-Case-2)”, while “beam ID information may be also provided as input to the AI/ML model”. Specifically, the detailed input to AI/ML model, and the options for Rx beam as AI/ML model input are captured in TR 38.843:

For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), study the following options as AI/ML model inputs: - Alt 1: Implicit information of Tx beam ID and/or Rx beam ID - e.g., measurements of Set B of beams together with default values (e.g., 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector. Detailed assumption can be reported. - Alt 2: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly. For the purpose of DL Tx beam prediction evaluations, consider the following options for Rx beam as AI/ML model input for training and/or inference if applicable: - Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample. - Companies expected to report how to select the "best" Rx beam(s). - Option 2: Measurements of specific Rx beam(s). - Companies expected to report how to select specific Rx beam(s). - Option 3: Measurements of random Rx beam(s) per model input sample. - Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. Companies expected to report the measurement/RS overhead together with the beam prediction accuracy, as well as, how to find the quasi-optimal Rx beam with "previous measurement". - Other options are not precluded and can be reported by companies.

No matter using the information of Tx beam ID and/or Rx beam ID explicitly or implicitly, there are two kinds of beam prediction considered in RAN1, i.e., DL Tx beam prediction with the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair. Till now, there is no conclusion to exclude or deprioritize any of these two beam prediction methods.

Observation 2: For UE-sided model, DL Tx beam prediction are considered with the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair, as the input to AI/ML network.

##### 4.2.2 Additional assistance information for UE sided model

Different from NW-sided model, AI/ML operation at UE side could require the additional assistance information, since UE-sided model have no information of the NW-side beambook configuration which is essential to AI/ML operation. Based on RAN1#116bis discussion, RAN1 is still discussing possible options for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2:

Agreement from RAN1#116bis Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

In RAN1#118, associated ID is agreed as working assumption to be configured within CSI framework:

Agreement from RAN1#118 For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

And the following agreement from RAN2:

Agreements from RAN2#126

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

Accordingly, from RAN4 perspective on testing requirement, we see the consistency/association of Set B beams and Set A beams across training and inference shall be guaranteed in RAN4 core requirement as the NW-side additional assistance information for inference at UE.

Proposal 13: For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed by applying the same associated ID as side condition.

## 4.4 Performance monitoring for UE-sided model

Regarding the performance monitoring for UE-sided model, key issues discussed in RAN1 include: performance metrics, benchmark/reference for the performance comparison, and monitoring type. Corresponding analysis for potential specification impact for performance metrics and benchmark/reference for the performance comparison is provided below from physical layer perspective:

For the performance monitoring of BM-Case1 and BM-Case2: - Performance metric(s) with the following alternatives: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Benchmark/reference for the performance comparison, including: - Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A) - Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams) - Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

In the normative phase, based on RAN1’s down-selection on performance metric(s) and benchmark/reference for the performance comparison, RAN4 requirement on performance monitoring can be specified accordingly. Specifically, RAN1 has provided two types of performance monitoring, i.e.,

For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable

Particularly, Type 1 performance monitoring is performed in the NW-side, for which Option 1 and Option 2 differ in which entity shall perform the calculation of the performance metric(s). On the other hand, Type 2 performance monitoring is performed in the UE-side, while no feedback is required.

In RAN1#117 and #118, the following agreement is achieved, particular for Type-1 (both Option 1 and 2) performance monitoring:

Agreement from RAN1#117 For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement from RAN1#118 For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Based on the above understanding on Type 1 and Type 2 performance monitoring, the following proposal is obtained.

Proposal 14: For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as:

- Type 1, Option 1 (NW-side performance monitoring): No RAN4 requirement is required on data collection for monitoring, because it is similar to data collection for other purposes.

- Type 1, Option 2 (UE-assisted performance monitoring): FFS until RAN1 agreement on different alternatives. At least for some of alternatives, RAN4 prediction accuracy requirement can be used to guarantee this kind of performance monitoring.

- Type 2 (UE-side performance monitoring): No RAN4 requirement is required because no UE feedback will be performed and the performance monitoring can be guarantee by model generalization test implicitly.

## 4.5 LCM for UE-sided model

Based on the above analysis for different types of performance monitoring for UE-sided model (NW-side or UE-side performance monitoring), the model/functionality control can be performed by the UE or gNB:

- Management: - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. - For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the gNB when the monitoring resides within the gNB or UE.

Based on the agreement obtained in RAN2#125bis, the “network decision, network-initiated” AI/ML management is supported as a baseline:

Agreements from RAN2#125bis:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In RAN2#126, the following agreements are achieved on LCM for UE-sided model:

Agreements from RAN2#126

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

Proposal 15: For UE-sided model LCM, the necessity of RAN4 requirement:

- “Network decision, network-initiated” LCM: RAN4 requirement is needed.

- FFS the detailed RAN4 requirement (depending on RAN1/2 input).

## 4.5 Summary of requirement impact for UE-sided model

Based on the table in Proposal 2, we would like to provide the summary on the expected RAN4 requirement impact for Rel-19 AI-BM UE-sided model.

Proposal 16: Based on corresponding proposals, the expected RAN4 requirement impacts from AI-BM UE-sided model are summarized as:

|  | New Sub-Feature introduced in RAN1/2 | Whether RAN4 requirement impact is expected? | Clause in TS 38.133 impacted, if any | Related Proposal(s) |
| --- | --- | --- | --- | --- |
| UE-sided model | Data collection for training | No |  | Proposal 10 |
|  | Inference (including additional assistance information) | Yes | New clause for UE-side AI/ML model inference (to guarantee (1) prediction accuracy; (2) the delay of reporting inference results) | Proposal 11-13 |
|  | Perf. monitoring | No: For Type 1 (Option 1) and Type 2 FFS: Type 1 (Option 2) |  | Proposal 14 |
|  | LCM | Yes | FFS detailed RAN4 requirement | Proposal 15 |

# 5. Conclusion

In this contribution, we provided our viewpoints on testability and interoperability issues for beam management in this contribution, accordingly the following observations and proposals are obtained:

General handling for AI-BM core requirement

Proposal 1: For AI-BM core requirement, RAN4 shall firstly agree on the expected RAN4 requirement impacts from the perspectives of (1) NW and UE-sided model; (2) different sub-features introduced in RAN1/2, such as: Data collection, Inference, Perf. monitoring, LCM and additional assistance information (for UE-sided model only).

Proposal 2: RAN4 use the below table as the template for collecting views from companies, on RAN4 core requirement impact for AI-BM use case.

|  | New Sub-Feature introduced in RAN1/2 | Whether RAN4 requirement impact is expected? | Clause in TS 38.133 impacted, if any |
| --- | --- | --- | --- |
| NW-sided model | Data collection (for inference and/or training) | e.g., Yes or No | e.g., new clause or clause x.x or no impact |
|  | Inference |  |  |
|  | Perf. monitoring |  |  |
|  | LCM |  |  |
| UE- sided model | Data collection for training |  |  |
|  | Inference (including additional assistance information) |  |  |
|  | Perf. monitoring |  |  |
|  | LCM |  |  |

RAN4 Core Requirements for Supporting NW-sided Model

Observation 1: For NW-sided model, for inference report, enhancement on beam reporting in L1 signaling is agreed to be introduce to accommodate Top-M (where M can be larger than 4) beams with corresponding beam information.

Proposal 3: RAN4 shall introduce the necessary L1-RSRP reporting requirement on supporting data collection for NW-side AI/ML model inference/training (for both BM-Case1 & 2), by considering:

- L1 measurement/reporting for inference: e.g., Top-M (where M can be larger than 4) beams with corresponding beam information;

Proposal 4: No RAN4 impact is expected for NW-side data collection.

Observation 2: RAN1 has studied the impact of measurement error on prediction accuracy (1) by modeling BB and RF factors as combined or differentiated errors; (2) by studying the different level of relative measurement error; and (3) by studying the impact for DL TX beam predication and TX-RX beam pair prediction.

Observation 3: Lower prediction accuracy is expected for both DL TX beam predication and TX-RX beam pair prediction, but no clear alignment is achieved among companies’ results from RAN1 evaluation.

Proposal 5: For measurement accuracy requirement for AI-BM NW-sided model:

- Unless the feasibility of tightened measurement accuracy is demonstrated, RAN4 shall reuse the accuracy requirement from Rel-15 L1-RSRP for both absolute and relative RSRP reporting.

Proposal 6: No RAN4 impact is expected for NW-side AI/ML model inference for BM-Case1 & 2, except data collection for model inference.

Proposal 7: FFS RAN4 impact from NW-side AI/ML model performance monitoring for BM-Case1 & 2 (if RAN1/2 introduce any mechanism for UE side additional conditions)

Proposal 8: No RAN4 impact is expected for NW-side AI/ML model LCM for BM-Case1 & 2.

Proposal 9: Based on corresponding proposals, the expected RAN4 requirement impacts from AI-BM NW-sided model are summarized as:

|  | New Sub-Feature introduced in RAN1/2 | Whether RAN4 requirement impact is expected? | Clause in TS 38.133 impacted, if any | Related Proposal(s) |
| --- | --- | --- | --- | --- |
| NW-sided model | Data collection (for inference and/or training) | Yes: for L1-RSRP reporting No: for NW-side data collection | • Clause 9.5 L1-RSRP measurements for reporting (or similar new requirement for L1-RSRP measurement for reporting for AI-BM) | Proposal 3,4,5 |
|  | Inference | No |  | Proposal 6 |
|  | Perf. monitoring | FFS RAN4 impact |  | Proposal 7 |
|  | LCM | No |  | Proposal 8 |

RAN4 Core Requirements for Supporting UE-sided Model

Proposal 10: No RAN4 impact is expected from data collection for UE-sided model training.

Proposal 11: RAN4 requirement of UE-side AI/ML model inference shall be specified.

Proposal 12: RAN4 shall specify RRM requirement for UE-side AI/ML model inference: to guarantee (1) accuracy of inference results accordingly to the agreed KPI; (2) the delay of reporting inference results (containing the inference latency).

Observation 2: For UE-sided model, DL Tx beam prediction are considered with the measurements from (1) the best Rx beam or (2) Tx-Rx beam pair, as the input to AI/ML network.

Proposal 13: For the testing of UE-sided model inference and monitoring (if any), the consistency of NW-side additional condition across training and inference shall be guaranteed by applying the same associated ID as side condition.

Proposal 14: For different types of performance monitoring for UE-sided model, the necessity of RAN4 requirement is provided as:

- Type 1, Option 1 (NW-side performance monitoring): No RAN4 requirement is required on data collection for monitoring, because it is similar to data collection for other purposes.

- Type 1, Option 2 (UE-assisted performance monitoring): FFS until RAN1 agreement on different alternatives. At least for some of alternatives, RAN4 prediction accuracy requirement can be used to guarantee this kind of performance monitoring.

- Type 2 (UE-side performance monitoring): No RAN4 requirement is required because no UE feedback will be performed and the performance monitoring can be guarantee by model generalization test implicitly.

Proposal 15: For UE-sided model LCM, the necessity of RAN4 requirement:

- “Network decision, network-initiated” LCM: RAN4 requirement is needed.

- FFS the detailed RAN4 requirement (depending on RAN1/2 input).

Proposal 16: Based on corresponding proposals, the expected RAN4 requirement impacts from AI-BM UE-sided model are summarized as:

|  | New Sub-Feature introduced in RAN1/2 | Whether RAN4 requirement impact is expected? | Clause in TS 38.133 impacted, if any | Related Proposal(s) |
| --- | --- | --- | --- | --- |
| UE-sided model | Data collection for training | No |  | Proposal 10 |
|  | Inference (including additional assistance information) | Yes | New clause for UE-side AI/ML model inference (to guarantee (1) prediction accuracy; (2) the delay of reporting inference results) | Proposal 11-13 |
|  | Perf. monitoring | No: For Type 1 (Option 1) and Type 2 FFS: Type 1 (Option 2) |  | Proposal 14 |
|  | LCM | Yes | FFS detailed RAN4 requirement | Proposal 15 |

# 6. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.