3GPP TSG-RAN WG4 Meeting #112bis                                                          R4-2416497

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Discussion on RAN4 aspects for SBFD system parameters

Agenda item:			6.20.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our further analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters.

# 2. RAN4 aspects for SBFD system parameters

### 2.1 Band/channel bandwidth support for SBFD

Based on the discussion in RAN4#111, SBFD was recognized as “a feature with requirements of which can potentially be applied to all TDD band”, and the declaration-based method for the SBFD-support in a particular TDD band was agreed in RAN4#112. Particularly, the following agreements was provided [7]:

<From RAN4#112> - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

One of the discussion points in RAN4#112 is the minimum channel bandwidth value to support SBFD and X = [20] MHz is considered as baseline for further study in next RAN4 meeting. The logic behind setting the minimum channel bandwidth value as [20] MHz is to preclude the FR1 channel bandwidth values of 3, 5, 10, and 15 MHz, which could be too small to accommodate the SBFD guardband and are not able to demonstrate the benefits of SBFD due to the relatively high overhead.

Our understanding is that [20] MHz is already a reasonable compromise since a larger X value (e.g., 100MHz as proposed by some companies) will preclude SBFD operation in the bands in which 3GPP can’t support such large channel bandwidth. Furthermore, another important observation is that even setting X = 100, only two more bands will be precluded from SBFD operation, i.e., band n38 and n39, while these bands are such important for Region 3 which should better be kept to allow SBFD deployment if needed.

Proposal 1: For gNB support of SBFD for a certain TDD band, it is based on gNB vendor’s declaration. For the SBFD-supported band, the channel bandwidth(s) supported for SBFD shall follow the below principles:

- The supported channel bandwidth(s) is declared by gNB vendors

- The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

- The supported channel bandwidth shall be no less than X=20 MHz

By listing the supported channel bandwidth for all FR1 TDD bands (given in Table 5.3.5-1 in TS 38.104), we could find that the above principle for minimum channel bandwidth, i.e.,  “The supported channel bandwidth shall be no less than X=20 MHz”, will preclude TDD bands n34, n51, n53, n54 and n101 from SBFD deployment.

Proposal 2: By following the principle of “The supported channel bandwidth shall be no less than X=20 MHz”, TDD bands n34, n51, n53, n54 and n101 shall be precluded from SBFD deployment.

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 30 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
| n38 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
| n39 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |  |  |  |  |  |  |  |  |
|  | 15 |  | 54 | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n40 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 58 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
| n41 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  | 106 |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n46 | 30 |  |  | 106 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 60 |  |  | 106 |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 15 |  | 52 | 10 | 15 | 20 |  | 30 |  | 40 |  | 501 |  |  |  |  |  |  |
| n48 | 30 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 501 | 601 | 701 | 801 | 901 | 1001 |  |
|  | 60 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 501 | 601 | 701 | 801 | 901 | 1001 |  |
|  | 15 |  | 52 | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n50 | 30 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 | 60 |  | 80 |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 |  | 30 |  | 40 |  | 50 | 60 |  | 80 |  |  |  |
|  | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n51 | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n54 | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n77 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n78 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n79 | 30 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |  |
| n90 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n96 | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
| n101 | 15 |  | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  | 20 |  |  |  | 40 |  |  |  |  |  |  |  |  |
| n102 | 30 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 60 |  |  |  |  | 20 |  |  |  | 40 |  |  | 60 |  | 80 |  | 100 |  |
|  | 15 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
| n104 | 30 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | 60 |  |  |  |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | NOTE 1: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. NOTE 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 3: For this bandwidth, it only applies for UL transmission. NOTE 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 5: Void. NOTE 6: This bandwidth can only be applied in certain regions where the absence of non 3GPP technologies can be guaranteed on a long term basis in this version of specification. NOTE 7: For this bandwidth, it only applies for DL transmission. NOTE 8: Not all frequency positions of 5 MHz carriers are possible due limitations of the SSB position relative to the 5 MHz channels. 5 MHz channels with Fc such that 2499+N*1.2 ≤Fc<2499.3+N*1.2MHz for 0≤N<157 are not compatible with SSB positions and cannot be used for 5 MHz n41. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Next issue will be how to implement the above-mentioned principle in the specifications. Accordingly, the following two options are provided in the approved way forward [7]:

- Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

- Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

Because of that SBFD is “a feature with requirements of which can potentially be applied to all TDD band”, and we see no needs to add another entry for SBFD, hence the above Option 1 is preferred.

Proposal 3: RAN4 shall introduce the following text in Clause 5.2:

- “Subband full duplex can be applied to all FR1 TDD bands given in Table 5.2-1, except bands n34, n51, n53, n54 and n101, and all FR2 bands given in table 5.2-2.”

For channel bandwidth, except channel bandwidth values < 20MHz, there is no necessity to preclude other supported channel bandwidth values for each band from SBFD deployment. Accordingly the following Note can be added in Table 5.3.5-1.

Proposal 4: Except channel bandwidth values smaller than 20MHz, there is no necessity to preclude other supported channel bandwidth values for each band from SBFD deployment given in Table 5.3.5-1.

Proposal 5: RAN4 shall introduce the following note in Table 5.3.5-1:

- “Note 9: Subband full duplex can be applied to all supported channel bandwidth values except the ones smaller than 20MHz.”

### 2.2 Transmission bandwidth configuration for SBFD

In RAN4#112, much progress has been achieved on the definition clarification of BS channel bandwidth and BS transmission bandwidth configuration for SBFD, as follows:

Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. Transmission bandwidth configuration for SBFD Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed

In the existing specification TS 38.104, the figure 5.3.1-1 is used to clarify the definition of channel bandwidth and transmission bandwidth configuration for one NR channel. For SBFD operation, the variable for  transmission bandwidth configuration is changed to NRB,SBFD. Additionally, the illustrations of transmission bandwidth configuration for SBFD DL subband, UL subband and guardband between SBFD UL and DL subband are also provided in the below figure.

Proposal 6: The following figure is introduced for the illustrations of BS channel bandwidth, BS transmission bandwidth configuration for SBFD NRB,SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL, and guardband between SBFD UL/DL subbands NRB,SBFD,GB for SBFD operation:

Figure 1: Definition of channel bandwidth and transmission bandwidth configurations for one NR channel for SBFD operation

### 2.3 DL/UL subband configurations support for SBFD

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified.

In RAN4#112, progress has been achieved in clarifying the definition of transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL. The detailed agreements are provided as follows [7]:

Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#112, the following three options with different levels of configuration flexibilities have been provided [7]:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

Among the three levels of flexibilities, we firstly identify Option-1 as being not only unreasonable but also against RAN1 agreements.

Simply setting a number of selected DL/UL subband configurations is actually not attainable, since it is hard in the standardization stage to determine the size of subbands and guardband(s). If RAN4 is tasked to only pick up few configurations, companies will end up by proposing lots of configurations which could not be actually used to make sure future products are standard-compatible. In reality, the following techniques shall be considered in the system design, from RF and digital domains respectively:

- In the field of RF, to address interference from the DL and ensure accurate reception of the desired UL signal, an analog filter can be employed prior to the ADC. This filter serves to attenuate DL interference and protect the dynamic range of the ADC. An active approach can also involve an analog interference canceller, although in mMIMO systems, the complexity of the analog canceller increases as the number of Tx and Rx chains grows. All these factors could potentially impact the determination of the size of subband and guardband.

- In the digital domain, various techniques can be utilized to reduce the impact on the sensitivity of the BS receiver, aiming to keep it below 1 dB. One such technique is digital interference cancellation (IC), which aims to eliminate interference from the desired UL signal and reduce any remaining self-interference (SI) to an acceptable level. Digital IC involves synthesizing interference from baseband Tx samples with accurate modeling of Tx chain non-linearity or capturing interference from the RF chain on the transmit side and converting it to baseband. This process requires estimating the SI channel taps for each Tx chain to each Rx chain. Additionally, other digital techniques can help mitigate interference. For example, Digital Pre-distortion (DPD) can reduce leakage into the UL subband. Other techniques, such as beam nullification to lessen the impact of clutter reflections or optimizing digital Tx beamforming to decrease SI, can also be employed. All these digital domain techniques could be relevant in determination of the size of subband and guardband.

From RAN1 perspective, it is observed that RAN1 agree to support the flexible frequency domain configuration with the cell-specific configuration, by explicitly configuring frequency locations of UL subband and DL subband(s) thus making the guardband(s) if any can be derived implicitly. If only few configurations are specified for RAN4 RF requirements, we see no reason for RAN1 to define such flexible configuration:

<From RAN1#117> Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

Observation 1: To specify DL/UL subband configuration in BS RF requirements, Option-1 (i.e., only a number of selected DL/UL subband configurations are specified) is impracticable for standardization and against RAN1 agreements.

Based on our understanding, the good tradeoff between configuration flexibility and standardization complexity can be achieved by Option 2, i.e., DL (or UL) subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. Among Option-2a and 2b, we prefer Option-2b because selecting UL subband configuration from the existing values of BS transmission bandwidth configuration will simplify the standardization procedure for receiver requirement.

Proposal 7: To specify DL/UL subband configuration in BS RF requirements, the following Option-2b is adopted:

- Option-2b:

  - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

  - DL subband size(s) and guardband size(s) depends on BS implementation

  - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

### 2.4 Restriction on guardband size

By adopting Option-2b which allows flexible DL subband size(s) and guardband size(s) depending on BS implementation. Accordingly, we suggest no restriction is required for both maximum and minimum number of RBs for guardband.

Proposal 8: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

### 2.5 Text proposal for BS channel bandwidth for SBFD

The following text proposal with the specification structure for SBFD-capable BS could be considered as the baseline for RAN4 discussion.

Proposal 9: A new 5.3B clause is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

## 5.3B	BS channel bandwidth for SBFD

### 5.3B.1	General

<To add the illustrative definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

### 5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

### 5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

# 3. Other general aspects for SBFD operation

### 3.1 BS RF specification structure for SBFD requirements

In previous RAN4 discussion, two options, i.e., creating new sub-clauses and embedding the SBFD RF requirement in corresponding section, were listed on how to incorporate SBFD requirements in the BS RF core specification (i.e., TS 38.104). Particularly, in RAN4#111, the following agreements are achieved:

<From RAN4#111> Issue 1-4-1: BS RF Specification structure for SBFD requirements - Agreement: - How to introduce BS RF new requirements for SBFD-capable BS: - FFS firstly the feasibility of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements - FFS detailed how to implement such approach in the later phase of WI. - FFS secondly the new specification for SBFD-capable BS

By considering the readability of the specification for SBFD-capable BS, we think the way of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 can be followed for SBFD-specific existing or new gNB RF requirements. As mentioned in previous RAN4 discussion by some companies, one comparable precedent is using suffix-B for Mobile IAB-node in the core and conformance specification for IAB nodes (TS 38.174, TS 38.176-1 and TS 38.176-2). One different issue is that RAN4 could introduce the new BS RF requirement dedicated for SBFD symbols, and the below proposal also takes that into account:

Proposal 10: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B.

In other words, for both cases (modifying existing BS RF requirements or specifying new BS RF requirements for SBFD-capable BS), setting up new sub-clauses with suffix-B shall be used to guarantee specification readability:

- For the SBFD BS RF requirements specified by modifying existing requirements:

  - Set up new sub-clause with suffix-B

- For the new BF RF requirements dedicated to SBFD BS

  - Set up new sub-clause with suffix-B

# 4. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, accordingly with the following observations and proposals obtained:

Proposal 1: For gNB support of SBFD for a certain TDD band, it is based on gNB vendor’s declaration. For the SBFD-supported band, the channel bandwidth(s) supported for SBFD shall follow the below principles:

- The supported channel bandwidth(s) is declared by gNB vendors

- The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

- The supported channel bandwidth shall be no less than X=20 MHz

Proposal 2: By following the principle of “The supported channel bandwidth shall be no less than X=20 MHz”, TDD bands n34, n51, n53, n54 and n101 shall be precluded from SBFD deployment.

Proposal 3: RAN4 shall introduce the following text in Clause 5.2:

- “Subband full duplex can be applied to all FR1 TDD bands given in Table 5.2-1, except bands n34, n51, n53, n54 and n101, and all FR2 bands given in table 5.2-2.”

Proposal 4: Except channel bandwidth values smaller than 20MHz, there is no necessity to preclude other supported channel bandwidth values for each band from SBFD deployment given in Table 5.3.5-1.

Proposal 5: RAN4 shall introduce the following note in Table 5.3.5-1:

- “Note 9: Subband full duplex can be applied to all supported channel bandwidth values except the ones smaller than 20MHz.”

Proposal 6: The following figure is introduced for the illustrations of BS channel bandwidth, BS transmission bandwidth configuration for SBFD NRB,SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL, and guardband between SBFD UL/DL subbands NRB,SBFD,GB for SBFD operation:

Figure 1: Definition of channel bandwidth and transmission bandwidth configurations for one NR channel for SBFD operation

Observation 1: To specify DL/UL subband configuration in BS RF requirements, Option-1 (i.e., only a number of selected DL/UL subband configurations are specified) is impracticable for standardization and against RAN1 agreements.

Proposal 7: To specify DL/UL subband configuration in BS RF requirements, the following Option-2b is adopted:

- Option-2b:

  - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

  - DL subband size(s) and guardband size(s) depends on BS implementation

  - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

Proposal 8: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

Proposal 9: A new 5.3B clause is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

## 5.3B	BS channel bandwidth for SBFD

### 5.3B.1	General

<To add the illustrative definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

### 5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

### 5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

Proposal 10: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[6] R4-2411637, “Discussion on SBFD general aspects”, Samsung, RAN4#112.

[7] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.