3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2416519

# Hefei, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Discussion on Power domain enhancements for single carrier

Agenda item:	6.1.1.2.1

Document for:	Discussion

Introduction

In 3GPP RAN #112, WF[1] was approved. In this paper, we provide our views on Power domain enhancements for single carrier.

### Topic #1: Power domain enhancements for single carrier ### Sub-topic 1-1: Approaches to enable MPR reduction for scenario 1 #### Issue 1-1-1: Approaches for scenario 1 - Proposals - Proposal 1: For the scenario when there is no adjacent in-band/out-of-band co-existence issue (single operator), introduce network-controlled relaxation on ACLR and SEM mask. (Huawei) - For ACLR, the relaxation means e.g. PC3 requirement 30dB can be relaxed to a smaller value. - For SEM mask, the relaxation means an upward shift on the whole mask. - No change on the reference channel bandwidth or the start of ΔfOOB - Check if followings can be adopted for the network-controlled relaxation towards no adjacent in-band/out-of-band co-existence- issue (single operator) scenario. - Three level of relaxation, e.g. 3dB, 5dB and waived - Per band and per region (can be based on operator request) - WF - Other ways to reduce MPR are not precluded #### Issue 1-2-1: Clarification of BS CBW - Proposals - Proposal 1: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios. (Huawei) - Moderator observation - For the first meeting discussion, in the WF (R4-2406584) it said “No relaxation of ACLR/SEM/SE outside of the BS CBW for one operator holding spectrum for scenario 2, i.e. Narrower UE channel BW within wider BS bandwidth” and companies had some discussion on operator holding spectrum in the AH. The common understanding is that the operator holding spectrum could be larger than a specific CBW. Some clarification would be helpful for the following discussion. - WF - FFS in next meeting #### Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - Proposals - Approach 1: UE CBW based - Option 1: The extended UE CBW based method as captured in the WF in last meeting. - Approach 2: BS CBW based - Option 2: BS CBW based inner/outer method. (Skyworks) - BS NRB instead of UE NRB. - Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart. - The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR. - For margin or stricter emissions, the BS can signal a reduced NRB and RBshift. - Option 2a: the feature ‘narrower UE channel BW within wider BS bandwidth’ is specified for a UE configured with a UE-specific CHBW within a wider BS bandwidth, the said UE compliant with selected OOBE requirements applicable for the wider BS bandwidth (the cell bandwidth). (Ericsson, Nokia) - WF - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach The following issues are mainly based on extended UE CBW based approach, but in principle the issues relevant to SE/OOBE/IBE are generic to the inner/outer approaches. Common understanding of issues relevant to extended UE CBW does not preclude other approaches to be further evaluated/discussed. #### Issue 1-2-3: whether extended UE CBW edge could exceed the BS CBW edge - Proposals - Proposal 1: The extended UE CBW should not exceed the BS CBW. (Samsung, Sony, vivo, MTK, LGE) - Proposal 2: One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge. (ZTE) - WF - proposal 1 is agreeable for singe CC case, FFS multi-CC/BS transmitted BW case #### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW - Proposals - Proposal 1: IBE should be used between edges of UE CBW and extended UE CBW. (Samsung, Sony, ZTE, CTC, MTK, vivo, Huawei, LGE) - And the IBE of this part should be equal to that of the original UE CBW edge. (vivo) - it is the “general” part of IBE requirements that applies to the gap between the shifted ΔfOOB and the edge of UE CBW with following adaptation (Huawei) - Use the aggregated NRB which corresponds to the frequency span between two shifted ΔfOOB - The index of the first adjacent RB outside of the UE CBW should be 1 or -1 - Proposal 2: Study the system performance impacts by this extended UE CBW approach, i.e. impacts due to applying IBE instead of ACLR/SEM in the out of band emission regions. (OPPO) - Proposal 3: IBE is unnecessary to use between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW. (Xiaomi) - WF - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW #### Issue 1-2-5: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE) - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson) - WF - ACLR/SEM are applicable to the UE extended CBW edge, up to the BS CBW - BS one CC case - FFS BS multi-CC/transmitted BW case #### Issue 1-2-6: Boundary to apply SE - Proposals - Proposal 1: Do not consider any change to the level and boundary of the spurious emission. (Sony, Xiaomi) - Proposal 2: The application range of SE should be altered with the shifting of the edge of the UE CBW. (Samsung, vivo, MTK, Huawei) - Proposal 3: SE is applied at BS channel bandwidth. (Nokia, Ericsson) - WF - [SE applicability depends on the extended CBW] #### Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE - Proposals - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom) - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony) - WF - FFS in next meeting

Discussion

Approaches for scenario 1

Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator) Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators) Scenario 2: Narrower UE channel BW within wider BS bandwidth

RAN4 are currently focusing on Scenario 2. We also expand the discussion to scenarios 1-1 or 1-2. For Scenario1-1, This can be discussed if a specific band is used by a specific operator.

Proposal 1: Scenario 1-1 can also be discussed if a specific band is being used by a specific operator.

Clarification of BS CBW

In case of TS38.104

[TS38.104] Aggregated BS Channel Bandwidth:The RF bandwidth in which a Base Station transmits and receives multiple contiguously aggregated carriers. The aggregated BS channel bandwidth is measured in MHz Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band NOTE: In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth. BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink NOTE 1: The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements. NOTE 2: It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration.

In last meeting #112. Above definition related to Base Station bandwidth was specified in TS38.104. So we need to clarify BS CHW meaning. According to TS38.104, we need to keep current BS CBS definition. If we want to extend meaning, we should change the word to “BS RF Bandwidth”.

Proposal 2: Keep current BS CBW Definition. If it need to extend the meaning, should change the word to “BS RF Bandwidth”.

Which CBW is utilized as the basis for the integral region of OOBE

if integral region and boundary of OOBE is based on extended UE CBW, spurious emission will not satisfy the ITU-R radio regulations. so The integral region and the boundary of OOBE should be based on UE CBW.

Proposal 3: The integral region and the boundary of OOBE should be based on UE CBW.

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT Device RF requirement impact and proposals are made as following:

Proposal 1: Scenario 1-1 can also be discussed if a specific band is being used by a specific operator.

Proposal 2: Keep current BS CBW Definition. If it need to extend the meaning, should change the word to “BS RF Bandwidth”.

Proposal 3: The integral region and the boundary of OOBE should be based on UE CBW.

- 3. Reference

[1] R4-2414278, “WF on Power domain enhancements”