3GPP TSG-RAN WG4 Meeting # 112bis	R4-2417123

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.8.5

Source:	vivo

Title:						WF on HPUE for IoT NTN

Document for:		Approval

- Tx Requirements

- General & Feasibity

Issue 1-1-1: Whether Iot NTN can be differentiated by handheld and non-handheld.

Agreement:

- Iot NTN can be differentiated by handheld and non-handheld

Issue 1-1-2: How many Tx number should be considered for Iot-NTN (in the specification).

Agreement:

- Focus on 1Tx for PC2 and PC1, further study 2Tx for PC1

Issue 1-1-3: For NB-Iot based Iot-NTN, whether PC1 is feasible for handheld device.

- Proposals

  - Option 1: Yes, (vivo, Qualcomm)

    - Based on measurements for two different PAs. (vivo)

    - Handheld satellite phones which have very high output power exist.(Qualcomm)

  - Option 2: Others

- WF

  - Continue discussion in next meeting.

Issue 1-1-4: For half duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device.

- WF

  - FFS.

Issue 1-1-5: For full duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device.

- WF

  - FFS.

Issue 1-1-6: For Iot-NTN, whether PC1 is feasible for non-handheld device.

Agreement:

- For Iot-NTN, PC1 is feasible for non-handheld device.

- Tx requirements

Issue 1-2-1: Regulatory requirements study.

- Proposals

  - RAN4 shall carefully study the corresponding emission limit from regulators and specify the method to ensure the 3GPP HPUE IoT NTN does not violate them. (Sony)

  - RAN4 may wait for ETSI to specify the new harmonized standard for HPUE IoT NTN before capturing any emission limit from ETSI. (Sony)

  - RAN4 should study applicable regulatory requirements in ECC/CEPT region and take those requirements into account in MPR and A-MPR evaluations. (Qualcomm)

  - Based on the study outcome, necessary corrections should be made also for PC3 in 36.102 (Qualcomm)

  - In case ETSI TC SES identifies later that some requirements are not required for market access in Europe, those requirements should be removed from 3GPP. (Qualcomm)

  - RAN4 to consider introducing protection limits below B255 for Iridium protection as specified in EN 301 681 and ITU-R M.1480 and evaluate how to meet these requirements with higher output power. (Qualcomm)

- WF

  - FFS.

Issue 1-2-2: ACLR

- Proposals

  - Proposal 1: Discuss whether to reuse IoT-NTN PC3 ACLR values for PC2, PC1.5, and PC1 ACLR because the NTN-TN coexistence evaluation [5] shows IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE. (MediaTek)

  - Proposal 2: Based on our co-ex study results, we suggest ACLR for NB IoT-NTN HPUE PC2 can be 24.5 dBc. And this can apply to both handheld and non-handheld devices. (Samsung)

- WF

  - Encourage more input next meeting.

Issue 1-2-3: SEM

- Proposals

  - Proposal 1: Regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing SEM requirements when the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements. (Mediatek)

  - Proposal 2: others.

- Recommended WF

  - Encourage more input next meeting.

Issue 1-2-4: MPR

- Proposals

  - Proposal 1: If IoT-NTN PC3 ACLR value could be reused for IoT-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing IoT-NTN PC3 MPR values for PC2 MPR would be one possible way

  - Proposal 2: Others

- Recommended WF

  - Encourage more input next meeting.

Issue 1-2-5: MOP tolerance

- Proposals

  - Proposal 1: Re-use LTE TN TS 36.101-1 nominal MOP and ±2 tolerance can be considered as starting point (same as for band n14) for PC2 for IoT-NTN HPUE. (Ericsson)

  - Proposal 2: Others

- Recommended WF

  - FFS.

- SAR related

Issue 1-3-1: Duty cycle based approach

- Proposals

  - Option 1: Support the study of this direction.

    - Other solutions than P-MPR need to be specified for HPUE in NTN to ensure a sufficient uplink budget. (Sony)

    - RAN4 shall check if the duty cycle-based approach for SAR can be applied in NTN system. (Sony)

    - Further investigate the duty cycle method for regulation of NTN hand-held UE (oppo)

    - Consider applying the current duty cycle-based SAR solution to Iot based NTN. (vivo)

  - Option 2: Not support this direction.

    - For SAR requirements for handheld UEs, only P-MPR mechanism shall be used, as it is the case for band n14 in LTE TN. (Ericsson)

    - In addition, regarding the SAR requirement for FDD NTN HPUE, we could follow the legacy approach to leave it up to p-MPR design instead of defining any uplink duty cycle to guarantee the SAR limits. (ZTE)

  - Option 3: Others

    - The duty cycle based SAR solution may not be applicable to IoT-NTN cases, as the IoT-NTN uplink duty cycle for each UE (or for each cell/beam) can be up to network implementation. (Samsung)

Agreement: Continue the effort to study other potential solutions in addition to P-MRP solution

Issue 1-3-2: Network related

- Proposals

  - Proposal 1: If the SAN can send its configured uplink ratio for a particular UE or cell or beam, the NTN HPUE should be allowed to transmit with its max Tx power as much as possible, considering the uplink transmission in a NTN scenario can be critical but short in time compared to 6mins SAR regulatory evaluation period. (Samsung)

  - Proposal 2: Send RAN1 LS to check whether the network scheduling can be enhanced to guarantee NTN UE comply with SAR requirements without NTN UE output power reduction. (Huawei)

- Recommended WF

  - FFS

- Rx Requirments

- RSD (for full duplex eMTC based)

Issue 2-1-1: 1Tx or 2Tx for RSD

- Proposals

  - Option 1: For Iot NTN requirements, only 1Tx case be considered. (vivo)

  - Option 2: others

- WF

  - Follow the agreement in Issue 1-1-2 as follows:

    - Focus on 1Tx for PC2 and PC1, further study 2Tx for PC1

Issue 2-1-2: RSD definition

- Proposals

  - Option 1: New RSD tables for different higher power class, similar to NR TN; (vivo)

  - Option 2: New Refsens tables for different higher power class, similar to LTE spec (Table 7.3.1-1b in 36.101); (Ericsson)

  - Option 3: Add additional rows in the existing tables. (Ericsson)

Agreement:

- Follow NR approach for RSD as much as possible.

Issue 2-1-3: PC2 RSD

- Proposals

  - 256:

    - RSD = 0dB (MediaTek, vivo, Samsung)

  - 255:

    - FFS (vivo)

    - Using n3 RSD as reference (Samsung)

  - 254: RSD = 0dB (MediaTek, vivo)

- WF

  - Reuse NR agreements:

- For 256: [0dB] RSD for PC2 for 1Tx and 2Tx (if considered)

  - FFS on the duplexer isolation and PA interference

Issue 2-1-4: PC1 RSD

- Proposals

  - Proposal 1: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (for n254/n255/n256).

- Recommended WF

  - FFS