3GPP TSG-RAN WG4 Meeting #113                                          	    R4-2417511
Orlando, USA, 18th – 22nd November 2024

Source:	Eutelsat Group, Thales, European Space Agency, Sharp, CHTTL, Airbus

Title:	Required corrections to UE transmit timing for Satellite Access

Agenda item:	5.21.3 RRM core and performance requirements

Document for:	Approval

# Introduction

This contribution addresses outstanding issues with the specification of the Timing Error Limit for mobile VSAT served by GSO which were unresolved from Rel-18 which were first raised in [1] with the following observations:

#### Observation 1: For Mobile VSAT served by GSO using 120 kHz SCS (with SSB SCS of 120 kHz or 240 kHz) the timing error formula contains a value indicated between square brackets. The presence of square brackets needs to be addressed as a maintenance action at the RAN4 #113 meeting.

#### Observation 2: The value of [10] in the timing error formula for 120 kHz SCS of uplink signals (for both SCS of SSB of 120 kHz and 240 kHz) results in a Timing Error Limit larger than 50% of the CP. while the timing error needs to be within 50% of the CP for reliable operation. This needs to be addressed as a maintenance action at the RAN4 #113 meeting.

This contribution provides the historical analysis of how the current figures were derived from the agreed UL timing error model agreed in [2] and proposes a revised model and revised assumptions for GNSS positioning errors which resolve the current open issues. A CR to TS 38.133 is then proposed.

# Discussion

## Open issues in TS 38.133 v18.7.0 for mobile VSAT to GSO UL timing

TS 38.133 v18.7.0 Table 7.1C.2-3 defines the minimum requirements for Te_NTN:

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [9] |  |  |  |

Observation 1: For Mobile VSAT served by GSO using 120 kHz SCS (with SSB SCS of 120 kHz or 240 kHz), the value for Te_NTN contains figures in square brackets.

It is not acceptable to have core requirements in a published specification to be in square brackets and this needs to be resolved with a maintenance CR.

It is the accepted principle that for reliable system operation, the guard period Tg should not be negative.

As will be described in more detail later, the guard period Tg =  0.5*TCP – (Te_NTN + Tr + Ta)

The following table repeats TS 38.133 Table 7.1C.2-3 adding six columns to show how the specified Te_NTN relates to the half CP assumption.

Table 1. Te_NTN Timing Error Limit for mobile VSAT is served by GSO 
showing relationship to the guard period Tg

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Te_NTN (ns) | Tr | Ta | 0.5 * CP duration (ns) | Tg Guard period | Tg % of 0.5 * CP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc | 423.2 | 65.1 | 65.2 | 585.9 | 32.55 | 94.44 % |
|  |  | 120 | [10]*64*Tc | 325.5 | 32.6 | 16.3 | 293 | -81.38 | 127.8 % |
|  | 240 | 60 | 13*64*Tc | 423.2 | 65.1 | 65.2 | 585.9 | 32.55 | 94.44 % |
|  |  | 120 | [10]*64*Tc | 325.5 | 32.6 | 16.3 | 293 | -81.38 | 127.8 % |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |  |  |  |  |  |

Observation 2: For Mobile VSAT served by GSO using 120 kHz SCS, the value of [10] for Te_NTN causes the guard period Tg to extend 27.8 %  beyond the half CP limit which is not acceptable for reliable system operation.

## Background to the current NTN RRM UE timing specifications

The subject of UE transmit timing for Satellite Access has been discussed for many meetings but was not resolved before the publication of the specifications after RAN #104 in June 2024. The system error model used to derive the timing specifications was agreed in the RAN4 #108bis Oct 2023 WF [2] as:

### Issue 1-6: Te_NTN for 60kHz and 120kHz Agreement: Companies should provide ‘the exact value of Te_NTN and values assumed for X and Y’ and ‘the analysis result based on the following criterion.’ Otherwise, the values/proposals won’t be captured in the list of options. Tg = 0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm): an effective guard period in CP - Tcp: a length of CP for the given SCS of UL channel/signal - Td: UE downlink synchronization error for the given SCS of SSB (BW of PBCH DMRS, i.e. 20 PRBs) - Tp = Tp,ue + Tp,sat: a round trip propagation delay estimation error due to UE position and satellite position estimation errors - Tp,ue: a round trip propagation delay estimation error due to [X]m of UE position error - Tp,sat: a round trip propagation delay estimation error due to [Y]m of satellite position estimation error - Tr: TAC resolution error (from TS38.213) - Ta: TA adjustment accuracy error (from Table 7.3.2.2-1 of TS38.133) - Tf: an accumulated timing drift over 160ms due to a frequency offset of 0.1ppm - Tm: a margin needed at gNB receiver to accommodate any additional impairments if needed. - If a non-zero value is assumed in the proposal for Tm, the source of the impairments shall be provided too. Technical analysis is required if any number will be provided for each of the components in the next meeting. Whether the same or different values for different channels is contribution driven.

For convenience, the terms are further explained with examples for 120 kHz SCS as follows. All T values are in units of κ = 64 * Tc where Tc = 1/(480 kHz * 4096) = 0.5086 ns.

### Tg – Guard time

Tg is the guard time remaining in the system once all the timing errors have been subtracted from half the CP.

Tg =  0.5*TCP – (Td + TP + Tr + Ta + Tf + Tm)

Observation 3: The guard period is defined as worst case assuming all the errors are uncorrelated and add up linearly. There is an opportunity to consider a more pragmatic  analysis should it be necessary.

### TCP - Length of the CP

The length of the CP for 120 kHz uplink SCS is 18*64*Tc [3] Section 5.3.1.

Tc is the basic time unit of NR based on 480 kHz minimum SCS and 4096 FFT size

Tc = 1/(480,000 * 4096) = 0.509 ns

TCP = 18 * κ = 0.586 µs so half the CP = 0.293 µs

### Td - UE downlink synchronization error

The UE downlink synchronization error is the error in the UE estimation of the SSB timing. It is a function of the bandwidth of the SSB and the assumed SNR. Under high SNR, the minimum error is given by 1/(2*BW) where BW is for the PBCH DMRS, i.e. 20 PRBs or 240 subcarriers. For 120 kHz SCS this BW is 28.8 MHz. This gives a minimum error of 17.36 ns [4].

### TP - Round trip propagation delay estimation error due to positioning

This is the uncertainty in the round-trip propagation delay comprising the sum of Tp,ue, the uncertainty in the round-trip path length due to the VSAT position converted to time, and Tp,sat, the uncertainty in the round-trip path length due to the SAN position converted to time. Because TP is a round-trip uncertainty, Tp,ue and Tp,san represent double the actual positioning uncertainty of the UE and SAN.

Observation 4: From R4-2317275 [7], when setting the requirements for fixed VSAT served by GSO and NGSO, the UE and SAN positioning errors were assumed to be X (UE) = 30 m and Y (SAN) = 15 m.

### Tr – Timing Advance Command (TAC) resolution error

This can be found in [6] Section 4.2 Transmission timing adjustments:

For a SCS of 2μ∙15 kHz, the timing advance command for a TAG indicates the change of the uplink timing relative to the current uplink timing for the TAG in multiples of 16∙64∙Tc2μ. The start timing of the random access preamble is described in [4, TS 38.211].

Therefore, TAC resolution for 120 kHz UL SCS = 16*64*Tc/23 = 128 * Tc

Where µ=3

and Tc = 1/(480000 x 4096) seconds = 0.509 ns

Thus, for 120 kHz SCS, TAC resolution = 128 * 0.509 ns = 65.1 ns.

The maximum error Tr due to the TAC step size is half the step size i.e. 32.6 ns (1 * κ).

### Ta - TA Timing accuracy adjustment error

From the available reference to UE Timing Advance adjustment accuracy in [5] referenced in [2] we can obtain the UE timing advance accuracy.

Table 7.3.2.2-1: UE Timing Advance adjustment accuracy

| UL Sub Carrier Spacing(kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| --- | --- | --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc | ±10 Tc | ±6 Tc |

This pre FR2-NTN reference was subsequently confirmed for FR2-NTN in Table 7.3C.2.2-2 in [5] as follows:

| Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN |  |  |
| --- | --- | --- |
| UL Sub Carrier Spacing(kHz) | 60 | 120 |
| UE Timing Advance adjustment accuracy | 128 Tc | 32 Tc |
| NOTE 1: VSAT UE are defined in TS 38.101-5 |  |  |

For 120 kHz SCS the maximum value of timing advance adjustment accuracy is 32 Tc which is 16.3 ns (0.5 * κ).

Note Table 7.3C.2.2-2 does not define the FR2-NTN Timing Advance adjustment accuracy as double sided. This is inconsistent with the way the accuracy is defined for TN, REDCAP, FR1-NTN Satellite access and ATG in Tables 7.3.2.2-1, 7.3A.2.2-1 7.3C.2.2-1 and 7.3D.2.2-1 respectively. There is no reason to specify FR2-NTN accuracy as a single sided value so this needs to be corrected.

### Tf - Accumulated timing drift

The accumulated timing drift is defined over 160 ms (the longest delay between SSB reception) under the assumption of a frequency offset of 0.1 ppm. The timing drift scales linearly with the relative frequencies so for a 0.1 ppm frequency drift, there will be an equivalent 0.1 ppm timing drift. Thus

Tf = 0.1 /1,000,000 * 160 ms = 16 ns (0.49 * κ).

### Tm - Additional impairments

One possible additional impairment would be multipath delay spread; however, this was not considered in the development of the current NTN RRM UE timing specifications. Provisional analysis suggests a figure of 10 ns may be sufficient given the narrow antenna apertures, so this is not a major component of the timing analysis. Therefore Tm = 0 (0 * κ).

## Derivation of the current FR2-NTN UE transmit Timing Error Limit Te_NTN

Using the error model definitions and figures from the previous section, the current FR2-NTN UE transmit Timing Error Limit Te_NTN was agreed in [8] which was incorporated into the big CR to TS 38.133 in [9].

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | [10]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

### Derivation and analysis of the Fixed VSAT to GSO and NGSO for 120 kHz SCS

For fixed VSAT served by GSO and NGSO at 120 kHz SCS, Te_NTN is given as 7.5 * 64 * Tc (7.5 * κ). From the error model in [2] we can calculate the guard period (in units of κ = 64*Tc) as:

Tg =  0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm)

Tg is a system level figure which includes factors influenced by the UE and the SAN. The goal is to specify Te_NTN so that Tg does not become negative as this indicates unstable system performance.

The Ta error, which impacts the guard period, is due to the UE but is specified independently from Te_NTN in Table 7.3C.2.2-2.

The TAC resolution error Tr also impacts the guard period but is not a function of the UE and is dealt with at the SAN receiver, so Tr is also not included in Te_NTN.

Thus Te_NTN =  Td + Tp  + Tf + Tm

and

Tg =  0.5*Tcp – (Te_NTN + Tr + Ta)

Tp =  0.5*Tcp – (Td + Tr + Ta + Tf + Tm + Tg)

The components of Te_NTN being can now be evaluated.

Tm was assumed to be 0.

From [4], Td is assumed to be 1/(2*SSB BW) for 20 RB which is 1/(2*28.8 MHz) = 17.36 ns (0.53 * κ)

Tf = 0.1 /1,000,000 * 160 ms = 16 ns (0.49 * κ)

Te_NTN was defined in Table 7.1C.2-2 as 7.5 * 64 * Tc = 7.5 * κ

From above, Te_NTN = TP + Td + Tf + Tm  therefore

TP = Te_NTN - Td – Tf – Tm = 7.5 – 0.53 – 0.49 – 0 = 6.48.

The resulting system guard period can then be calculated as:

Tg =  0.5*TCP – (Td + TP + Tr + Ta + Tf + Tm)

Tg =  9 – (0.53 + 6.48 + 1 + 0.5 + 0.49 + 0) = 0

This indicates that a Te_NTN value of 7.5  is the largest that can be specified for a Tg ≥ 0.

By setting Tg to zero we can also calculate the maximum allowable path length error:

Max_Tp =  0.5*Tcp – (Td + Tr + Ta + Tf + Tm)

The derived TP of 6.48 can be converted into a round-trip path length timing error as follows.

Round trip path timing error = TP * 64 * Tc = 6.48 * 64 * 1/(480,000 * 4096) ns = 210.94 ns

This timing error translates into a round-trip path length error of:

210.94 ns * c = 210.94 *10-9 sec * 2.9979 * 108 m/sec= 63.23 m

Using the earlier definitions for the UE one-way path length uncertainty as X, and the SAN one-way path length uncertainty as Y, then the round-trip path length uncertainty = 2 * (X + Y) = 63.23 m. From this we can deduce that the one-way path length error (X+Y) = 31.61 m. This derived figure for the maximum one-way path length error figure is significantly lower than the assumption of X = 15 m and Y = 30 m from [7] noted above in Observation 4.

Observation 5: For fixed VSAT served by GSO and NGSO at 120 kHz SCS, the defined value of 7.5 for Te_NTN implies a maximum one-way path length uncertainty of 31.61 m which is 18.47 m less than the assumption used in R4-2317275 [7] of 45 m.

If the system were to operate with a 45 m one-way path length error this would represent a TP of 45m / 31.61 m * 6.48 = 9.25.

With Tp at 9.25, Tg can be recalculated as:

Tg =  9 – (0.53 + 9.25 + 1 + 0.5 + 0.49 + 0) = -2.77 which is 30.8 % longer than the half CP limit.

Observation 6: For fixed VSAT served by GSO and NGSO at 120 kHz SCS, operating with the assumed (X + Y) one-way positioning error of 45 m results in a guard period Tg of -2.77. This represents a 30.8 % incursion beyond the half CP limit and predicts loss of system performance.

Observation 7: The conclusion from observations 5 and 6 is that for Fixed VSAT the current specification of Te_NTN = 7.5 is inconsistent with the assumptions made for the worst-case SAN + UE positioning error of 45 m. Operating with such an error would result in the guard period exceeding the 0.5 CP limit by 30.8 %.

### Derivation and analysis of the Mobile VSAT to GSO and NGSO for 120 kHz SCS

When the timing analysis for the Mobile VSAT to GSO was carried out, it was assumed that the positioning uncertainty of the SAN plus mobile VSAT would be higher than for the SAN + Fixed VSAT. Using higher estimates, it was not possible to agree Te_NTN any better than [10]. This figure comprises a Td of 0.53, a Tf of 0.49 and hence a derived TP of [8.98].

The TP of 8.98 represents a round-trip path length uncertainty of 87.4 m giving (X + Y) = 43.7 m which is 1.3 m less than the 45 m assumption used for the easier SAN + Fixed VSAT case.

Observation 8: For mobile VSAT served by GSO and NGSO at 120 kHz SCS, the defined value of [10] for Te_NTN implies a maximum one-way path length error of 43.7 m which is 1.3 m less than the assumption used in R4-2317275 [7] of 45 m for the easier SAN + Fixed VSAT case.

The specified value of [10] for Te_NTN results in a negative guard period:

Tg =  9 – (0.53 + 8.98 + 1 + 0.5 + 0.49 + 0) = -2.5

A -2.5 guard period represents a 27.8 % incursion beyond the half CP limit for reliable operation.

Observation 9: For mobile VSAT served by GSO and NGSO at 120 kHz SCS, operating with a derived (X + Y) one-way positioning error of 43.7 m results in a guard period Tg of -2.5. This represents a 27.8 % incursion beyond the half CP limit and predicts loss of system performance.

Observation 10: The conclusion from observations 1, 7 and 8 is that for Mobile VSAT served by GSO, the current specification of Te_NTN = [10] is inconsistent with the assumptions made for the worst-case SAN + UE positioning error of 45 m used for the easier Fixed VSAT case and despite this, would result in the guard period exceeding the 0.5 CP limit by 27.8 %.

## Re-analysis of Te_NTN

In attempting to set the Te_NTN for Rel-18, there was a wide range of assumptions for what positioning accuracy to use, and the figures did not converge. These figures can be seen in [7]. What matters more to the satellite industry is the total positioning accuracy budget into which the system must operate.

For any numerology, the positioning accuracy budget can be calculated. If we assume the goal is to set the guard period Tg to zero, then the available Tp can be calculated as:

TP =  0.5*TCP – (Td + Tr + Ta + Tf + Tm)

In the case of Fixed VSAT with 120 kHz SCS we already have that specification:

TP =  9 – (0.53 + 1 + 0.5 + 0.49 + 0) – 6.48.

This gives an available positioning error of 63.23 m / 2 = 31.61 m.

When it comes to analyzing the budget for Mobile VSAT to GSO, although this is a harder scenario, from a system perspective nothing changes in the 3GPP specifications: there is still only a total positioning budget of Tp = 6.48 which translates to a 31.61 m one way error. It is not useful or realistic to specify a larger figure since this would just cause the system lose performance.

There is some further margin that could be extracted from the error model by adopting a more realistic statistical approach to how the five error terms sum in practise. A linear addition creates a worst-case analysis which is not reflective of the gradual performance degradation that would be observed if the worse-case Tg drops below zero. Such a statistical analysis of the five error terms might allow Tp to grow to around 7.5 which increases the positioning error budget to 36 m. A further analysis of the correlation between VSAT positioning on the ground would allow further margin. This could be the subject of simulations as the mathematics of the geometry is complex involving triple integrals.

In the meantime, it is proposed to resolve the current open issues with the Mobile VSAT to GSO Te_NTN specification by bringing it into line with the figures already specified for Fixed VSAT which already assume Tg = 0. The alignment would mean that for both Fixed and Mobile VSAT, the maximum allowed one-way positioning error for the UE and SAN, is 31.61 m, regardless of whether it is accepted to discount the Tp,san component. This change in Te_NTN definition is not mandating a change in VSAT positing requirements, rather it is indicating to the NTN ecosystem, the maximum allowable positioning error that can be tolerated for any given 3GPP NR NTN numerology.

Proposal 1: To resolve the current issues with Table 7.1C.2-3 the following changes are proposed:

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Also correct Table 7.3C.2.2-2. Table as presented does not define the accuracy as double sided as in Table 7.3.2.2-1 for TN. This should be corrected.

Proposal 2: To resolve the current issues with Table 7.3C.2.2-2 the following changes are proposed:

Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN

| UL Sub Carrier Spacing(kHz) | 60 | 120 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±128 Tc | ±32 Tc |
| NOTE 1: VSAT UE are defined in TS 38.101-5 [43] |  |  |

# Conclusions

Observation 1: For Mobile VSAT served by GSO using 120 kHz SCS (with SSB SCS of 120 kHz or 240 kHz), the value for Te_NTN contains figures in square brackets.

Observation 2: For Mobile VSAT served by GSO using 120 kHz SCS, the value of [10] for Te_NTN causes the guard period Tg to extend 27.8 %  beyond the half CP limit which is not acceptable for reliable system operation.

Observation 3: The guard period is defined as worst case assuming all the errors are uncorrelated and add up linearly. There is an opportunity to consider a more pragmatic  analysis should it be necessary.

Observation 4: From R4-2317275 [7], when setting the requirements for fixed VSAT served by GSO and NGSO, the UE and SAN positioning errors were assumed to be X (UE) = 30 m and Y (SAN) = 15 m.

Observation 5: For fixed VSAT served by GSO and NGSO at 120 kHz SCS, the defined value of 7.5 for Te_NTN implies a maximum one-way path length uncertainty of 31.61 m which is 18.47 m less than the assumption used in R4-2317275 [7] of 45 m.

Observation 6: For fixed VSAT served by GSO and NGSO at 120 kHz SCS, operating with the assumed (X + Y) one-way positioning error of 45 m results in a guard period Tg of -2.77. This represents a 30.8 % incursion beyond the half CP limit and predicts loss of system performance.

Observation 7: The conclusion from observations 5 and 6 is that for Fixed VSAT the current specification of Te_NTN = 7.5 is inconsistent with the assumptions made for the worst-case SAN + UE positioning error of 45 m. Operating with such an error would result in the guard period exceeding the 0.5 CP limit by 30.8 %.

Observation 8: For mobile VSAT served by GSO and NGSO at 120 kHz SCS, the defined value of [10] for Te_NTN implies a maximum one-way path length error of 43.7 m which is 1.3 m less than the assumption used in R4-2317275 [7] of 45 m for the easier SAN + Fixed VSAT case.

Observation9: For mobile VSAT served by GSO and NGSO at 120 kHz SCS, operating with a derived (X + Y) one-way positioning error of 43.7 m results in a guard period Tg of -2.5. This represents a 27.8 % incursion beyond the half CP limit and predicts loss of system performance.

Observation 10: The conclusion from observations 1, 7 and 8 is that for Mobile VSAT served by GSO, the current specification of Te_NTN = [10] is inconsistent with the assumptions made for the worst-case SAN + UE positioning error of 45 m used for the easier Fixed VSAT case and despite this, would result in the guard period exceeding the 0.5 CP limit by 27.8 %.

Proposal 1: To resolve the current issues with Table 7.1C.2-3 the following changes are proposed:

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Also correct Table 7.3C.2.2-2. Table as presented does not define the accuracy as double sided as in Table 7.3.2.2-1 for TN. This should be corrected.

Proposal 2: To resolve the current issues with Table 7.3C.2.2-2 the following changes are proposed:

Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN

| UL Sub Carrier Spacing(kHz) | 60 | 120 |
| --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±128 Tc | ±32 Tc |
| NOTE 1: VSAT UE are defined in TS 38.101-5 [43] |  |  |

# References

[1] R4-2415284	“Feasibility analysis of FR1 and FR2 Numerology” (Eutelsat Group) RAN WG4 #112bis, Hefei, Oct 2024

[2] R4-2317374	“RRM WF and CR work split for NR_NTN_enh” (Moderator (Qualcomm Incorporated)) RAN WG4 #108bis Oct 2023

[3] 3GPP 38.211 v18.4.0 NR; Physical channels and modulation

[4] R4-2318821 	“RRM requirements in above 10 GHz bands” (Ericsson) RAN WG4 #109 Nov 2023

[5] 3GPP 38.133 v18.7.0 NR; Requirements for support of radio resource management

[6] 3GPP 38.213 v18.4.0 NR; Physical layer procedures for control

[7] R4-2317275 “Ad-hoc minutes for [222] NR_NTN_enh” (Ad hoc chair Samsung) RAN WG4 #108bis Oct 2023

[8] R4-2403496 “Draft CR on VSAT UE timing requirements for NTN in above 10GHz” (Samsung) RAN WG4 #110 Feb 2024

[9] R4-2410432 “Big CR to TS 38.133 on RRM core requirements for NR NTN enhancement (Qualcomm Incorporated) RAN WG4 #111 May 2024