3GPP TSG-RAN WG4 Meeting #113				       R4-2417548

Orlando, USA, November, 2024

Agenda item:			7.11.4.1

Source:	Spark NZ

Title:	TP for Confidence Intervals in TR 38 908

Document for:	Discussion and Approval

# 1.0 Introduction

This contribution provides text proposals for the confidence intervals of an EIRP mask measurement. The test beam locations should be carefully selected so that they result in an unbiased estimate of the average EIRP. An example of such locations is provided in this contribution.

# 2.0  Reviewing the WRC 23 Resolution 220 Annex 1

# Compliance with WRC 23 specified mean values for EIRP requires that the mean values measured from a single base station are sufficiently accurate as stated in Annex 1 to resolution 220,

# The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p (e.g. to the confidence interval of 95%).

# Recently offline discussions on the EIRP mask and its related aspects have been led by the moderator of the agenda item. On the computation of the confidence interval, an agreement is:

- Some further clarification from WRC-23 might be needed for better understanding its motivation.

Regretfully the above is almost unachievable and its consequences are not well thought.

There was a lot of discussion during the WRC on confidence intervals and the quantification of the accuracy of an estimate via a 95% CI is a delicately worded compromise. The accuracy of the averaging cannot be considered on its own without the metric of a confidence interval. A CI of 95% means that for 95% of the time the unknown value ( ie the population mean to be defined later) will lie in between the lower and upper confidence limits and for 5% of the time it will not

Clarifications of a WRC decision is unlike sending an LS to the ITU R for clarification. The decisions of a WRC are of a treaty level status and can only be reviewed/clarified by a subsequent WRC. Any other views (or clarifications) will that be of an individual entity and not of the WRC.

There is a formal route to review/ clarify a WRC decision; this is to approach a subsequent WRC. The WRC 27 AI 4 states that:

in accordance with Resolution 95 (Rev.WRC-19), to review the resolutions and recommendations of previous conferences with a view to their possible revision, replacement or abrogation;

If a clarification on the motivation of confidence intervals is to be sought then it will need to be under WRC 27 AI 4 as above.  However, in order to submit a contribution to a WRC, a lot of preparation and ground work is needed, especially in canvassing various Administrations and regional preparatory bodies.

- Is it wise to wait until WRC 27 to finalise the mask compliance of this band?

- Furthermore, opening a discussion on the upper 6GHz band could open the door to many other issues and lead to unintended consequences. Is that desirable?

Observations:

A review of a WRC decision can only be done by a subsequent WRC ie WRC-27 in case of WRC-23.  If this route is to be followed, then it will need to be done via an Administration(s). But such a route is fraught with difficulties, as pointed above and is not advised.

Proposals

- All WRC-23 decisions can only be reviewed/clarified by WRC-27 under AI 4.  Clarifying a decision about the accuracy measure of the EIRP mask under AI 4 is not advisable and could also result in unintended regretful consequences. Such an approach should not be followed.

- The accuracy of the EIRP mask is defined in terms of Confidence intervals in the Annex to the WRC Resolution. This must be implemented in full by the 3GPP.

# 3.0 Definitions

# Below are some definitions that are relevant

# Population Mean; mean value of the  EIRP in the WRC resolution

# Sample mean: Mean value of EIRP computed from K samples following the WRC resolution procedure

# Individual mean:  Mean of an individual beam using step 2 but replacing  Paveθ0,φ0 with just Pθ0,φ0 ; αk,βk for each of the K beams.

Mean of the means, also referred to as global mean. This is the mean of the individual means and should return the same value as the sample mean.

# 4.0  Beam Locations and Steps to compute a Confidence Interval

# The following text shows how the CI is computed, and the number of samples required to achieve a high CI.

# Assume that the expected e.i.r.p values to be complied with are referred to as “Population Means”. The WRC requirement is to have a confidence interval (CI) on the population mean (per elevation angle bin) from a sample of measurements (say from the N beams) made on a single base station– an example of 95% is mentioned. CIs are in fact a margin of error on a point estimate of the parameter (in this case the mean of a population) derived from the given samples.

# There is no value for standard deviation σ specified of the population for each bin (angular range). Nor is the distribution type of EIRP specified; though it can be assumed that the interferers resulting in interference to the FSS uplink come from very large numbers of base stations, and in this case, the Central Limit Theorem applies, and the resulting interference is Normally distributed, and therefore, the corresponding EIRP requirements are also Normal.

# So, the problem boils down to determining the confidence intervals on the mean of a Normal distribution with unknown standard deviation. If the Normality assumption cannot be confirmed, we will need to derive the confidence intervals via the students t-distribution as this is the norm in statistics when the distribution type in unknown.

# If the population standard deviation σ is unknown, as is the case here, it will need to be estimated via the sample standard deviation, s, but this assumption is valid only when the sample size is 30 or more.

# It also assumed that the N beams in the sample are unbiased, independent and equally likely. The first two conditions are challenging to satisfy as the beams are all governed by the same array factor so they have a correlation amongst them.  The sample (test beams) must be carefully selected or else they could return a biased estimate of the mean. One practical way to account for reducing such a bias in the samples is to follow rules of systematic sampling.  According to Ref (1)  systematic samples are convenient to draw and execute but they give poor precision when unsuspected periodicity is present. This could be done following a procedure similar to soil sampling in Webster and Oliver (1990) and shown in the figure  below.  This is based on systematic unaligned sampling.

- Divide the space of all possible beams by means of a coarse grid. Square cells are the norm but not mandatory.

- Superimpose a finer grid (reference grid) in each coarse cell. For example, if there are 5 rows and 5 columns in the coarse grid, it may be appropriate to choose to divide each coarse cell into 25 smaller cells. (Note other course grid configurations may be better suited for the beam configurations of an actual 6GHz antenna.)

- Choose a corner of the coarse grid, say top left, and randomly select 1 of the 25 reference cells. The example shown above has coordinates 2,2 but this is just an example.

- Move horizontally to the next coarse centre in the top row and keep the X coordinate the same but randomly select a new Y coordinate.

- Repeat the process for all the coarse cells in the top row.

- Return to the upper left comer and repeat the process down the first column of cells, this time keeping the Y coordinate the same, but changing the X coordinate in each successively lower coarse cell. The X coordinate of the reference cell immediately to the left and the Y coordinate of the reference cell immediately above the coarse cell of interest are used to determine the sampling points in the remaining coarse grid cells.

Proposal:

It is proposed that beam locations must be carefully chosen so that they provide an unbiased estimate of the average EIRP. An example of such a location for 25 beams is provided above. This method may be adopted by RAN4.

Steps to calculate the Confidence Interval:

- Once the beams are determined and located, for each beam in the sample set compute the local mean by substituting the beams EIRP Pθ0,φ0; αk,βk  as below. The computation of equation(1) below will need the integral to be converted to summations.

EIRPθL , θH=12πsinθH-sinθLθLθH-ππPθ0,φ0; αk,βk cosθ dφ dθ                          (1)

# EIRP per bin per beam= 12πsinθH- sinθL n=1n=Nm=1m=MEIRP (θn ,   , ∅m  ) cos(θn    ) 2πM (θH  - θLN)

- Let the left hand side of the above equation be denoted as pi. – the mean of the ith sample in the set of K beams.  Repeat for all K beams.

- Then the sample mean is just - 1K i=1Kpi , that is to add the above per beam EIRPs and divide by K to determine the mean EIRP (sample mean)

- Estimate the variance of  - 1K i=1Kpi  to compute a confidence interval. by assuming that this estimate derived from the sample of K  beams also provides a good estimate of the corresponding variance of the population. Compute the standard deviation s (square root of the variance) of the sample

- A 95% confidence interval on the population mean (per-bin) with unknown standard deviation for a normal distribution is then given by:

# Lower limit=

- Sample Mean Per bin - 1.96. s/√K

# Upper limit =

- Sample Mean Per bin + 1.96.s/√K

# But a finite population correction needs to be made

# Confidence interval = Sample mean Per bin  ±  21-KMs2K

# Here 1-KM is called the finite population correction.

Note 2: all calculations must be done in linear units

# 5.0 Text Proposals

# The following text shows how the CI is computed, and the number of samples required to achieve a high CI.

# Assume that the expected e.i.r.p values to be complied with are referred to as “Population Means”. The WRC requirement is to have a confidence interval (CI) on the population mean (per elevation angle bin) from a sample of measurements (say from the N beams) made on a single base station– an example of 95% is mentioned. CIs are in fact a margin of error on a point estimate of the parameter (in this case the mean of a population) derived from the given samples.

# There is no value for standard deviation σ specified of the population for each bin (angular range). Nor is the distribution type of EIRP specified; though it can be assumed that the interferers resulting in interference to the FSS uplink come from very large numbers of base stations, and in this case, the Central Limit Theorem applies, and the resulting interference is Normally distributed, and therefore, the corresponding EIRP requirements are also Normal.

# So, the problem boils down to determining the confidence intervals on the mean of a Normal distribution with unknown standard deviation. If the Normality assumption cannot be confirmed, we will need to derive the confidence intervals via the students t-distribution as this is the norm in statistics when the distribution type in unknown.

# If the population standard deviation σ is unknown, as is the case here, it will need to be estimated via the sample standard deviation, s, but this assumption is valid only when the sample size is 30 or more.

# It also assumed that the K beams in the sample are unbiased, independent and equally likely. The first two conditions are challenging to satisfy as the beams are all governed by the same array factor so they have a correlation amongst them.  The sample (test beams) must be carefully selected or else they could return a biased estimate of the mean. One practical way to account for reducing such a bias in the samples is to follow rules of systematic sampling.  According to Ref (1)  systematic samples are convenient to draw and execute but they give poor precision when unsuspected periodicity is present. This could be done following a procedure similar to soil sampling in Webster and Oliver (1990) and shown in the figure  below.  This is based on systematic unaligned sampling.

- Divide the space of all possible beams by means of a coarse grid. Square cells are the norm but not mandatory.

- Superimpose a finer grid (reference grid) in each coarse cell. For example, if there are 5 rows and 5 columns in the coarse grid, it may be appropriate to choose to divide each coarse cell into 25 smaller cells. (Note other course grid configurations may be better suited for the beam configurations of an actual 6GHz antenna.)

- Choose a corner of the coarse grid, say top left, and randomly select 1 of the 25 reference cells.

- Move horizontally to the next coarse centre in the top row and keep the X coordinate the same but randomly select a new Y coordinate.

- Repeat the process for all the coarse cells in the top row.

- Return to the upper left comer and repeat the process down the first column of cells, this time keeping the Y coordinate the same, but changing the X coordinate in each successively lower coarse cell. The X coordinate of the reference cell immediately to the left and the Y coordinate of the reference cell immediately above the coarse cell of interest are used to determine the sampling points in the remaining coarse grid cells.

Proposal:

It is proposed that beam locations must be carefully chosen so that they provide an unbiased estimate of the average EIRP. An example of such a location for 25 beams is provided above. This method may be adopted by RAN4.

Steps to calculate the Confidence Interval:

- Once the beams are determined and located, for each beam in the sample set compute the local mean by substituting the beams EIRP Pθ0,φ0; αk,βk  as  below or via the equivalent conversion of the integral equations to summations.

EIRPθL , θH=12πsinθH-sinθLθLθH-ππPθ0,φ0; αk,βk cosθ dφ dθ                          (1)

# EIRP per bin per beam= 12πsinθH- sinθL n=1n=Nm=1m=MEIRP (θn ,   , ∅m  ) cos(θn    ) 2πM (θH  - θLN)

- Let the left hand side of the above equation be denoted as pi. – the mean of the ith sample in the set of K beams. Repeat for all K beams.

- Then the sample mean is just - 1K i=1Kpi , that is to add the above per beam EIRPs and divide by K to determine the mean EIRP (sample mean)

- Estimate the variance of  - 1K i=1Kpi  to compute a confidence interval. by assuming that this estimate derived from the sample of K beams also provides a good estimate of the corresponding variance of the population. Compute the standard deviation s (square root of the variance) of the sample

- A 95% confidence interval on the population mean (per-bin) with unknown standard deviation for a normal distribution is then given by:

# Lower limit=

- Sample Mean Per Bin - 1.96. s/√K

# Upper limit =

- Sample Mean Per Bin + 1.96.s/√K

# But a finite population correction needs to be made

# Confidence interval = Sample mean Per Bin ±  21-KMs2K

# Here 1-KM is called the finite population correction.

Note 2: all calculations must be done in linear units

William G Cochran, Sampling Techniques , third edition, 1953, John Wiley and Sons

# Webster, R. and Oliver, M.A., 1990: Statistical methods in soil and land resource survey. Oxford: Oxford University Press.

William G Cochran, Sampling Techniques , third edition, 1953, John Wiley and Sons

# Webster, R. and Oliver, M.A., 1990: Statistical methods in soil and land resource survey. Oxford: Oxford University Press.