3GPP TSG-RAN WG4 Meeting #113	R4-2417561

Orlando, US, 18th – 22nd November 2024

Agenda item:	7.1.1.3.2

Source:	MediaTek Inc.

Title:	Feasibility evaluation of MIMO layer for 6Rx UE

Document for:	Discussion

# Introduction

The WI NR_ENDC_RF_Ph4 [1] was approved in RP#103 meeting with one objective “6Rx for handheld and FWA UE” within which one task to RAN4 is to study the gain and feasibility of 6 MIMO layers, targeting at support of handheld UEs.

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#112-bis meeting, an WF [2] was agreed with the relevant part captured as below. Also, companies spent lots of time on discussing link-level simulation assumptions. We copied the detail table in the Appendix A.

Issue 3-1-1: 6-layer Support Way Forward: RAN4 to further discuss the following options (options are not necessarily mutually exclusive). - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to further discuss and align on the feasibility criteria and simulation assumptions by RAN4#113 to evaluate 6-Layer feasibility for handhelds using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Updated simulation assumptions are provided in Annex A for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify and justify (e.g., where the number comes from) the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs. - Option 5: Companies to further discuss the following aspect of 6-layer support. - If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements. - If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only. Issue 3-1-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following. - Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx. Issue 3-1-3: Tx EVM for feasibility study Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following: - Option 1: Use Tx EVM = 1% - Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation - Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM - Option 4: use the existing BS Tx EVM core requirement - Option 5: The representative BS Tx EVM values provided by the BS vendor.

In this paper, we conducted link-level evaluations for the feasibility of 6 layer as well as more evaluated scenarios in system-level simulation.

- Link level simulation

## 2.1 Discussion for simulation assumptions

Tx EVM

According to Table 6.5.2.2-1 of RF specification TS 38.104, the BS EVM requirements is 3.5% for 256QAM.

Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

In RAN4 PDSCH and SDR requirements, RAN4 use the Tx EVM smaller than requirements in TS 38.104. Different Tx EVM values for different modulation order were agreed in [3]:

- Use the following RF impairments models to define the minimum UE performance requirements for FR1 - TX EVM = 6% for QPSK/16QAM/64QAM - TX EVM = 3% for 256QAM

Observation 1: For demodulation requirements, the assumed Tx EVM for 256 QAM is 3%, which is lower than the 3.5% requirement defined in TS38.104.

Observation 2: 256QAM will be used in high SNR regime when evaluating the performance 6layers.

Following the previous agreement in demodulation session and considering 256QAM will be used in high SNR regime, we propose to use Tx EVM = 3% for all of modulation order in MCS Table 2.

Proposal 1: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for feasibility evaluation.

DMRS configuration

There are four options for DMRS configuration in the table of simulation assumptions, which are

- Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L)

- Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L)

- Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L)

- Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L)

The additional DMRS can improve the accuracy of channel estimation and results in higher throughput, especially for high-speed mobility conditions where channel conditions vary rapidly. In [3], it was agreed to use 1 additional DMRS for low (5/10Hz) and medium (100 Hz) speed scenarios.

- DMRS - Type 1 single symbol front loaded - 1 additional DMRS for Low (5/10Hz) and Medium (100 Hz) Speed scenarios - i.e. for full DL slots OFDM symbols with DMRS: #2 and #9 - additional DMRSs for High (400 Hz) Speed scenarios - i.e. for full DL slots OFDM symbols with DMRS: #2, #6 and #9 - FDM is applied between DMRS and data for test cases with 1 Layer and 2 Layers

Therefore, we use DMRS configuration with additional DMRS for feasibility evaluation. Besides, for SU-MIMO transmission with 4 layers, the typical configuration is using antenna port 1000, 1001, 1002 and 1003 for PDSCH and DMRS transmission.

Observation 3: The additional DMRS can improve the accuracy of channel estimation and results in higher throughput

Observation 4: In TS38.101-4, all PDSCH requirements are configured with additional DMRS except PDSCH mapping type B

Observation 5: In TS38.101-4, antenna ports indexes {1000-1003} are used for Rank 4 SU-MIMO requirements

Based on above observations, we use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation.

Proposal 2: Use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation

Antenna efficiency and MIMO correlation

In the last meeting [2], it was agreed that RAN4 will take antenna efficiency into consideration for feasibility evaluation. The MIMO correlation options from Option 1 to Option 4 in Table 1 of Appendix A are ECC (envelop correlation coefficient) values, which gives a measure to show how correlated two different antennas’ radiation patterns are [4]. In RAN4 simulation, the diagonal terms in the MIMO correlation are always 1, which is fine when the test is carried out in a conducted manner. Now in the current feasibility study of 6 layer, companies proposed ECC measurement results of each antenna and suggested using the measurement results for evaluation. In this case, the assumption of all-1 diagonal terms does not hold anymore because the antenna efficiency is not possible to be perfect for all antennas. Therefore, it is fair to also consider the antenna efficiency in this feasibility study. According to [5], the correlation coefficient for antenna pairs depends on ECC and antenna efficiency,

ρpp=ηpp,   if p=qρpq=ηpηqECCp, q,   if p≠q                                                                   (1)

, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q. In Table 1, we provide our measured antenna efficiency and use that to obtain the MIMO correlation coefficient for feasibility study. It can be shown that there is a least 3dB efficiency loss on each antenna.

Table 1. Measured antenna efficiency

|  | ANT0 | ANT1 | ANT2 | ANT3 | ANT4 | ANT5 |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna efficiency (dB) | -7.8 | -6.7 | -5.5 | -3.5 | -4.4 | -3.7 |

With the measured antenna efficiency, equation (1) and ECC Option 1 in the table of Appendix A, we derive the MIMO correlation as below:

ρ=0.1660.0190.0650.0410.0320.0320.0190.2140.0250.0430.0840.060.0650.0250.2820.0430.0640.0420.0410.0430.0430.4470.0440.1310.0320.0840.0640.0440.3630.0390.0320.060.0420.1310.0390.427                                                  (2)

Observation 6: The UE correlation matrices Option 1 to Option 4 in Appendix of WF [2] are ECC, not correlation coefficient.

Observation 7: As RAN4 is considering real antenna performance by adapting the measured ECC values into the link-level simulation, the antenna efficiency needs to be considered as well.

Proposal 3: Use following equations to derive MIMO correlation, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q.

ρpp=ηpp,   if p=qρpq=ηpηqECCp, q,   if p≠q

## 2.2 Simulation results

We provide simulation results with assumptions in Table x below, which is based on above observations and Table in Appendix A.

Table 2. Simulation assumptions for 6Rx performance evaluation

|  |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] |
|  | DMRS Configurations | Rel-15 1+1(4L), Rel-15 2+2(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) |
| UE Correlation matrix |  | Option 1 (from HW R4-2412572): 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 5 (3GPP MIMO correlation matrix with beta=0.00718): 1.000.900.660.390.190.070.901.000.900.660.390.190.660.901.000.900.660.390.390.660.901.000.900.660.190.390.660.901.000.900.070.190.390.660.901.00 Option 6 (option 1 considering antenna efficiency): 0.1660.0190.0650.0410.0320.0320.0190.2140.0250.0430.0840.060.0650.0250.2820.0430.0640.0420.0410.0430.0430.4470.0440.1310.0320.0840.0640.0440.3630.0390.0320.060.0420.1310.0390.427 |

In figure 1, we show the throughout versus SNR for rank 4 and 6 transmission considering MIMO correlation Option 1 and Option 5 in Table 2.

Figure 1. Simulation results for MIMO correlation Option 1 and 3 in Table 1

Observation 10: With correlation matrix from Option 1 and 3, our simulation results show that the throughput of rank 6 is lower than that of rank 4 considering 3% Tx EVM.

With MIMO correlation Option 6 (Option 1 considering antenna efficiency), our results in figure 2 show that more signal power (more than 5dB) is needed to achieve the same throughput as MIMO correlation Option 1, where no antenna efficiency is considered (or assumed to be ideal with 0dB antenna efficiency). As we are now focusing on high SNR regime where 6L6R may outperform 4L6R, this means that it is difficult to achieve the required SNR for 6L6R outperforms 4L6R (if possible) in the field when considering realistic antenna efficiency.

Figure 2. Simulation results for MIMO correlation Option 1 and 6 in Table 2

Observation 11: When considering realistic antenna efficiency, it is difficult to achieve the required SNR for 6L6R outperforms 4L6R (if possible) in the field as more signal power (more than 5dB) is needed.

Based on the observations 10 and 11, we concluded that there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results by taking measured ECC and antenna efficiency into consideration.

Proposal 4: Do not introduce 6 MIMO layers requirements for 6Rx UE.

# Conclusion

In this contribution, we provide our evaluation results and views on the feasibility of 6 layers for 6 Rx UE. Observations and proposals are summarized as follows.

Observation 1: For demodulation requirements, the assumed Tx EVM for 256 QAM is 3%, which is lower than the 3.5% requirement defined in TS38.104.

Observation 2: 256QAM will be used in high SNR regime when evaluating the performance 6layers.

Proposal 1: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for feasibility evaluation.

Observation 3: The additional DMRS can improve the accuracy of channel estimation and results in higher throughput

Observation 4: In TS38.101-4, all PDSCH requirements are configured with additional DMRS except PDSCH mapping type B

Observation 5: In TS38.101-4, antenna ports indexes {1000-1003} are used for Rank 4 SU-MIMO requirements

Based on above observations, we use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation.

Proposal 2: Use DMRS configuration [1+1 (4L), 2+2 (6L)] for feasibility evaluation

Observation 6: The UE correlation matrices Option 1 to Option 4 in Appendix of WF [2] are ECC, not correlation coefficient.

Observation 7: As RAN4 is considering real antenna performance by adapting the measured ECC values into the link-level simulation, the antenna efficiency needs to be considered as well.

Proposal 3: Use following equations to derive MIMO correlation, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q.

ρpp=ηpp,   if p=qρpq=ηpηqECCp, q,   if p≠q

# Reference

- RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon

- R4-2417209, WF on 6Rx UE requirements, AT&T

- R4-1811394, Way forward on NR UE PDSCH demodulation requirements and General aspects, Intel

- Y. Kabiri, A. L. Borja, J. R. Kelly, and P. Xiao, ‘‘A technique for MIMO antenna design with flexible element number and pattern diversity,’’ IEEE Access, vol. 7, pp. 86157–86167, 2019.

- X. Mei and K.-L. Wu, “Capacity-oriented envelope correlation coefficient for multiple antennas of mobile devices,” IEEE Trans. Antennas Propag., vol. 70, no. 10, pp. 9785–9794, Oct. 2022.

- Appendix A: Feasibility Study Link-Level Simulation Assumptions

| Table: Feasibility Study Link-Level Simulation AssumptionsParameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} as baseline Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6}, interested companies can bring {5} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 Interested companies can bring MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) [To be further discussed online] |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx, companies are encouraged to provide details on the SVD update periodicity |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Option 5: 3GPP MIMO correlation matrix with beta=0.00718 (Option 5 is parametric and to be justified since Options 1-4 are based on measurements) Note: For the feasibility analysis, antenna efficiency can be considered. Companies are encouraged to provide the antenna efficiency numbers. |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113 |