3GPP TSG-RAN WG4 Meeting #113	R4-2417573

Orlando, Florida, USA, 18th – 22nd November, 2024

Source:	SoftBank

Title:	TP for TR 38.746: 3Tx inter-band CA_n1-n79, CA_n3-n79 and CA_n28-n79

Agenda item:			6.8.3

Document for:	Approval

1. Introduction

This contribution is a text proposal for TR38.746[3] adding the following 3Tx PC2 combinations.

- CA_n1A-n79A

- CA_n3A-n79A

- CA_n28A-n79A

2. Reference

- 3GPP TR21.905: "Vocabulary for 3GPP Specifications".

- 3GPP RP-241679: “Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)”.

- 3GPP TR38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”: V0.2.0

- 3GPP TS38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)”: V18.7.0

- 3GPP TR37.865-01-01: “NR Carrier Aggregation for intra-band (m Down Link (DL) / 1 Up Link (UL) bands) and inter-band (n Down Link (DL) / 1 Up Link (UL) bands) (Release 15)”: V15.3.0

- 3GPP TR38.716-02-00: “NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 16)”: V16.0.0

Text Proposal

-- Start of TP –

-- Unaffected parts omitted –

5.x	CA_n1-n79

5.x.1	Operating Table 5.x.1-1:  CA band combination of band n1+n79

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n1-n79 | n1, n79 |

5.x.2	Maximum output power

Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n79A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

5.x.3	MSD scenario studies

Based on TR38.716-02-00[6] Table 6.6.1.3-2, there is no harmonic mixing issue for the band combination of n1 and n79.

Based on TR38.716-02-00[6] Table 6.6.2.1-1, the 3rd and 4th order IMD may fall into own Rx of band n79. Since band n79 is TDD band, it is no need to consider the IMD issue for band n79 which means no self-interference for the TDD band n79.

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n1-n79.

Table 5.x.3-1: MSD scenarios for CA_n1-n79 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD | N/A | N/A | N/A |

5.x.4	REFSENS requirements

5.x.4.1	REFSENS requirements for total power class 2

- There is no 2Tx MSD due to harmonic mixing, cross band leakage and IMD, which is applicable for 3Tx.

5.x	CA_n3-n79

5.x.1	Operating Table 5.x.1-1:  CA band combination of band n3+n79

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n3-n79 | n3, n79 |

5.x.2	Maximum output power

Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n79A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

5.x.3	MSD scenario studies

Based on TR37.865-01-01[5] Table 6.6.1.3-1 and 6.6.1.3-2, there is no harmonic issue for the band combination of n3 and n79.

Based on analysis of TR38.716-02-00[6] clause 6.2.2.2, no IMD issues are expected for this CA configuration with dual uplink carrier.

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n3-n79.

Table 5.x.3-1: MSD scenarios for CA_n3-n79 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD | N/A | N/A | N/A |

5.x.4	REFSENS requirements

5.x.4.1	REFSENS requirements for total power class 2

- There is no 2Tx MSD due to harmonic mixing, cross band leakage and IMD, which is applicable for 3Tx.

5.x	CA_n28-n79

5.x.1	Operating Table 5.x.1-1:  CA band combination of band n28+n79

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n28-n79 | n28, n79 |

5.x.2	Maximum output power

Maximum output power for inter-band CA

Table 5.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n79A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

5.x.3	MSD scenario studies

Based on TR38.716-02-00[6] Table 6.9.1.3-2, there is no harmonic mixing issue for the band combination of n28 and n79.

Based on TR38.716-02-00[6] Table 6.9.2.2-1, there is no IMD issue for band combination of n28 and n79.

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n28-n79.

Table 5.x.3-1: MSD scenarios for CA_n28-n79 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD | N/A | N/A | N/A |

5.x.4	REFSENS requirements

5.x.4.1	REFSENS requirements for total power class 2

- There is no 2Tx MSD due to harmonic mixing, cross band leakage and IMD, which is applicable for 3Tx.

-- End of TP –

-- Unaffected parts omitted –