3GPP TSG-RAN WG4 Meeting #113	R4-2417635

Orlando, FL, US, Nov. 2024

Source	:	Qualcomm Incorporated

Title	:	CC activation based enhancement for FR2

Agenda item	:	7.1.1.2.3

Document for	:	Approval

# 1.	Introduction

In RAN#103 a new work item [1] was created to investigate if lower MPRs can be made applicable to FR2 UEs configured for UL CA without the need for a new effort to simulate back off requirements. In a previous WF [4] it was agreed to study activation-based enhancement. In this contribution we share our view on an activation-based MPR enhancement.

# 2. 	Discussion

## 2.1	Background

The relevant agreements from WF [4] are reproduced here for reference:

#### Issue 3-2-2: CC activation-based MPR improvement - WF - Further discuss activated based MPR improvement. - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements. - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case) - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

## 2.2	Common capability for configuration- and activation-based enhancements

From an implementation perspective, the two variants above pose different levels of challenge for a UE to optimize its internal resources in the allowed timeline. As is well understood, the timeline for a configuration-based enhancement is much more relaxed compared to activation-based.

Moreover, logically, if the same capability were used for both enhancement variants, there would be no need to define a configuration-based enhancement at all, which goes against previous agreement.

Observation 1: Adopting a common capability for configuration- and activation-based enhancements conflicts with prior agreement to specify a configuration-based enhancement.

Proposal 1: For Rel-19 FR2 UE MPR, define separate capabilities for configuration-based enhancements and activation-based enhancement.

## 2.3	BW basis for activation-based enhancement

In general, the BW basis can change to BW of activated CCs. In the standard however, it is necessary to address corner cases also, like the scenarios identified below (reproduced from [3]):

- Non-contiguous CCs are activated inside a contiguous CA configuration.

- The activated CCs that are also enabled for UL do not form a contiguous block.

The first option is to identify all the corner case like the two above and accommodate them with careful wording. Experience has shown there can be significant readability problems as well as potential for missing cases that come to light over time. The other option for compactness and simplicity is to limit the enhancement to cases where the activated CCs enabled for UL form a contiguous block. This latter method is also preferable from a deployment standpoint because it encourages operators to activate CCs in a way that maximizes the enhancement.

Proposal 2: RAN4 to consider as an optional capability:

FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration that are also enabled for UL, provided the activated CCs with UL comprise a contiguous block. (For reference, legacy CA MPR is based on cumulative aggregated channel BW).

# 3. 	Conclusion

Observation 1: Adopting a common capability for configuration- and activation-based enhancements conflicts with prior agreement to specify a configuration-based enhancement.

Proposal 1: For Rel-19 FR2 UE MPR, define separate capabilities for configuration-based enhancements and activation-based enhancement.

Proposal 2: RAN4 to consider as an optional capability:

FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration that are also enabled for UL, provided the activated CCs with UL comprise a contiguous block. (For reference, legacy CA MPR is based on cumulative aggregated channel BW).

# 3.	References

[1] 	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, March 18th – March 21st , 2024

[2] 	R4-2410749, ‘WF on power domain enhancements’, Huawei, RAN4#111, May 2024

[3] 	R4-2413226, ‘Activation-based FR2 MPR enhancement’, Qualcomm, RAN4#112, Aug. 2024

[4] 	R4-2417204, ‘WF on power domain enhancements’, Huawei, RAN4#112-Bis, Oct. 2024