3GPP TSG-RAN4 #113	R4-2417672

Orlando, USA, 18th Nov. – 22nd Nov. 2024

Title:	Numerical solution of equations of satellite motion under the ECEF coordinate system

Source:			Anritsu Corporation

Agenda Item:			7.8.4

Document for:			Approval

# 1.	Introduction

At the RAN4 #112-bis meeting in Hefei, we discussed a methodology for time varying doppler shift and propagation delay modelling of satellite motion, and agreed to use the two-body Keplerian propagator model [1, 2], extracted for reference.

Sub-topic 1-1 TE emulated channel model for satellite mobility Issue 1-1-1 Methodology for Time varying Doppler Shift and propagation delay modelling - WF - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay model matching a satellite motion with following option: - Satellite motion emulated using the two-body Keplerian propagator model, [i.e., spherical gravity model] - RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure - Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to derive the Doppler shift and propagation delay.

In this contribution we share our proposal on a numerical solution of equations of satellite motion under the ECEF (Earth Centred Earth Fixed) coordinate system, utilizing the 4th order Runge-Kutta method.

# 2.	Discussion

  - Equations of motion followed by satellites

A motion followed by satellites under the ECEF frame, in other words CTRS (Conventional Terrestrial Reference System), can be expressed by the following equation (1) [3].

d2xdt2=-∂∂rUr+2  dxdt×Ω +Ω×x×Ω                                                                               (1)

x=(x,y,z)                                                                                                                                  (2)

r=x2+y2+z2                                                                                                                                      (3)

Here Ω is the WGS-84 Earth rotation rate, Ω=0, 0, 7.2921151467×10-5 radsec.. And x, y, z are a coordinate of satellite position state vector in ECEF.

Based on the previous agreement [2], assuming the gravitational potential follows the two-body Keplerian, the potential term U(r) is expressed as follows:

Ur= -μr                                                                                                                                                  (4)

Here μ is the Earth gravitational parameter (μ= GM ). μ=3.986005×1014[m3/sec.2]

Thus, the motion equation (1) to be solved under the condition of the ECEF frame and two-body Keplerian can be expressed as follows:

d2xdt2=-μ  r2 xr +2  dxdt×Ω +Ω×x×Ω                                                                            (5)

Transforming the expression of the second-order ordinary differential equation (5) into the form of a simultaneous equation yields the following:

dvdt=-μ  r2 xr +2 v×Ω+Ω×(x×Ω)                                                                                   (6)

dxdt=v                                                                                                                                                           (7)

Observation 1: The motion equation to be solved under the condition of the ECEF frame and two-body Keplerian can be expressed as follows:

dvdt=-μ  r2 xr +2 v×Ω+Ω×(x×Ω)

dxdt=v

2.2 Numerical solution (4-th order Runge Kutta Method) [4]

Next, we solve the simultaneous equations (6) and (7) utilizing the 4-th order Runge Kutta method. Δt is a step size of time. From the position and velocity (xn,vn) at time tn(=n Δt), equations (12), (13), (14), (15) and (16) are successively calculated to determine (xn+1,vn+1) at time tn+1.

xn= xn ,yn ,zn                                                                                                                                  (8)

vn=vx, n ,vy,n ,vz,n                                                                                                             (9)

rn=xn2+yn2+zn2                                                                                                                 (10)

a xn ,vn  =-μ rn2  xnrn +2 vn×Ω+Ω×xn×Ω                                                           (11)

k(1,  v)=a xn,vn  Δt                                                                                                                              (12-1)

k(1, x)=vn Δt                                                                                                                                              (12-2)

k(2, v)=a xn+12k1, x ,vn+12k1, v Δt                                                                                              (13-1)

k(2,  x)=vn+12k1, vΔt                                                                                                                           (13-2)

k(3, v)=a xn+12k2, x ,vn+12k2, v Δt                                                                                             (14-1)

k(3,  x)=vn+12k2, vΔt                                                                                                                           (14-2)

k(4, v)=a xn+k3, x ,vn+k3, v  Δt                                                                                                       (15-1)

k(4,  x)=vn+k3, v Δt                                                                                                                                 (15-2)

vn+1=vn+16k(1,  v)+2k(2,  v)+2k(3,  v)+k(4,  v)                                                                             (16-1)

xn+1=xn+16k(1,  x)+2k(2,  x)+2k(3, x)+k(4,  x)                                                                              (16-2)

The vector function a( xn ,vn  ) with a condition Ω=0,0,Ω can be expressed specifically by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2                                                      (17-1)

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2                                                       (17-2)

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn                                                                                          (17-3)

Note that the condition Ω=0,0,Ω is set when using the ECEF frame such that the centre of earth's gravity is the origin, and the rotation axis is the z-axis.

Observation 2: By utilizing the 4-th order Runge-Kutta method, the vector function can be expressed by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn

2.3 Initial conditions for numerical solution

Based on the definition in TS 38.508-1 [5], we use the following position and velocity of a LEO satellite and position of a UE as initial conditions. Note these values are described with ECEF frame in Table 5.6.2.1-2 and described with the ellipsoidal coordinate system in sub-clause 5.6.1.

Observation 3: Note the position and velocity of satellite are described with ECEF frame, while the position of the UE is described with the ellipsoidal coordinate system in TS 38.508-1.

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

UE location (TS 38.508-1)

5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

The position and velocity of the satellite described under ECEF Frame need to be converted based on the definitions in TS 38.331 [6] as EphemerisInfo field descriptions.

EphemerisInfo field descriptions

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

The converted satellite coordinate under ECEF Frame (X_SV, Y_SV, Z_SV):

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

The converted satellite velocity under ECEF Frame (VX, VY, VZ):

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

Also, the position of the UE needs to be converted to ECEF Frame by the following equations since the definitions of the UE location in TS 38.508-1 above are described based on the ellipsoidal coordinate system.

xyz=N+hcosϕcosλN+hcosϕsinλN1-e2+hsinϕ#(18)

Here f is geodetic latitude, l is east longitude, h is altitude from geoid, a is average earth equatorial radius, e=f(2-f)  where f is the earth’s declination, and

N=a1-e2sin2ϕ1 2

The converted UE coordinate under ECEF Frame:

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

Observation 4: Initial conditions of satellite position, velocity and UE position under ECEF frame can be as follows.

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

2.4 Calculation results of propagation delay and doppler frequency shift

Using the aforementioned equations, we calculated the propagation delay and doppler shift of the LEO-600 satellite with two different step sizes, one with Dt = 1 ms and the other with 10.24 sec. Figure 2.4-1 depicts the propagation delay [ms] vs time [min], and Figure 2.4-2 depicts the doppler shift [kHz] vs time [min].

Propagation delay [ms] vs Time[min.]

Plot with blue line: Results with Δt = 1 [msec.]
Plot with blue circle: Results with Δt = 10.24 [sec.]

Figure 2.4-1: Propagation delay of LEO-600

Doppler frequency shift [kHz] vs Time[min.] , (Centre Frequency = 2 [GHz])

Plot with blue line: Results with Δt = 1 [msec.]
Plot with blue circle: Results with Δt = 10.24 [sec.]

Figure 2.4-2: Doppler frequency shift of LEO-600

As can be seen above, we observed that the calculated results which are numerically analysed by 4-th order Runge Kutta method fits quite well regardless of the step size of time (1 ms vs 10.24 sec.). Errors of the delay between 1 ms step size and 10 sec step size is within the range of +/- 100 ns and the doppler shift is within +/- 3 mHz. The actual calculation comparisons are shown in the appendix A of this contribution.

Observation 5: Calculation results of propagation delay and doppler shift utilizing the 4-th order Runge Kutta method fit quite well regardless of the step size of time (1 ms vs 10.24 sec.).

Observation 6: Errors of the calculated delay between 1 ms step size and 10 sec step size is within a range of +/- 100 ns, and the errors of doppler shift is within +/- 3 mHz.

Proposal 1: For numerical solution of equations of satellite motion under the ECEF (Earth Centred Earth Fixed) coordinate system, use the 4-th order Runge Kutta method.

- Conclusion

In this contribution we shared our proposal on a numerical solution of equations of satellite motion under the ECEF (Earth Centred Earth Fixed) coordinate system, utilizing the 4th order Runge-Kutta method.

Observation 1: The motion equation to be solved under the condition of the ECEF frame and two-body Keplerian can be expressed as follows:

dvdt=-μ  r2 xr +2 v×Ω+Ω×(x×Ω)

dxdt=v

Observation 2: By utilizing the 4-th order Runge-Kutta method, the vector function can be expressed by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn

Observation 3: Note the position and velocity of satellite are described with ECEF frame, while the position of the UE is described with the ellipsoidal coordinate system in TS 38.508-1.

Observation 4: Initial conditions of satellite position, velocity and UE position under ECEF frame can be as follows.

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

Observation 5: Calculation results of propagation delay and doppler shift utilizing the 4-th order Runge Kutta method fit quite well regardless of the step size of time (1 ms vs 10.24 sec.).

Observation 6: Errors of the calculated delay between 1 ms step size and 10 sec step size is within a range of +/- 100 ns, and the errors of doppler shift is within +/- 3 mHz.

Proposal 1: For numerical solution of equations of satellite motion under the ECEF (Earth Centred Earth Fixed) coordinate system, use the 4-th order Runge Kutta method.

# 4.	References

[1] R4-2415377, “Views on TE emulated channel model for satellite motion”, Anritsu, RAN4 #112-bis, Hefei, China

[2] R4-2416556, “Way Forward for [112bis][321] NTN_testing_NGSO_channel_model”, Moderator (Samsung), RAN4 #112-bis, Hefei, China

[3] “Mechanics”, L. D. Landau and E. M. Lifshitz, 2nd edition

[4] “Numerical Recipes in C”, William H. Press, Saul A. Teukolsky, William T. Vetterling, and Brian P. Flannery, 2nd edition

[5] TS 38.508-1, "User Equipment (UE) conformance specification; Part 1: Common test environment", v 18.4.0, 2024-09

[6] TS 38.331, “Radio Resource Control (RRC); Protocol specification”, v18.3.0, 2024-9

Appendix A: Comparison of calculation results between the 1 ms step size and 10.24 sec. step.

Errors between the calculation results of the 1 ms step and 10.24 sec. step are shown below.

Propagation delay calculation error: Less than +/- 100 ns

Doppler frequency shift calculation error: Less than +/- 3 mHz