3GPP RAN WG4 Meeting #113	R4-2417704

Orlando, USA, November 18th – November 22nd, 2024

Title: 	Discussion on testability and interoperability issues for BM

Source: 	CATT

Agenda item:	7.17.4.2

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 discussed on Rel-19 AI/ML for NR air interface and agreements are captured in [1]. There are still some issues related to testability and interoperability for beam management that needs to be further discussed. In this contribution, we will continue to discuss the open issues summarized in [2].

Discussion

RSRP absolute accuracy

In last meeting, the definition of RSRP absolute accuracy was agreed with a bracket. The discussion point is whether to remove the bracket.

Issue 2-3: RSRP absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Considering the above definition of absolute RSRP accuracy is applicable for any beams including predicted beams, we prefer to remove the bracket.

Proposal 1: Remove the bracket in the definition of absolute RSRP accuracy.

Relative RSRP accuracy

The following definition of relative RSRP accuracy actually is the same as the legacy definition:

Option 2: Relative RSRP accuracy is the RSRP difference between the predicted L1-RSRP of different beams

In general, we are okay with this definition if both absolute RSRP accuracy and relative RSRP accuracy are verified in one test case.

Proposal 2: If absolute RSRP accuracy and relative RSRP accuracy are verified in one test case, relative RSRP accuracy can be defined as the RSRP difference between the predicted L1-RSRP of different beams.

Metrics/KPIs for beam prediction

There were no agreements on metrics/KPIs for beam management in previous RAN4 meeting. The latest list of options is summarized below for reference:

RAN4#111: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options RAN4#112bis: - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs. - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: Other

The definitions of absolute RSRP accuracy and relative RSRP accuracy are discussed above. In regards with the beam prediction accuracy, all three sub-options in Option 2 in RAN4#111 are feasible. The main difference is whether a margin shall be considered for beam ID prediction as the figure below illustrate.

Figure1. Illustrations of three sub-options for Option 2

In addition, a common issue to calculate the beam prediction metric is how to determine the Top-1 or Top-K measured beams, if UE does not measure the full set A, which is very likely considering the measurement overheads. We find that RAN1 currently has been discussing monitoring metrics and the alternatives are captured in section 2.1. In our opinion, the RAN1 agreements on beam prediction monitoring metric can be directly reused for tests. Hence, RAN4 can wait for RAN1 progress.

Proposal 3: For beam prediction accuracy, RAN4 waits for RAN1 progress in monitoring metric which could be reused for beam prediction test.

Conclusions

This paper discussed some issues related to AI/ML for beam management, and following proposals are provided:

Proposal 1: Remove the bracket in the definition of absolute RSRP accuracy.

Proposal 2: If absolute RSRP accuracy and relative RSRP accuracy are verified in one test case, relative RSRP accuracy can be defined as the RSRP difference between the predicted L1-RSRP of different beams.

Proposal 3: For beam prediction accuracy, RAN4 waits for RAN1 progress in monitoring metric which could be reused for beam prediction test.

References

[1] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#112bis.

[2] R4-2415231, Topic summary for [112-Bis][127] NR_AI/ML_air, Qualcomm Incorporated, RAN4#112bis.