3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417722

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	7.15.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and give our views.

2 Discussion

In last meeting, as agreed below [1], faster beam sweeping always applies during handover and RRC re-establishment while trigger conditions are needed for intra-frequency and inter-frequency L3 measurements to activate or deactivate the Multi-Rx operation. And it was agreed that the criteria/threshold(s) are configured by the network through RRC signaling. But the detailed conditions are FFS which need to be discussed this meeting.

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

With this agreement, individual network signaling to indicate Multi-Rx operation in active mode is not needed anymore. As long as the criteria/threshold(s) are configured, UE can decide whether to activate the Multi-Rx operation according to its current status. For the detailed conditions, there are the following options listed in the last meeting:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

- [FFS QoS requirement]

- Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline

In the existing RRC IDLE/INACTIVE state requirements, the conditions for relaxed measurement are defined, i.e., when the certain conditions are met, the cell reselection measurement requirements can be relaxed. The same principle is required here for the use of Multi-Rx operation, so we think the same criteria can also be used as baseline and the applied threshold can be further discussed. For simplification, we think the threshold can also be reused since the intention and principle are same as relaxed measurements, but we are open to further think about whether there are any differences between the two procedures.

Then the next issue is the UE behaviours according to the different conditions. Combining the request of Multi-Rx, we understand the framework of the Multi-Rx operation is that:

- Network configures the criteria and thresholds by system information;

- For a UE supporting Multi-Rx operation,

  - When both low mobility criterion and not at cell edge criterion are fulfilled, UE is required to perform the measurement with legacy beam sweeping factor, i.e., 8.

  - When one of the criteria is not fulfilled, UE is required to perform the measurement with Multi-Rx operation, i.e., enhanced beam sweeping factor.

The above configurations and UE behaviors are needed to be specified in RAN2 spec, but since this is RAN4 led work item and no dedicated TU is allocated to RAN2, it is better to decide all the conditions and thresholds in RAN4 and send LS to RAN2 after the details are decided.

Proposal 1: The existing relaxed measurement criteria including low mobility and not at cell edge can be reused to define trigger conditions of Multi-Rx operation for L3 measurement.

Proposal 2: The UE behaviors based on trigger conditions of Multi-Rx operation are as below:

- For a UE supporting Multi-Rx operation,

  - When both low mobility criterion and not at cell edge criterion are fulfilled, UE is required to perform the L3 measurement with legacy beam sweeping factor, i.e., 8.

  - When at least one of the criteria is not fulfilled, UE is required to perform the L3 measurement with Multi-Rx operation, i.e., enhanced beam sweeping factor.

Proposal 3: Send LS to RAN2 about the criteria/thresholds for triggering Multi-Rx operation on L3 measurements and the corresponding UE behaviors after the whole framework is decided.

For searcher assumption, the following way forward was captured in the approved WF [1]:

Issue 1-1-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - Option 1 (CATT, Apple, Samsung, ZTE, QC, NTT DCM): For Multi-Rx based L3 measurement, 1 searcher is assumed for single carrier. - Option 2 (vivo, LGE, OPPO): For Multi-Rx based L3 measurement, 2 searcher is assumed for single carrier. - Option 3 (CTC): Searcher occupied by single carrier is depended on UE implementation, 1 searcher or 2 searchers can be occupied by the single carrier. - Option 4 (HW): RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion.

The issue was not discussed in last meeting, but according to the previous discussion, the common understanding is that 2 searchers in total are assumed for a UE which is same as legacy. And based on the existing CSSF definition as below, for the measurement outside gap, one searcher is used for PCC and the other searcher is used for SCC, inter-frequency without gap and inter-RAT measurement without gap. With this assumption, we understand even if no SCC is configured, the searchers still need to be shared by intra-frequency, inter-frequency and inter-RAT frequency measurement without gap.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap; CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD]. FR2 only intra band CA; N/A; N/A; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only inter band CA; N/A; N/A; 1+NPCC_CSIRS; 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM).

The issue here is only one target carrier is assumed in this BSF optimization discussion since we do not consider CA case at this stage, so one searcher may be spare in some cases (e.g., for intra-frequency measurement, only one serving carrier is measured) if following legacy assumption. In such case, it is possible to consider using the other searcher in this carrier.

But based on the previous agreements, for Multi-Rx based L3 measurement, the target carrier can be same or different with serving carrier. If the two carriers are different, i.e., inter-frequency measurement, it cannot be assumed that 2 searchers are used for single carrier, because the two searchers need to be allocated to different carriers, i.e., one searcher for serving carrier and the other one for inter-frequency measurement.

To accommodate the possible CA/DC discussion and align the assumption for intra-frequency and inter-frequency measurement, we think one searcher for single carrier can be assumed.

Proposal 4: For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

With the assumption above, according to the discussion in previous meeting, another issue is the additional SSB processing time. If the UE receives reference signals from different cells within the same SMTC using Multi-Rx, since the multiple SSBs within the SMTC are from different directions and have different beam index, UE may not be able to process the SSBs simultaneously. Thus additional processing time is needed. Usually we think the interval between two SMTCs can accommodate this processing time, and the only exception case is that the measurement starts from the end of one SMTC occasion. To consider this case and define the general requirements, we are fine to add this additional processing time in the delay requirements. But this additional processing time is only required for L3 measurement and can be added after the last SMTC occasion of the total identification delay, i.e., Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms. For handover and RRC re-establishment/RRC Connection Release with Redirection, we do not think this additional delay is needed since there are already margins for processing time.

Proposal 5: With the assumption in proposal 4, additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

Proposal 6: For handover and RRC re-establishment/RRC Connection Release with Redirection, no additional processing delay is needed.

For DRX issue, we understand the requirements applicability of enhanced solution is agnostic to the DRX status and no need to define the priority. If the concern is necessity or power saving issue, it can be controlled by network configuration and trigger conditions, and the requirement is better to be general.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX - Option 1 (Xiaomi): The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration. - Option 2 (CATT, CMCC, Samsung, CTC, NTT DCM, Nokia): L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case. - Option 3 (Ericsson): - L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance. - When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective.

Proposal 7: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

For the solutions of optimizing Rx BSF, we think the conclusions in Rel-18 Multi-Rx can be used as baseline, i.e., for UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. And according to the agreed scenarios in previous meeting, the reduced Rx BSF applies to intra/ inter-frequency SSB based measurement, handover, RRC Re-establishment/RRC Connection Release with Redirection.

Although we do not explicitly agree to discuss the requirements for CSI-RS measurements, after the requirements for SSB measurement is decided, the enhanced Rx BSF can also apply to the associated SSB in CSI-RS based measurement since it refers to formula of SSB based measurement, but no spec updates are needed. Similarly for handover, since the requirements for CHO and DAPS HO refer to the formula of SSB based measurement and handover respectively, the enhanced Rx BSF can also apply but no specification impact is expected.

Proposal 8: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 9: The reduced Rx BSF in proposal 8 can be applied to the following formula according to the trigger conditions to be specified:

- TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO;

- Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

- Tidentify-NR for RRC connection release with redirection to NR.

Proposal 10: The reduced Rx BSF in proposal 8 can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

After defining the requirements for Multi-Rx based L3 measurement, one additional issue raised in previous meeting was that whether transition period requirements are needed for the transition between measurements with Multi-Rx and without Multi-Rx operation.

Issue 1-4-1: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement - Proposal (ZTE): - The requirement at transition between the normal L3 measurement and FBS L3 measurement should be identified. There are two options: - Option 1: Reuse similar solution as the requirements at transition between measurement within gap and measurement outside gap, i.e. the longer delay is the period when transition happens. - Option 2: Once the transition happens, apply the requirements after the transition, no matter it is the normal L3 measurement or the FBS L3 measurement.

In the existing specification, there are such requirements defined for the transition between measurements with gaps and without gaps. If we are going to define the requirements for transition between faster and slower measurements due to Multi-Rx operation, we think it is simple and reasonable to reuse the existing requirements, i.e., use the longer delay between the two statuses. But we see there may be some differences between the Multi-Rx operation and legacy gap impact. Based on the discussions on trigger condition, whether UE is performing the measurements with or without Multi-Rx operation depends on UE current status, and network is not aware of it and never know whether UE has such transition. Then it is possible that UE always applies the longer delay, i.e., with the legacy beam scaling factor. So we think it is meaningless to define the transition requirements.

Proposal 11: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

3 Summary

In this paper, we have some further discussions on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Proposal 1: The existing relaxed measurement criteria including low mobility and not at cell edge can be reused to define trigger conditions of Multi-Rx operation for L3 measurement.

Proposal 2: The UE behaviors based on trigger conditions of Multi-Rx operation are as below:

- For a UE supporting Multi-Rx operation,

  - When both low mobility criterion and not at cell edge criterion are fulfilled, UE is required to perform the L3 measurement with legacy beam sweeping factor, i.e., 8.

  - When at least one of the criteria is not fulfilled, UE is required to perform the L3 measurement with Multi-Rx operation, i.e., enhanced beam sweeping factor.

Proposal 3: Send LS to RAN2 about the criteria/thresholds for triggering Multi-Rx operation on L3 measurements and the corresponding UE behaviors after the whole framework is decided.

Proposal 4: For Multi-Rx based L3 measurement, one searcher is assumed for single carrier.

Proposal 5: With the assumption in proposal 4, additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement.

Proposal 6: For handover and RRC re-establishment/RRC Connection Release with Redirection, no additional processing delay is needed.

Proposal 7: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Proposal 8: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 9: The reduced Rx BSF in proposal 8 can be applied to the following formula according to the trigger conditions to be specified:

- TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO;

- Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay;

- Tidentify-NR for RRC connection release with redirection to NR.

Proposal 10: The reduced Rx BSF in proposal 8 can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

Proposal 11: Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

4 References

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#112bis

- R4-2415656, Topic summary for [112bis][209] NR_RRM_Ph5_Part1, Apple, RAN4#112bis