3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417738

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on RAN2 LS on Relaxed measurement

Source:	CATT

Agenda Item:	8.3

Document for:	Discussion

1 Introduction

In this meeting we received one LS [1] from RAN2 on Rel-16 relaxed measurement in which RAN2 thinks that RAN4 specification defines a behaviour for the condition “if the serving cell measurement is better than s-NonIntraSearch thresholds but the cell-edge criteria is not fulfilled” while the case will not happen from RAN2 side. And based on the observation, RAN2 asks RAN4 to check the spec and to make the updates if needed.

In this paper, we discuss the case mentioned in the LS and give our views. The draft response LS is provided in the Annex based on the discussions.

2 Discussion

The RAN2 LS on Rel-16 relaxed measurement is cited as below:

1. Overall Description: RAN2 has discussed about the scenarios for the Rel-16 RRM relaxation in Idle/Inactive state during the RAN2#127bis meeting. As a conclusion, RAN2 has in-principle agreed the CR to TS38.304 [1], removing the description of conditions when measurements are relaxed and adding a reference to RAN4 specification. During the discussion, RAN2 noticed that RAN4 specification defines a behaviour for the condition “if the serving cell measurement is better than s-NonIntraSearch thresholds but the cell-edge criteria is not fulfilled”. From RAN2 point of view, the case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criteria is not fulfilled” will not happen and cannot happen because of the following field description for s-SearchThresholdP and s-SearchThresholdQ in TS38.331. s-SearchThresholdP, s-SearchThresholdP2 Parameters "SSearchThresholdP" and "SSearchThresholdP2" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP.. s-SearchThresholdQ, s-SearchThresholdQ2 Parameters "SSearchThresholdQ" and "SSearchThresholdQ2" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ.. 2. Actions: To RAN4. ACTION: RAN2 respectfully asks RAN4 to take the above into consideration and inform that RAN4 can update their part of their specs, if they see a need.

RAN2 noticed the case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criteria is not fulfilled” will not happen because the thresholds for cell-edge criteria s-SearchThresholdP and s-SearchThresholdQ are configured to be less than or equal to s-IntraSearchP/ s-NonIntraSearchP and s-IntraSearchQ/s-NonIntraSearchQ based on the description in TS 38.331.

We agree the observation that this case will not happen, but it is not clear in RAN2 LS which parts in RAN4 spec have defined requirements for this case. After checking TS 38.133, we do not see the problem in RAN4 spec since no specific requirements are defined for this case.

In RAN4 specification, taking inter-frequency measurement as example, we have defined the following three cases for relaxed measurement:

- Case 1: Measurements for UE fulfilling low mobility criterion

- Case 2: Measurements for UE fulfilling not-at-cell edge criterion

- Case 3: Measurements for UE fulfilling low mobility and not-at-cell edge criterion

It is obvious that the mentioned case is not case 2 or case 3 since the cell-edge criterion is fulfilled. Then the only possible is for case 1.

For case 1, we have defined the requirements for the condition “when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ” and the condition “when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ”. And the requirements apply for the following two sub-cases:

- Sub-case 1-1: UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion

- Sub-case 1-2: UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion, and UE has fulfilled only the lowMobilityEvaluation [2] criterion.

The impossible case is when sub-case 1-2 is applied, i.e., cellEdgeEvaluation criterion is not fulfilled and the condition “when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ” is met.

But we understand RAN4 spec doesn’t mean that all the four scenarios in case 1 are necessary. The serving cell measurement criteria and relaxed criteria are two individual aspects. UE can check the two criteria and apply the corresponding requirements if the criteria are met. Even if any scenario will not happen based on network configuration, UE will not apply that requirement and there is no harm to RAN4 specification. For example, when the condition “Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ” is met, if cellEdgeEvaluation criterion cannot be fulfilled, it does not matter since we still have the case when lowMobilityEvaluation criterion is fulfilled and the requirements are still needed. From RAN4 side, we think we do not need to remove the scenario just because the case is precluded by network configuration especially when the restriction is already reflected in high layer specification.

Based on the discussions above, we think no updates are needed for RAN4 spec and suggest sending response LS to RAN2 to avoid any misunderstanding between different groups.

Proposal 1: No updates are needed for RAN4 spec due to the impossible case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criterion is not fulfilled”.

Proposal 2: Send response LS to RAN2 to inform the RAN4 conclusions.

3 Summary

In this paper, we discuss the RAN2 LS on relaxed measurement, and the following proposals are given:

Proposal 1: No updates are needed for RAN4 spec due to the impossible case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criterion is not fulfilled”.

Proposal 2: Send response LS to RAN2 to inform the RAN4 conclusions.

4 References

- R4-2417507 (R2-2409308), LS on Relaxed measurement, LGE (RAN2), RAN4#113

Annex draft reply LS

3GPP TSG-RAN WG4 Meeting #113				                                               R4-24xxxxx

Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS on Relaxed measurement

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:	Qiuge Guo

E-mail Address: guoqiuge@CATT.CN

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN4 thanks RAN2 for the LS R4-2417507 (R2-2409308) on relaxed measurement. RAN4 discussed the issue and has the following conclusion:

- No updates are needed for RAN4 spec due to the impossible case “if UE measurements are better than s-NonIntraSearch threshold but the cell-edge criterion is not fulfilled”.

RAN4 kindly asks RAN2 to take the above information into account.

2. Actions:

To RAN2:

RAN4 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 #114			17 Feb. – 21 Feb. 2025	Athens, Greece

TSG-RAN4 #114bis 	      7 Apr. – 11 Apr. 2025	     China