3GPP RAN WG4 Meeting #113		R4-2417743

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.2.4.2

Source:	Apple

Title:		TP on UE RF design options for the 14800-15350MHz frequency range

WI/SI:	FS_NR_IMT_4400_7125_14800MHz

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The WRC-23 conference identified three frequency ranges as potential IMT candidates and tasked ITU-R WP5D to conduct the corresponding sharing and compatibility studies. In turn, the ITU-R WP5D sent the LS to a number of bodies, including 3GPP, asking to provide technical parameters for the candidate frequencies [1]. In response to that LS from WP5D and based on the technical input from RAN WG4 [2], the 3GPP RAN#103 meeting approved a new SI [3], purpose of which will be to study and provide the corresponding IMT technical parameters.

During the RAN4#111 meeting an initial discussion took place and RAN WG4 concluded that for this frequency range both FR1- and FR2-like approaches for the UE RF architecture will be studied further. Next, RAN4#112bis decided that FR1 omnidirectional UE RF design would be taken as a baseline for the co-existence studies, which however does not preclude more advanced RF options. Thus, in this document we present a text proposal to capture a UE RF design for the 15GHz based on the patch antennas.

# 2	Text proposal

-------------------------------------------- TP BEGIN --------------------------------------------

# 6	14800 - 15350 MHz frequency range

## 6.5	Antenna characteristics

### 6.5.1	BS antenna characteristics

#### 6.5.1.1		Antenna model

#### 6.5.1.2	Antenna parameters

### 6.5.2	UE antenna characteristics

#### 6.5.2.x	FR1-like design with patch antennas

The FR1-like approach with patch antennas gives more flexibility to the OEM vendors because even if a device implements more than one Tx, there is an additional flexibility on where Tx antennas can be placed. For instance, 6.5.2.x-1 presents a general mobile device form factor with two patch antennas placed within the device as presented at the distance corresponding to approximately three wavelengths.

Figure 6.5.2.x-1: Exemplary placement of two patch antennas within a mobile device.

Each patch antenna gain is around 2dB with the Tx power 23dBm. So, Figure 6.5.2.x-2 below presents separately distribution of antenna and EIRP gains for the considered antennas, where the baseline curve is only one antenna. Gain values are obtained by probing all points in 3D space around considered antennas, which allows us to find the best beam at each point of space independent of the base station location. In other words, these figures illustrate all possible gain values, including the best one, regardless of the phone orientation with respect to the base station. As can be seen, from the antenna gain perspective, the average gain comparing to one Tx is around 2.5dB. From the EIRP perspective, the average gain comparing to one Tx is around 5.5dB, which comprises 2.5dB antenna gain and 3dB coming from the fact that there are two Tx antennas each transmitting at 23dBm.

### Figure 6.5.2.x-2: Antenna and EIRP gain for the considered two Tx antennas.

Another aspect for the FR1-like antenna design is the resulting radiation pattern. It is worth noting that unlike at the antenna and EIRP gain, which are not very sensitive for the actual placement of Tx antennas, the radiation pattern may vary a lot. For instance, Figure 6.5.2.x-3 below presents the resulting beam pattern from two Tx antennas physical location of which is the same as in Figure 6.5.2.x-1. For the sake of better clarity, we present resulting the resulting radiation pattern in 2D and 3D spaces. Since two Tx antennas are placed at the back side of the phone, the resulting radiation pattern has a big negative gain in the opposite direction.

Figure 6.5.2.x-3: Radiation pattern for two Tx antennas presented in Figure 2.1-1.

-------------------------------------------- TP END --------------------------------------------

# 3	References

RP-240024, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz", ITU-R WP5D

R4-2102840, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)", RAN WG4.

RP-240765, "Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz"

3GPP TR 38.921, "Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz", v17.1.0