3GPP RAN WG4 Meeting #113		R4-2417764 R4-2417763

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	7.1.1.1.3

Source:	Apple

Title:	On Rel-19 HPUE for inter-band UL CA/DC

WI/SI:	NR_ENDC_RF_Ph4-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The Rel-19 RAN4 non-spectrum WID/SID package has been approved in RAN #103 meeting. In consideration that the uplink coverage is crucial for 5G and 5G-adavanced roll-out and the continued strong interests from operators in higher UE maximum output power such as PC1.5 for their band combinations, one of the key objectives under the new Rel-19 “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” work item [1] is to specify the generic requirements of HPUE for NR uplink CA in FR1 and EN-DC with NR FR1 bands where the features include,

    - PC1.5 for intra-band contiguous UL CA with and without UL MIMO

    - PC1.5 Intra-band non-contiguous UL CA without UL MIMO

    - PC1.5 2-band inter-band UL CA with 2Tx or 3Tx for handheld UE and FWA

    - PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx for handheld UE and FWA UE

    - Increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band UL CA and EN-DC HPUE

In last RAN4 meeting, companies had provided their further views on the respective features above where the additional agreements were captured in the approved “WF on HPUE RF requirements” [2]. For HPUE inter-band UL CA/EN-DC, there has been no new development in the past two RAN4 meetings. In our view, part of the reasons is due to that there was no example band combination proposed, and another reason could be that the potential new general requirement for PC2 or PC1.5 inter-band UL combinations involving FDD bands is tied to the ongoing discussions on the Rel-19 HPUE MSD rules which may need to be concluded first before the decision on whether new MSD framework is needed or not for the new UL combination feature.

In this contribution, we propose to prioritize PC1.5 inter-band UL CA combinations composed by (PC2 TDD + PC2 TDD) with 2Tx or 3Tx and postpone the development of PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band till the conclusion of Rel-19 HPUE MSD rules in order to make progress on the objective for PC1.5 inter-band UL combinations.

# Discussion

PC1.5 inter-band UL combinations have been introduced in Rel-18 which however is only limited to 3Tx and targeted for FWA, though in principle PC1.5 inter-band combinations with 2Tx should have been introduced earlier than 3Tx as it is considered as a less advanced UE feature.

While PC1.5 inter-band UL combinations with 2Tx is included in the Rel-19 objective for PC1.5 inter-band UL combinations, there has been no progress made on this configuration after four RAN4 meetings. In our view, part of the reasons is due to that there was no example band combination proposed, and another reason could be that the potential new general requirement for PC2 or PC1.5 inter-band UL combinations involving FDD bands is tied to the ongoing discussions on the Rel-19 HPUE MSD rules which may need to be concluded first before the decision on whether new MSD framework is needed or not for the new UL combination feature.

To avoid further delay on the development of PC1.5 inter-band UL combinations, we propose to prioritize inter-band UL CA combinations composed by (PC2 TDD + PC2 TDD) with 2Tx or 3Tx as such configurations are free from MSD impact and would not be subject to the decision of Rel-19 HPUE MSD rules. On the other hand, the introduction of PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band in either or both UL bands which are subject to MSD impact may need to wait till the conclusion of Rel-19 HPUE MSD rules.

Observation 1: PC1.5 inter-band UL combinations have been introduced in Rel-18 which is only limited to 3Tx and targeted for FWA

Observation 2: There has been no progress made on PC1.5 inter-band UL combinations with 2Tx after four RAN4 meetings.

Observation 3: PC1.5 inter-band UL combinations composed by (PC2 TDD + PC2 TDD) are free from MSD impact and would not be subject to the decision of Rel-19 HPUE MSD rules.

Observation 4: PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band in either or both UL bands are subject to MSD impact which may need to wait till the conclusion of Rel-19 HPUE MSD rules.

Proposal: RAN4 to prioritize PC1.5 inter-band UL CA combinations composed by (PC2 TDD + PC2 TDD) with 2Tx or 3Tx and postpone the development of PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band till the conclusion of Rel-19 HPUE MSD rules.

# 3	Conclusion

In this contribution, we propose to prioritize PC1.5 inter-band UL CA combinations composed by (PC2 TDD + PC2 TDD) with 2Tx or 3Tx and postpone the development of PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band till the conclusion of Rel-19 HPUE MSD rules in order to make progress on the objective for PC1.5 inter-band UL combinations.

Observation 1: PC1.5 inter-band UL combinations have been introduced in Rel-18 which is only limited to 3Tx and targeted for FWA

Observation 2: There has been no progress made on PC1.5 inter-band UL combinations with 2Tx after four RAN4 meetings.

Observation 3: PC1.5 inter-band UL combinations composed by (PC2 TDD + PC2 TDD) are free from MSD impact and would not be subject to the decision of Rel-19 HPUE MSD rules.

Observation 4: PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band in either or both UL bands are subject to MSD impact which may need to wait till the conclusion of Rel-19 HPUE MSD rules.

Proposal: RAN4 to prioritize PC1.5 inter-band UL CA combinations composed by (PC2 TDD + PC2 TDD) with 2Tx or 3Tx and postpone the development of PC2 and PC1.5 inter-band UL combinations consisting of PC2 FDD band till the conclusion of Rel-19 HPUE MSD rules.

# 4	References

RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 Chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18th – 21st, 2024

R4-2417098 “WF on HPUE RF requirements”, Samsung, 3GPP TSG RAN WG4 Meeting #112bis, Hefei, China, October 14th – 18th, 2024