3GPP TSG-RAN WG4 Meeting #112-bis	                               R4-2417783

Orlando, USA, November 18 – November 22, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on MSD for simultaneous Rx-Tx of CA_n40-n41 based on the additional test point

Agenda item:	6. 11. 2

Document for:	Discussion

Introduction

In the last RAN4#112-bis meeting, the remaining issues on the simultaneous Rx-Tx in Rel-19 was discussed and the WF [1] was approved.

In this contribution we discuss MSD for simultaneous Rx-Tx of CA_n40-n41 based on the new test point and provide our view.

Discussion

Below is the way forward from [1]. The agreement is to generate a new additional test point for simultaneous Rx-Tx of CA_n40-n41 based on operators’ input and propose MSD values. We provided our view why the new additional test point is needed in [2].

<Way forward>

- Companies are asked to verify whether the same MSD value defined for CA_n40-n41 with 4Rx can be applied to 2Rx RF front end architecture for CA_n40-n41.

- Provide further information on the need for new test point for CA_n40-n41, taking into consideration the applicable bandwidth and frequency range of n40 and n41 from operator(s). For example, wider frequency separation between n40 and n41, and wider DL bandwidth.

- Companies are asked to propose suitable test point reflecting operator demand in next meeting

- 2Rx RF front end architecture can be considered.

- For n41, consider if a new signalling is necessary to support maximum of 2-layer MIMO with 2Rx antenna ports, if the UE reports both the capability of 4-layer MIMO for band n41 and simultaneous Rx-Tx for CA_n40A-n41A.

After RAN4#112-bis meeting, one proponent provided one test point so we will discuss the MSD for this test point first.

After that we will propose another MSD test point which has more reasonable MSD requirement.

MSD using proponent’s proposed test point

Table 1 shows the test point proposed by one proponent. Both UL BW and DL BW are 100MHz for n40 and n41. Figure 1 and Figure 2 show the frequency landscape for crossband isolation between n40 and n41 based on the test point in Table 1.

Table 1 Additional MSD test point for simultaneous Rx-Tx CA_n40A-n41A proposed by one proponent

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2565 | 100 | [TBD] | ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | [TBD] | ACLR2 |

Figure 1. Landscape when n40 is aggressor (100MHz BW) and n41 is victim (100MHz BW)

Figure 2. Landscape when n41 is aggressor (100MHz BW) and n40 is victim (100MHz BW)

It should be noted that most of Rx band noise is still under the ACLR2 region which is same as the existing MSD test point. So, we can expect huge MSD similar to the existing MSD requirement.

Table 2 and Table 3 show the MSD using this test point based on our evaluation result for PC3 and PC2 respectively.

Table 2 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A proposed by one proponent’s test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2565 | 100 | 25.4 | ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 28.8 | ACLR2 |

Table 3 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A proposed one proponent’s test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2565 | 100 | 28.4 | ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 31.8 | ACLR2 |

In conclusion, the MSD is lowered about 2.6~2.7dB but still huge.

Observation 1: MSD using the test point in Table 1 is still huge.

MSD using Murata’s proposed test point

We propose another MSD test point as Table 4, Figure 2 and Figure 3. The n40 channel bandwidth is 50MHz so the Rx band noise for n41 DL is under >ACLR2 range which means that the lower MSD for n41 DL can be expected.

Table 4 Additional MSD test point for simultaneous Rx-Tx CA_n40A-n41A proposed by Murata

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [TBD] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [TBD] | ACLR2 |

Figure 3. Landscape when n40 is aggressor (50MHz BW) and n41 is victim (100MHz BW)

Figure 4. Landscape when n41 is aggressor (100MHz BW) and n40 is victim (50MHz BW)

Table 5 and Table 6 show the MSD using the test point in Table 4 based on our evaluation result for PC3 and PC2 respectively. It should be noted that the MSD for n40 DL is still huge but the MSD for n41 DL is improved by over 16dB.

Table 5 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A proposed by Murata test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 11.8 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 28.6 | ACLR2 |

Table 6 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A proposed by Murata test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 14.4 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 31.6 | ACLR2 |

Observation 2: MSD using the test point in Table 2 is still huge for n40 DL but MSD for n41 DL is reasonable.

Proposal 1: Consider Table 5 and Table 6 for the additional MSD test point for simultaneous Rx-Tx CA_n40-n41

Conclusion

In this contribution, we discussed the remaining issue regarding the simultaneous Rx-Tx of CA_n40-n41 and provided our view as below.

Observation 1: MSD using the test point in Table 1 is still huge.

Observation 2: MSD using the test point in Table 2 is still huge for n40 DL but MSD for n41 DL is reasonable.

Proposal 1: Consider Table 5 and Table 6 for the additional MSD test point for simultaneous Rx-Tx CA_n40-n41

Table 5 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A proposed by Murata test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 11.8 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 28.6 | ACLR2 |

Table 6 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A proposed by Murata test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 14.4 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 31.6 | ACLR2 |

References

[1]	R4-2417058, “WF on simultaneous Rx-Tx for CA_n40-n41”, Huawei, Hisilicon, 3GPP TSG RAN WG4 Meeting #112-bis, Hefei, China, October 14th – 18th, 2024

[2]	R4-2415359, “Further discussion on simultaneous Rx-Tx of CA_n40-n41”, Murata Manufacturing Co Ltd., 3GPP TSG RAN WG4 Meeting #112-bis, Hefei, China, October 14th – 18th, 2024