3GPP TSG-RAN WG4 Meeting #113                                                                                             R4-2417816

Orlando, US, Nov. 18 – Nov. 22, 2024

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	7.26.2.1

Document for:	Approval

- Introduction

In RAN4 #112-bis meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

## Issue 1-1: PC3 output power for HD-FDD (e)RedCap Agreement: Nominal power for PC3 is 23 dBm. ## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: Agree n256 Refsens as below. Operating Band; SCS (kHz); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). 15; -100.0; -96.8; -95.0; -93.8. n256; 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. ## Issue 1-4: Specification updates Agreement: - The maximum transmission bandwidth for NTN eRedCap with bandwidth reduction should be 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths. - REFSENS of NTN eRedCap with bandwidth reduction for wider channel BW in FR1-NTN bands, it should be defined as below: - The 5MHz REFSENS requirements are applicable for all the channel bandwidths including 5MHz, 10MHz, 15MHz, 20MHz channel bandwidth. - 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FD-FDD, where both UL and DL allocations are following the 5MHz NTN RedCap REFSENS configurations and located at the center of the channel BW. - UL RB size follow the 5MHz UL configuration table - Reuse the same scaling factor of NTN (e)RedCap without bandwidth reduction for REFSENS for NTN eRedCap with bandwidth reduction with 2Rx HD-FDD, 1Rx HD-FDD and 1Rx FD-FDD. - The bandwidth reduction in TS 38.101-1 5.3I applies also for NTN (e)RedCap ## Issue 1-3: Simultaneous operation with GNSS Open items: - Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN - Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies - Encourage further evaluations in RAN4 including but not limited to - Whether potential new solution(s) apply only from rel-19 or are early implementable - Whether UE speed needs to be considered

In the previous meeting, the 23 dBm nominal Tx power for PC3 HD-FDD RedCap UE was agreed. However, the test tolerance is still under discussion. The test tolerance takes into account manufacturing tolerances, temperature and other factors that may have an effect on the transmitter power. A margin of 2 dB already introduces a large fluctuation in the transmitter power of the UE, and this margin is mainly defined for the conformance test procedure of the UE.

In fact, the maximum transmission power of PC3 is already close to PC2 (after considering tolerance), which means that further relaxation may cause the NTN (e)RedCap UE to encounter the problems faced by PC2. For instance, additional SAR limit would require the introduction of additional signalling, and higher Tx power may cause some coexistence issues. Therefore, we believe that the test tolerance value does not need any additional modification.

Proposal 1: The 2 dB test tolerance for PC3 could be considered for non-RedCap HD-FDD NTN UE and (e)RedCap HD-FDD NTN UE.

A preliminary discussion on whether NTN RedCap UE can operate simultaneously with GNSS function took place at the previous meeting. In fact, the issue of simultaneous operation with GNSS was raised and discussed during the BL/CE/NB-IoT NTN topic and the following conclusions were made and captured in TS 36.331:

### 5.5.9 GNSS measurement triggering and reporting For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods. The UE shall: 1> if an indication to perform GNSS measurement is received from lower layers: 2> perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23]; 2> stop timer T390, if running; 1> if gnss-AutonomousEnabled is configured: 2> if the gap length is indicated by lower layers: 3> set the autonomous gap length to the gap length indicated by lower layers; 2> else: 3> set the autonomous gap length to the latest reported time duration required for the UE to acquire a GNSS position; 2> perform GNSS measurement using the autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry; NOTE: UE can autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid. The exact time of starting GNSS measurements during available idle periods is left to UE implementation. 1> upon starting GNSS measurement: 2> stop timer T318, if running; 1> upon indication that a new GNSS position becomes valid: 2> instruct lower layers to report the remaining GNSS validity duration (see TS 36.321 [6]); 1> upon indication that GNSS measurement has failed: 2> if GNSS position is out-of-date; and 2> if ul-TransmissionExtensionEnabled is not configured or T390 is not running: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

Meanwhile, the valid duration of GNSS information was captured in TS 36.331 as well:

#### – GNSS-ValidityDuration The IE GNSS-ValidityDuration indicates the remaining GNSS validity duration in the UE. Value s10 corresponds to 10 seconds, s20 corresponds to 20 seconds and so on. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on. GNSS-ValidityDuration information element -- ASN1START GNSS-ValidityDuration-r17 ::= ENUMERATED{ s10, s20, s30, s40, s50, s60, min5, min10, min15, min20, min25, min30, min50, min90, min120, infinity}

For LTE NTN UE, as it mainly serves the IoT scenarios, the limited hardware capability and cost result in the inability of simultaneous operation with GNSS module. Therefore, the design of the LTE NTN UE focuses on two main aspects: the introduction of aperiodic measurements of the GNSS information to ensure the accuracy of the position information and ephemeris information; and the introduction of the valid duration of the GNSS information to ensure that the validity of GNSS information is not aged too much. Since the GNSS measurement performed by using the measurement gap, which requires that the GNSS information must be correctly acquired during RRC_CONNECTED state, as the measurement gap is configured in the RRC_CONNECTED state.

Observation 1: LTE NTN UE is NOT considered to be able to operate simultaneously with GNSS module, therefore an aperiodic GNSS measurement gap was introduced for LTE NTN scenario. The GNSS information obtained from GNSS measurement has a valid duration.

For NR NTN UE, this problem no longer exists. Due to the capability improvement of the NR UE, the UE was considered that could operate simultaneously with GNSS module by default. The GNSS module can unconditionally provide GNSS information to the UE, eliminating the need for a dedicated measurement gap. This is also in accordance with the description in TS 38.331, where the descriptions related to the GNSS measurement gap and the GNSS valid duration has been completely removed.

Observation 2: NR NTN UE is considered to be able to operate simultaneously with GNSS module and the GNSS module can unconditionally provide GNSS information to the UE. Therefore the aperiodic GNSS measurement gap and GNSS information valid duration was removed from NR specification.

For NR NTN RedCap UEs, the requirements that are affected compared to NTN normal UE are channel bandwidth and REFSENS, which means that their hardware capabilities are similar to NTN normal UE. Therefore, we believe that NTN RedCap UE can also operate with GNSS without additional impact on the existing spec.

Proposal 2: There is no impact to RAN4 specs for simultaneous operation of RedCap/eRedCap NTN UEs with GNSS.

Considering the above discussion, since the GNSS module could provide GNSS information to NR NTN UE unconditionally, we believed that NTN RedCap UE can correctly obtain the GNSS information and correctly perform pre-compensation. Therefore, the frequency error requirements of NR RedCap UE could be reused for NTN RedCap UE.

Proposal 3: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

- Conclusion

This contribution provides our views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Proposal 1: The 2 dB test tolerance for PC3 could be considered for non-RedCap HD-FDD NTN UE and (e)RedCap HD-FDD NTN UE.

Observation 1: LTE NTN UE is NOT considered to be able to operate simultaneously with GNSS module, therefore an aperiodic GNSS measurement gap was introduced for LTE NTN scenario. The GNSS information obtained from GNSS measurement has a valid duration.

Observation 2: NR NTN UE is considered to be able to operate simultaneously with GNSS module and the GNSS module can unconditionally provide GNSS information to the UE. Therefore the aperiodic GNSS measurement gap and GNSS information valid duration was removed from NR specification.

Proposal 2: There is no impact to RAN4 specs for simultaneous operation of RedCap/eRedCap NTN UEs with GNSS.

Proposal 3: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

- References

- R4-2416586, Way Forward for [112-bis][310] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4# 112-bis.