3GPP TSG-RAN WG4 Meeting #113                                                                                            R4-2417844

Orlando, US, 18th – 22nd November, 2024

Source: 	CATT

Title: 	Discussion on AIoT device RF requirements

Agenda item:	7.21.3.2

Document for:	Approval

- Introduction

In last RAN4 meeting, some progress was made for AIoT device RF requirements [1]. Many TBDs still exist in the WF. This contribution provides our further analysis and views for the open issues.

- Discussion

  - Tx requirements

For Tx requirements, the followings are not decided.

The open issues for

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD | TBD | TBD |
|  | In band emissions | NO, as RAN4#112 agreed | TBD | TBD |
|  | Carrier leakage | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | SEM | TBD | TBD | TBD |
|  | ACLR | TBD | TBD | TBD |
|  | Spurious emissions | TBD | TBD | TBD |
| Unwanted emissions |  | TBD | TBD | TBD |
| Transmit intermodulation |  | NO | NO | TBD |

For the requirements for passive devices, as radiated test is the baseline, limiting the number of requirements brings benefit to the ecosystem considering the test cost. On the other hand, the transmitted signal for passive devices is generated by reflection through impedance matching, the very simple implementation make the performance also not so various and relatively easy to be under control. So we still think very limited requirements should be defined for passive devices. In ETSI EN 302 208 [3], only two requirements are defined for tag, i.e. radiated power and the unwanted emission.

Proposal 1: Limit the number of the RF requirements for passive devices, including device 1 and device 2a.

For the specific requirements, they can be discussed one by one. For the frequency error for device 2a, the frequency shift support for device 2a is not concluded yet in RAN1, so it’s better to delay the decision for this requirement. For modulation quality, we’re not very sure if passive devices need to have this requirement, we still prefer to do more analysis both from core and test method/effort aspects. The modulation quality requirement for device 2b may be defined considering device 2b can be more powerful and requirements can be conducted. However, the device architecture is not confirmed in RAN4, the decision should be revisited in WI phase. For in-band emission for both device 2a and device 2b, it’s not clear if there’s any clear decision for D2R FDMA transmission, even there’s FDMA decision, if it’s FDM based on channel not RB, the requirement is different with in-band emission. So we think it should be FFS. For carrier leakage for device 2a, we don’t think it’s needed because the carrier is from outside not generated by the device. For output RF spectrum emissions or unwanted emission, we think only one of them is needed. For device 1 and device 2a, the requirement can be simple, similar with the unwanted emission requirements in ETSI EN 302 208 [3]. The following two figures are copied from ETSI EN 302 208. The unwanted emission requirements are also called spectrum mask in the figures.

Whether the unwanted emission requirements are relative power rejection or absolute requirement, it can be discussed further in WI phase. For device 2b, if unwanted emission requirements would include three requirements, i.e. SEM, ACLR and spurious emission requirements can be FFS. Our preliminary thinking is that it may not be needed, the similar mask can be defined for device 2b. However, there’s no co-existence simulation for device 2b in SI phase, if companies want to further check in WI phase, we’re also ok. For transmitter intermodulation for device 2b, if NB-IoT UE requirement is taken as the reference, it seems it can be defined. However, if no requirement for device 1 and device 2a works, we’re not very sure why the requirement should be defined for device 2b. For the deployment scenarios difference for passive devices and active devices, there’s no detail discussion. From RAN1 R2D signal FDM discussion, it seems the deployment for passive devices and active devices may be separated. If device 2b has more opportunity to be deployed in the same area with NR and LTE UEs, Tx IMD requirement may be needed. However, due to the very limited time in SI, we would prefer to keep this requirement FFS to be further discussed in WI phase.

According to the above analysis, Table 2 summarized our view for the open issues for Tx requirements.

Table2: CATT views summary on open issues for device Tx requirements

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit signal quality | Frequency error | - | FFS | - |
|  | Modulation quality requirements | FFS | FFS | [Yes] |
|  | In band emissions | - | FFS | FFS |
|  | Carrier leakage | - | No | - |
| Output RF spectrum emissions | SEM | No | No | No |
|  | ACLR | No | No | No |
|  | Spurious emissions | No | No | No |
| Unwanted emissions |  | Yes | Yes | Yes If the requirements are separated to SEM, ACLR and spurious requirement FFS. |
| Transmit intermodulation |  | - | - | FFS |

  - Rx requirements

For the Rx requirements, we have the same view in our contribution [2] in last meeting, no need to define many Rx requirements for passive requirements. For maximum input power for device 1 and device 2a, our understanding is that the Tx radiated power requirement will use the maximum input power to check the reflected maximum output power, so no need to define maximum input power for device 1 and device 2a.

Proposal 2: No need to define maximum input power for device 1 and device 2a.

For the interference rejection requirement, it seems it’s related to the deployment scenario and it’s a totally new concept, the details may need more discussion to make the final decision. So we would prefer to further study it in WI phase.

Proposal 3: FFS the interference rejection requirement in WI phase.

For the whole set of Rx requirements for device 2b, we have the similar view with Tx side that it’s hard to decide them in current stage because the deployment scenarios are not clear, the system design is not finished, there’s no co-existence study. So we think even some decision is made in SI phase, they should be able to be revisited when more technical aspects are clear in WI phase.

Proposal 4: All of the decisions on device 2b can be revisited in WI phase when more technical aspects are clear.

- Conclusion

This contribution provides our further analysis for the remaining issues for AIoT device requirements. We have the following observations and proposals,

Proposal 1: Limit the number of the RF requirements for passive devices, including device 1 and device 2a.

Table 2 summarized our view for the open issues for Tx requirements.

Table2: CATT views summary on open issues for device Tx requirements

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit signal quality | Frequency error | - | FFS | - |
|  | Modulation quality requirements | FFS | FFS | [Yes] |
|  | In band emissions | - | FFS | FFS |
|  | Carrier leakage | - | No | - |
| Output RF spectrum emissions | SEM | No | No | No |
|  | ACLR | No | No | No |
|  | Spurious emissions | No | No | No |
| Unwanted emissions |  | Yes | Yes | Yes If the requirements are separated to SEM, ACLR and spurious requirement FFS. |
| Transmit intermodulation |  | - | - | FFS |

For Rx requirements, we have the following proposals,

Proposal 2: No need to define maximum input power for device 1 and device 2a.

Proposal 3: FFS the interference rejection requirement in WI phase.

Proposal 4: All of the decisions on device 2b can be revisited in WI phase when more technical aspects are clear.

- References

[1] R4-2414305, “WF on RF requirements for A-IoT”, RAN4#112, Huawei

[2] R4-2415099, “Discussion on AIoT device RF requirement, RAN4#112b, CATT

[3] ETSI EN 302 208