3GPP TSG-RAN WG4 Meeting # 113                                                               R4-2417852

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on modification of existing Tx requirements for SBFD operation

Source: 	CATT

Agenda item:	7.20.2.2

Document for:	Discussion

- Background

WF [1] of RAN4#112bis concluded that further studies are needed for some TX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Tx RF Requirement

  - ACLR and OBUE

In WF [1], WF of ACLR and OBUE are shown following.

WF:

- FFS on the followings:

  - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly?

    - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence.

    - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement.

  - How to define the additional co-existence requirements:

    - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level.

      - [1dB DESENS in the victim SBFD receiver]

      - [typical CL values based on typical SBFD-capable gNB implementation]

      - [frequency offset for the additional OBUE requirement is declared]

    - Option 2: by improving ACLR1 and ACLR2 requirements

    - Option 3: others

We agree with 1dB degradation of victim BS sensitivity.

For FR1 aggressor WA BS, the output power is 49dBm, bandwidth of channel is 100MHz, if assume the degradation of victim BS sensitivity is 1dB, the required total isolation between the two BSs is 144dB, that is, the sum of ACLR, couple loss of co-location and the rejection of RF BPF used after PA must larger than 144dB.

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim BS sensitivity(dB) | [1] |
| Required total isolation(dB) | 144 |
| 1st ACLR(dB) | [45] |
| Coupling loss of co-location (dB) | [30] |
| Rejection of RF BPF used after PA (dB) | 69 |

If ACLR is 45dB, coupling loss of co-location is 30dB, rejection from RF BPF at victim BS UL sub-band must be larger than 69dB.

When two SBFD-capable BSs are co-location and the operating frequency bands of the two BSs are adjacent, the interference from BS to BS of four different BS spectrum configurations is listed in table 2.

Table 2 Mutual interference between two SBFD-capable BSs of four cases

| Case | Configuration of SBFD capable BSs | SBFD BS1 DL to SBFD BS2 UL |
| --- | --- | --- |
| 1 | SBFD BS 1: {DUD} SBFD BS 2: {DUD} |  |  |
| 2 | SBFD BS 1: {UD} SBFD BS 2: {DU} |  |  |
| 3 | SBFD BS 1: {DU} SBFD BS 2: {UD} |  |  |
| 4 | SBFD BS 1: {UD} SBFD BS 2: {UD} |  |  |

From table 2, it can be seen that case 2 is the best configuration. When two SBFD-capable BSs with adjacent operating bands configured as {DU}, and two DL sub-bands are adjacent, frequency offset between aggressor channel to victim channel is max, and this configuration can optimize the rejection at victim BS UL sub-band by RF BPF used after PA. The best configuration is not a general requirement, but can be used for some scenario.

If channel bandwidth is 100MHz, the DL sub-band widths of two BSs are both 80MHz and UL sub-band width of two BSs are both 20MHz, when the two DL sub-bands are adjacent, victim BS UL sub-band is 80MHz offset to the edge of aggressor BS passband, the BPF after PA can provide 70dB suppression.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Observation 1: For two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration.

Observation 2: The best configuration is not a general requirement, but can be used for some scenario.

Observation 3: For WA BS, if victim BS DESENS is 1dB and CL for co-location BS is 30 dB, RF BPF rejection performance should be larger than 69dB.

Observation 4: When channel bandwidth is 100MHz, the RF BPF after PA can provide 70dB suppression outside 80MHz offset to the edge of passband.

Proposal 2: For two SBFD-capable BSs are co-location and the operating channels of two BSs are adjacent, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration can be used for some scenario.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: We agree with 1dB degradation of victim BS sensitivity.

Observation 1: For two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration.

Observation 2: The best configuration is not a general requirement, but can be used for some scenario.

Observation 3: For WA BS, if victim BS DESENS is 1dB and CL for co-location BS is 30 dB, RF BPF rejection performance should be larger than 69dB.

Observation 4: When channel bandwidth is 100MHz, the RF BPF after PA can provide 70dB suppression outside 80MHz offset to the edge of passband.

Proposal 2: For two SBFD-capable BSs are co-location and the operating channels of two BSs are adjacent, the best configuration is both BSs configured as {DU}, and two DL sub-bands are also adjacent, and this configuration can be used for some scenario.

- Reference

[1] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, HiSilicon, RAN4#112bis

[2] R4-2415108, “Discussion on modification of existing Tx requirements for SBFD operation”, CATT, RAN4#112bis