3GPP TSG RAN WG4 Meeting #113		R4-2417860

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	Further discussion on BS RF requirements for LP-WUS

Agenda Item:	7.23.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, RAN4 continued discussion on BS RF requirements related topics, and made some progress accordingly [1]. However, many issues are still open, and some of them are pending on RAN1 progress. In this contribution, we will discuss further on the following open issues:

#### Issue 1-2: Concept of LP-WUS power boosting - Proposals - Proposal 1: Since EPRE ratio has the merit that power boosting declaration for LP-WUS does not vary with different CBWs, RAN4 to introduce and specify power boosting represented by EPRE ratio between LP-WUS and NR REs excluding LP-WUS. (CATT, Huawei, [vivo]) - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs). - Proposal 2: Adopt following definition rather than introduce EPRE ratio. (Ericsson, ZTE, Nokia) - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - WF - Compare the Pros and Cons of the two definitions - At least the following aspects should be considered - Convenience for manufacturer’s declaration - Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc. - Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided - Comparison of power boosting with NB-IoT with the same definition should be provided - Test aspects of power boosting should be considered - Other aspects are not precluded - Try to define the power boosting concept next RAN4 meeting with one definition as baseline but allow flexibility to manufacturer declaration of boosting level for the supported CBW #### Issue 1-3: On minimum power boosting level for LP-WUS - Proposals - Need to have it. - If power boosting will be defined as EPRE ratio. - Proposal 1: Use 3dB as minimum requirement. (CATT, Huawei, [vivo]) - If power boosting will not be defined as EPRE ratio. - Proposal 2: Use 3dB as minimum requirement. (Ericsson, [vivo]) - Proposal 3: The core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0dB. (Nokia) - Proposal 4: No matter how to define power boosting, the requirement should consider the assumption in RAN1 of 3dB or 6dB means the EPRE ratio. The requirement can be different for different CBW. (vivo) - Proposal 5: No need to have it. (CMCC, ZTE, Nokia) #### Issue 1-4: Whether a cap for LP-WUS power boosting should be considered - Proposals - Yes, because impact on NR signal available transmission power should be considered. - Proposal 1: 3dB. (Ericsson) - Proposal 1: 6dB. (Huawei) - Proposal 2: Not to introduce maximum value for LP-WUS power boosting. (ZTE, Nokia, CATT, [vivo]) - WF - [Leave the maximum power dynamic range to manufacturer’s declaration] #### Issue 1-5: Whether to preclude small CBW for consideration of LP-WUS power boosting - Proposals - Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting. (Ericsson) - Proposal 1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting. (CATT) - Proposal 2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded. (Huawei) - Proposal 3: For 30kHz SCS LP-WUS operation, power boosting within 5MHz and 10MHz CBW can be precluded. For 15kHz SCS LP-WUS operation, no CBW should be precluded. (vivo) - No need to preclude any CBW. (CMCC, Nokia) - Proposal 4: Only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement. (Nokia) - WF - Consider exclusion of small CBW together with LP-WUS power boosting definition, manufacturer’s declaration as a package for further discussion #### Issue 1-6: On applicable BS type for LP-WUS feature - Proposals - Proposal 1: Not to set restriction on applicable BS types to support LP-WUS. (CMCC, CATT, Nokia) - Proposal 2: BS type 1-C, 1-H and 1-O are all applicable BS types for LP-WUS. (Ericsson) - Proposal 3: Use BS Type 1-C as applicable BS type as a start point. (ZTE) - Proposal 4: BS types 1-C, 1-H and 1-O are more like the applicable requirements to a certain BS as the same BS can support both type 1-C and type 1-H requirements, which is up to the manufacturer’s declaration. Focusing on the applicable requirements rather than the BS types (Huawei) - WF - Non-AAS, AAS could be considered for LP-WUS feature #### Issue 1-7: Whether to define the LP-WUS requirements only for supported BS types - Proposals - Proposal 1: RAN4 to only define conductive based BS RF requirements for LP-WUS in Release-19. (CATT) - Proposal 2: Only conductive based BS type 1-C and 1-H requirements are considered for LP-WUS. (Huawei) - WF - FFS.

In addition, with RAN1 agreeing that the number of PRBs for LP-WUS is 11, we also update our numerical results accordingly in this contribution compared contributions submitted in previous meetings.

# 2 Discussion

Issue 1-2: Concept of LP-WUS power boosting

RAN1 has agreed 11 as the number of PRBs for LP-WUS in Hefei, while the tables in [3] are calculated based on 25 RBs for LP-WUS, hence, we update the corresponding tables as below for SCS = 15 kHz:

Table – 1 Power dynamic range for LP-WUS (Updated with 11 RBs for LP-WUS from 25 RBs)

| EPRE ratio (dB) | CBW 3MHz | CBW 5MHz | CBW 10MHz | CBW 15MHz | CBW 20MHz | CBW 25MHz | CBW 30MHz | CBW 35MHz | CBW 40MHz | CBW 45MHz | CBW 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +3dB | 0.62 | 1.43 | 2.18 | 2.44 | 2.58 | 2.66 | 2.72 | 2.76 | 2.79 | 2.82 | 2.84 |
| +6dB | 0.97 | 2.37 | 3.89 | 4.50 | 4.84 | 5.06 | 5.21 | 5.32 | 5.40 | 5.47 | 5.52 |
| +9dB | 1.09 | 2.73 | 4.65 | 5.49 | 5.97 | 6.28 | 6.50 | 6.67 | 6.80 | 6.89 | 6.98 |

Table – 2 EPRE ratio for LP-WUS (Updated with 11 RBs for LP-WUS from 25 RBs)

| Power Dynamic Range (dB) | CBW 3MHz | CBW 5MHz | CBW 10MHz | CBW 15MHz | CBW 20MHz | CBW 25MHz | CBW 30MHz | CBW 35MHz | CBW 40MHz | CBW 45MHz | CBW 50MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +3dB | N/A | 9.7 | 4.37 | 3.78 | 3.54 | 3.42 | 3.34 | 3.29 | 3.25 | 3.22 | 3.20 |
| +6dB | N/A | N/A | 13.12 | 8.91 | 7.87 | 7.39 | 7.11 | 6.92 | 6.78 | 6.69 | 6.61 |
| +9dB | N/A | N/A | N/A | 14.97 | 11.54 | 10.38 | 9.78 | 9.40 | 9.14 | 8.96 | 8.82 |

It can be observed that there are still not applicable configurations for a power dynamic range value from the above table with the updated number of LP-WUS RBs as 11.

Observation 1: There are still not applicable configurations for a power dynamic range value with the number of LP-WUS RBs updated from 25 to 11.

During the discussion in the previous meetings, one concern was raised on the EPRE ratio for the case where LP-WUS occupies all RBs and NR has 0 RBs (i.e., channel bandwidth = 5MHz, SCS = 30 kHz). In this case, EPRE ratio between LP-WUS RE and NR RE is not applicable. In our view, there is no power boosting operation in this case since power boosting does not change the total maximum transmit power, hence, it can be excluded from definition of power boosting.

Observation 2: Since power boosting does not change the total maximum transmit power of BS, there is no power boosting for the case where all RBs are used by LP-WUS and 0 RBs for NR, i.e., SCS 30kHz and CBW 5MHz.

If looking at the calculations in the latter part of this contribution when considering 2dB power degradation as illustrated in Fig. 1(b), the minimum allowed channel bandwidth for power boosting with SCS 30kHz is 10MHz, which automatically preclude the configuration of 5MHz CBW and SCS 30kHz.

Observation 3: 2dB NR power degradation already precludes the configuration from power boosting where LP-WUS occupies all REs within the NR carrier, i.e., SCS 30kHz and CBW 5MHz.

To account for such considerations, we can update the definition for power boosting in terms of EPRE ratio as following:

The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs) when both LP-WUS and NR are transmitted at the same time in the NR carrier.

Proposal 1: RAN4 to define LP-WUS power boosting in terms of EPRE ratio as:

The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs) when both LP-WUS and NR are transmitted at the same time in the NR carrier.

Issue 1-5: Whether to preclude small CBW for consideration of LP-WUS power boosting

It is possible to preclude some small CBW for LP-WUS power boosting, however, the small channel bandwidth threshold may depend on EPRE ratio (or power boosting level) and the allowed NR power degradation.

The power degradation ∆ could be calculated as the following equation where NRB is the number of RBs in the NR carrier, and NLP_RB is the number of LP-WUS RBs [2]:

∆=1+(α-1)NLP_RBNRB                                       (1)

The below figures show NR power degradation Vs EPRE ratio and channel bandwidth for SCS 15kHz and SCS 30kHz respectively. For example, if allowed NR power degradation is 2dB, for SCS 15kHz, channel bandwidth no less than 5MHz is allowed for power boosting with 3dB boosting, 15MHz with 6dB power boosting, and 20MHz with 9dB power boosting, while under the same allowed NR power degradation level, it is 10MHz, 25MHz and 40MHz for power boosting level 3dB, 6dB and 9dB respectively.

For different SCSs, for the same power boosting level and allowed NR power degradation, the small channel bandwidth threshold for power boosting operation is different, hence the configuration for an allowed NR power degradation due to power boosting should be defined by the combination of SCS, power boosting level and minimum allowed channel bandwidth, as shown in the below table.

Proposal 2: channel bandwidths for power boosting operation are indicated by the combinations consisting of allowed NR power degradation, SCS, power boosting level, minimum allowed channel bandwidth as shown in below table.

| Allowed NR power degradation (dB) | SCS (kHz) | Power boosting level (dB) | Minimum allowed channel bandwidth (MHz) |
| --- | --- | --- | --- |
| 2 | 15 | 3 | 5 |
| 2 | 15 | 6 | 15 |
| 2 | 15 | 9 | 20 |
| 2 | 30 | 3 | 10 |
| 2 | 30 | 6 | 25 |
| 2 | 30 | 9 | 40 |

- SCS = 15 kHz

- SCS = 30 kHz

Fig. 1, NR power degradation Vs. EPRE ratio & CBW

However, each vendor may have its own implementation and consideration for power boosting operation, the supported minimum channel bandwidth for the operation may be higher than the values in the table above. Moreover, operators may also have different allowed NR power degradation, hence, it is better not to specify but to declare the configurations for power boosting. Of course, the declaration should be made via allowed NR power degradation, SCS, power boosting level and minimum supported channel bandwidth. Note that the minimum supported channel bandwidth shall be no less than the corresponding minimum allowed channel bandwidth.

Proposal 3:  Power boosting operation is declared via enumerations of allowed NR power degradation, SCS, power boosting level and minimum supported channel bandwidth (no less than the corresponding minimum allowed channel bandwidth).

To facilitate the declaration, only non-zero power boosting is allowed to be declared, and a minimum power boosting level, e.g., 3dB, is needed, however, the maximum power boosting level is up to declaration thus no need to introduce a cap for this purpose.

Proposal 4: Introduce a minimum power boosting level, e.g., 3dB, for LP-WUS power boosting.

Proposal 5: Do not introduce a cap for power boosting level for LP-WUS power boosting.

Issue 1-6: On applicable BS type for LP-WUS feature

In our views, allowing both AAS and non-AAS BS for LP-WUS may promote deployment flexibility at least which supports the diverse needs of various applications and environments.

Proposal 6: Do not introduce any restriction on applicable BS type for LP-WUS feature.

Issue 1-7: Whether to define the LP-WUS requirements only for supported BS types

Considering the progress across WGs and time-line in Rel-19 for this WI, RAN4 could focus on defining conductive requirements at this stage.

Proposal 7: RAN4 to focus on defining conductive requirements for supported BS types at this stage.

# 3 Conclusion

In this contribution, we have the following observations and proposals on BS RF requirements for LP-WUS:

Observation 1: There are still not applicable configurations for a power dynamic range value with the number of LP-WUS RBs updated from 25 to 11.

Observation 2: Since power boosting does not change the total maximum transmit power of BS, there is no power boosting for the case where all RBs are used by LP-WUS and 0 RBs for NR, i.e., SCS 30kHz and CBW 5MHz.

Observation 3: 2dB NR power degradation already precludes the configuration from power boosting where LP-WUS occupies all REs within the NR carrier, i.e., SCS 30kHz and CBW 5MHz.

Proposal 1: RAN4 to define LP-WUS power boosting in terms of EPRE ratio as:

The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs) when both LP-WUS and NR are transmitted at the same time in the NR carrier.

Proposal 2: channel bandwidths for power boosting operation are indicated by the combinations consisting of allowed NR power degradation, SCS, power boosting level, minimum allowed channel bandwidth as shown in below table.

| Allowed NR power degradation (dB) | SCS (kHz) | Power boosting level (dB) | Minimum allowed channel bandwidth (MHz) |
| --- | --- | --- | --- |
| 2 | 15 | 3 | 5 |
| 2 | 15 | 6 | 15 |
| 2 | 15 | 9 | 20 |
| 2 | 30 | 3 | 10 |
| 2 | 30 | 6 | 25 |
| 2 | 30 | 9 | 40 |

Proposal 3:  Power boosting operation is declared via enumerations of allowed NR power degradation, SCS, power boosting level and minimum supported channel bandwidth (no less than the corresponding minimum allowed channel bandwidth).

Proposal 4: Introduce a minimum power boosting level, e.g., 3dB, for LP-WUS power boosting.

Proposal 5: Do not introduce a cap for power boosting level for LP-WUS power boosting.

Proposal 6: Do not introduce any restriction on applicable BS type for LP-WUS feature.

Proposal 7: RAN4 to focus on defining conductive requirements for supported BS types at this stage.

# Reference

[1] R4-2416589, “Way Forward for [112bis][308] NR_LPWUS”, vivo

[2] R4-2415134, “On BS RF requirements for LP-WUS”, CATT

[3] R4-2411094, “Further discussion on BS RF requirements for LP-WUS”, CATT