3GPP TSG-RAN WG4 Meeting #113	R4-2417878

Orlando, US, Nov 18th – 22nd, 2024

Title: 	Further discussion on additional information for IMT parameters

Agenda Item:	7.2.5

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In the LS from ITU-R WP5D requesting for parameters for compatibility and sharing studies, other information on AAS implementation is also requested in additional to parameters related to three frequency ranges (4400 MHz – 4800 MHz, 7125 MHz – 8400 MHz, 14.8 GHz – 15.35 GHz) [12]:

d) IMT base station (inclusive of its AAS-related parameters) and IMT user equipment technical parameters for analyzing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations. WP 5D also seeks information about how the AAS performs in adjacent bands i.e. does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation? e) Considering the system-level simulation work typically conducted in WP5D for sharing and compatibility studies using 3D modelling of antenna pattern, WP5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern is welcomed.

Accordingly, in order to respond to these additional information requests from ITU-R, the study item sets up two objectives accordingly [11]:

• As part of additional information requested by ITU-R WP 5D establish a model for array correlation factor (ρ) roll-off to describe how the array factor collapses moving away from the centre frequency. If the beamforming performance degrades towards a single element radiation pattern, then the frequency when the continuity of array features is regarded as degraded, will also need to be determined. • Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

In RAN4#112, several papers were submitted to discuss these additional information requests from ITU-R LS besides parameters directly related to three frequency ranges [1 – 10], although there were no online or offline discussions on this topic since the focus was on the reply LS on the second frequency range.

And in RAN4#112bis, the discussion was not sufficient due to the limited of time.

In this contribution, we present our views on these additional information requests.

# 2 Discussion

## 2.1 Array modelling for adjacent channel

In [14], studies show that in some cases ACLR becomes spatial sensitive based on the evaluation for a channel bandwidth 5MHz and a lower carrier frequency (~ 2GHz). Similar studies could be interesting for a much larger channel bandwidth (100 MHz ~ 400 MHz) at a higher carrier frequency (15GHz).

Observation 1: Evaluation may be interesting for a channel bandwidth 100 MHz ~ 400 MHz at 15GHz for the cases where spatial sensitivity of ACLR is observed for 5MHz channel bandwidth at 2GHz.

It is important to note that in coexistence simulations, the interference for each UE is determined by summing all the received signals based on the transmitter and receiver antenna patterns, along with the configured ACIR, which is uniform in all directions and identical to the value used when the desired signal and interference are aligned in the same direction. Therefore, even though spatial variations in ACLR may be observed in some scenarios, the current method of calculating interference for each UE assumes the worst-case scenario in terms of spatial ACLR. This serves as the basis for defining the minimum ACIR requirement, making it unnecessary to introduce a spatial ACLR requirement.

Observation 2: The current coexistence simulation assumes the worst-case scenario in terms of spatial ACLR by applying a uniform ACIR value, which is the basis for defining the minimum ACIR requirement.

Proposal 1: RAN4 to reply WP5D that the current coexistence methodology is sufficient without a need of taking into account spatial ACLR.

## 2.2 Multi-user spatial beam-forming

In ITU Recommendation M.2101, K active UEs are chosen for each BS in downlink simulation in each snapshot, and these K active users share all available RBs equally without overlapping [13]:

… d. Select K UEs randomly from all the UEs linked to one BS as “chosen” UEs. These K “chosen” UEs will be scheduled during this snapshot. If beamforming is used, point selected BS/UE beams toward each other. e. All available resource blocks (RBs) will be allocated to “chosen” UEs and each UE is scheduled with the same number n of RBs. Thus, the BS transmitted power per UE is fixed. f. BSs are transmitting with full power or are silent with a load probability, i.e. x% of the BSs is randomly selected to transmit and the rest remain silent. See § 6. The value of x can either be a single number or selected randomly from a range in each snapshot. For those BSs that transmit, the power per UE is calculated as follows: Let PBSMax denote the maximum transmit power of BS M = n x K is the number of all available RBs for each BS PBSUE is the transmit power from BS to the “chosen” UE, and n is the number of resource blocks per UE, PBSUE=PBSMaxnM (5) …

Hence, no multi-user spatial beam-forming is used in the downlink simulation methodology.

Observation 3: In M2101 downlink simulation methodology, no multi-user spatial beam-forming is considered.

If considering multi-user spatial beam-forming techniques, it is actually applying MU-MIMO at the downlink, which means the same set of RBs are used for multiple co-scheduled UEs at the same time. In our understanding, if co-scheduled UEs are chosen ideally, it is equivalent to increase the number of snapshots in the downlink simulation, and if the number of snapshots is sufficient for one simulated scenario, it has no impact on the assessment of interference and accuracy of studies.

Observation 4: With sufficient number of snapshots for one simulated scenario and ideally paired UEs for co-scheduling, multi-user spatial beam-forming does not improve the assessment of interference and accuracy of studies.

Proposal 2: RAN4 to reply WP5D that no multi-user spatial beam-forming is needed for assessing AAS implementation with a sufficient number of snapshots for one simulated scenario and ideally paired UEs for co-scheduling.

# 3 Conclusion

In this contribution, we have the following observations and proposals for additional information requests from WP5D:

Observation 1: Evaluation may be interesting for a channel bandwidth 100 MHz ~ 400 MHz at 15GHz for the cases where spatial sensitivity of ACLR is observed for 5MHz channel bandwidth at 2GHz.

Observation 2: The current coexistence simulation assumes the worst-case scenario in terms of spatial ACLR by applying a uniform ACIR value, which is the basis for defining the minimum ACIR requirement.

Observation 3: In M2101 downlink simulation methodology, no multi-user spatial beam-forming is considered.

Observation 4: With sufficient number of snapshots for one simulated scenario and ideally paired UEs for co-scheduling, multi-user spatial beam-forming does not improve the assessment of interference and accuracy of studies.

Proposal 1: RAN4 to reply WP5D that the current coexistence methodology is sufficient without a need of taking into account spatial ACLR.

Proposal 2: RAN4 to reply WP5D that no multi-user spatial beam-forming is needed for assessing AAS implementation with a sufficient number of snapshots for one simulated scenario and ideally paired UEs for co-scheduling.

# Reference

[1] R4-2411520	Views on Additional AAS aspects, Qualcomm

[2] R4-2411874	On general aspects related to IMT parameter SI, Ericsson

[3] R4-2411873	On the topic of additional information requested by ITU-R WP 5D, Ericsson

[4] R4-2411948	IMT parameters: General system and UE aspects, Nokia

[5] R4-2412592	Study of AAS performance in adjacent bands, Nokia

[6] R4-2413368	Impact of beamforming schemes on the coexistence of IMT with other services, Nokia

[7] R4-2412713	Discussion on other issues in ITU-R LS, ZTE

[8] R4-2413281	AAS modelling considerations for IMT base stations, Huawei

[9] R4-2411093	TP for other issues (Adjacent channel modelling), CATT

[10] R4-2411021	Text Proposals on MIMO models and PA nonlinearity impacts and ACLR, Spark NZ Ltd.

[11] RP-241326 	Revised SID: Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson

[12] R4-2400333	LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R WP5D

[13] ITU-R M.2101-0, Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies

[14] E. Sienkiewicz, N. McGowan, B. Göransson, T. Chapman and T. Elfström, "Spatially dependent ACLR modelling," 2014 IEEE Conference on Antenna Measurements & Applications (CAMA), Antibes Juan-les-Pins, France, 2014, pp. 1-4,