3GPP TSG-RAN WG4 Meeting # 113								    R4-2417902

Orlando, US, 18th – 22nd November, 2024

Agenda Item: 7.1.1.3.4

Source: MediaTek Inc.

Title: UE SRS IL imbalance issue for 6Rx UE

Document for: Discussion

# 1	Introduction

In RANP#103, the WI [1] was agreed with one objective for 6Rx. One of the objectives is to study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#112 meeting, the issue of 6Rx insertion loss (IL) was discussed. A WF [2] was agreed with the relevant parts captured below:

# Topic 4: SRS IL imbalance issue ## Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 4-2-1. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on the compromised solution proposals in Issue 4-2-1. ## Sub-topic 4-2: SRS IL imbalance issue solutions Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the candidate solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded. - Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded - Proposals - Proposal 1: Only introduce UE compensation and new accurate SRS power headroom reporting for non-CA/DC scenarios can be a compromised solution. Further study the granularity of new SRS power headroom report (China Telecom) - Proposal 2: IL imbalance reporting mechanism for SRS AS should include the configured maximum output power per SRS resource, the power headroom per SRS resource and ΔPPowerClass . The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission (Ericsson). - Proposal 3: Another way to resolve the SRS IL imbalance reporting issue would be to introduce two types of reporting for a UE: a “baseline” and an “advanced” reporting. The baseline reporting would not require any calibration at the UE and it could include e.g. the PHR and ΔPPowerClass information. The advanced reporting could in addition include the information on e.g. the configured maximum output power (Ericsson). - Proposal 4: Solution for IL imbalance issue as identified below. Reporting power threshold needs to be considered for static and dynamic reporting. The granularity of reporting could be per band (Spreadtrum). - If UE reports statically, report the actual SRS insertion loss with no UE self-compensation - If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time - Proposal 5: Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE) (Intel). Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) } - Proposal 6: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient (Huawei, HiSilicon). - Proposal 7: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency (Huawei, HiSilicon). - Proposal 8: To keep the current UE implementation untouched and avoid specification impact to most extent, following solution can be considered in Rel-19 (Huawei, Hi Silicon): - UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation. - Proposal 9: Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. (ZTE, Sanechips) - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

In this paper, we provide our views for above issues.

# 2	Discussion

In our previous paper [3], we provided the technical analysis and the system-level simulation results for both UE-side pre-compensation and BS-side post-compensation. We do not plan to repeat the analysis again here. Nevertheless, the observations are copied as below.

Observation 1: Additional UE transmit power reduction due to IL, if not compensated, will degrade DL performance due to lower SNR in gNB channel estimation on SRS for determining DL precoder.

Observation 2: Pre-compensation by UE can directly improve the SNR condition at gNB, while post-compensation by gNB may amplify the errors in the channel estimation.

Observation 3: From the system-level simulation results, UE pre-compensation is always better than BS post-compensation in all cases.

Observation 4: From the system-level simulation results, using ‘Type-I CSI’ can achieve better performance than ‘SRS-based CSI’ for cell edge UEs.

Observed from the simulation results in [3], BS-side post-compensation does not always bring system-level gain. In some cases, it could be even worse than doing nothing. Therefore, we are currently not fully convinced to introduce any insertion loss reporting mechanism from UE to network. Nevertheless, we are looking forward understanding how network can deal with this insertion loss report. Once the mechanism is clear, we can help to conduct the corresponding system-level simulation to check how much throughput gain can be achieved. In our views, we would only agree to introduce a new reporting mechanism, if the network corresponding handling is clear and the gain to the whole system is justified.

Proposal 1: Before agreeing on introducing a UE report on insertion loss, it is important to clarify how network can leverage this report and how much system benefit can be achieved.

If such a system-level benefit cannot be justified, RAN4 can still take UE pre-compensation as the baseline solution. One additional thing we can do in Rel-19 could be to introduce a UE feature on whether UE can support this pre-compensation (and possibly the corresponding test cases). From the discussions in last meeting, whether UE will do pre-compensation is never defined in 3GPP spec. There is ambiguity in legacy UE’s behaviour. Therefore, introducing this UE feature may help the market to differentiate different UE behaviours. However, introducing a UE feature does not implies introducing a UE capability. We still need to know how network can treat UEs differently when this UE capability is reported. Otherwise, there is no point to introduce such a UE capability.

Proposal 2: RAN4 should take UE pre-compensation as the baseline solution.

Proposal 3: As a compromise, a UE feature of pre-compensation can be introduced in Rel-19. FFS whether to introduce a corresponding UE capability.

# 3	Conclusion

In this paper, we discuss the issue of SRS insertion loss. We have the following observations and proposals.

Observation 1: Additional UE transmit power reduction due to IL, if not compensated, will degrade DL performance due to lower SNR in gNB channel estimation on SRS for determining DL precoder.

Observation 2: Pre-compensation by UE can directly improve the SNR condition at gNB, while post-compensation by gNB may amplify the errors in the channel estimation.

Observation 3: From the system-level simulation results, UE pre-compensation is always better than BS post-compensation in all cases.

Observation 4: From the system-level simulation results, using ‘Type-I CSI’ can achieve better performance than ‘SRS-based CSI’ for cell edge UEs.

Proposal 1: Before agreeing on introducing a UE report on insertion loss, it is important to clarify how network can leverage this report and how much system benefit can be achieved.

Proposal 2: RAN4 should take UE pre-compensation as the baseline solution.

Proposal 3: As a compromise, a UE feature of pre-compensation can be introduced in Rel-19. FFS whether to introduce a corresponding UE capability.

# 4	References

RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

R4-2417209, WF on requirements for 6Rx, AT&T

R4-2411774, UE SRS IL imbalance for 6Rx UE, MediaTek inc.