3GPP TSG-RAN WG4 Meeting #113	R4-2417917
Orlando, US, 18th – 22nd November, 2024

Title:	RAN task enable HPUE of 1UL CC with DL CA

Source:	OPPO

Agenda item:	9.2

Document for:	Approval

- Introduction

The power class ambiguity issue has been discussed for a long time and in RAN4#111 meeting the group decided to stop the discussion unless there is clear offline consensus. And then in RAN#104 meeting, a company paper was submitted to further pursue this topic, especially for the DL CA only case and would like to enable HPUE in DL CA with single UL CC case. And RAN task is given to RAN4 as below [1].

Way Forward - RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL. - RAN task shall conclude by RAN#106 - RAN4 online meeting time should be limited to <0.1 TUs

In RAN4#112bis, discussions continue but without conclusions, and the views can be represented by draft WF as below:

To enable UE to support higher power class than default power class for single configured UL carrier with DL CA configuration when HPUE requirements for the band combination have not been specified or the HPUE applicability note for the CA configuration (clause 5.5A) is not present: - Option A: - UE is allowed to indicate higher power class than default power class of single configured UL carrier with DL CA configuration when single carrier requirements with the higher power class requirements and default power class requirements for the CA configuration exist in specification - Applies for single configured UL carrier with both intra-band DL CA and inter-band DL CA configurations - UE can support UL-MIMO in the single carrier aligned with UL-MIMO single carrier maximum output power requirements - It is optional for UE to support higher than default power class for the above band combinations. - Solution is applied starting from rel-17 - Reference sensitivity requirements (e.g. MSD) for configurations with single configured UL carrier and DL CA: - Option 1: are only verified with default power class transmission - Option 2: are verified based on the requirements defined in clause 7 of 38.101-1. - Option B: - By following normal RAN4 basket WI procedure, HPUE in downlink CA with a single configured UL can be specified in RAN4 specification. - It is observed that one RAN4 meeting cycle is feasible to complete a band combination after it is added into basket WI. - RAN4 is also discussing how to further reduce the efforts in introducing the band combinations to RAN4 specification by for example look up table approach or only verify the default power class requirements, etc. With that it is expected that the introduction of HPUE for downlink CA with a single configured UL can be even more convenient in the future. - With above situation, RAN4 considers this RAN task can be closed.

This paper further discusses this issue.

- How to enable HPUE for DL CA with UL single CC

In last meeting, there are some offline discussions on how to enable UE to transmit higher power than default power class in 1ULCC / DLCA band combination. There are three candidate approaches as in table 1.

Table 1 Candidate approaches to enable HPUE for 1UL CC / DL CA

|  | Pros | Cons |
| --- | --- | --- |
| Approach 1: Follow normal basket approach, introduce the missing MSD | - Normal procedure followed, no ambiguity in the spec - Band combination HPUE is clear for implementation | - The MSD need to be introduced before HPUE can be enabled. - Need wait for 3 months before enable HPUE (i.e., add the band combination in this meeting, and requirement can be specified in next meeting) |
| Approach 2: Test PC3 first, then test PC2 after MSD is introduced | - HPUE can be enabled before spec is ready (3 months earlier) | - Normal basket procedure is broken, it introduces a precedent to bypass the basket approach - The requirements for a band combination will be changed from meeting to meeting, which cause chaos for implementation - UE implemented by following PC3 requirements may fail the PC2 requirements defined later which will cause certification problems - To NW, the uncertified PC2 UE may cause problem to the network. |
| Approach 3: Only test PC3 | - HPUE can be enabled before spec is ready (3 months earlier) - The HPUE MSD definition can be simplified once for all - It is clear for UE implementation and no risk of violating certification requirements | - MSD procedure need to be changed for HPUE, and no HPUE MSD is needed anymore |

In last meeting, approach 2 has been discussed widely, here, we have to emphasize below observations if go with this approach, and the potential problems need to be well understood:

Observation 1:   Many not properly tested PC2 UEs (without test against real PC2 MSD requirements) will flood into the network. These UEs may have bad PC2 REFSENS and thus have problems in access the network. This may cause user experiences degraded.

Observation 2:   These UEs once go into the market cannot update their hardware, even after the PC2 MSD requirements are introduced into the spec. These not properly tested UEs will exist for the whole UE life in the network which will last for many years.

Observation 3:   These UEs already on market must not be retested with the new 3GPP PC2 MSD requirements, they can only comply with the PC3 MSD. This needs to be clearly aware in 3GPP spec.

Based on above observations, in our view, approach 1 and 3 are more clean solutions if approach 2 is deemed acceptable.

- References

- RP-241687, Moderator's summary for offline discussion on CA Power Class Support, RAN vice-chair, RAN#104 2024-06