3GPP TSG-RAN WG4 Meeting #113										R4-2417935

Orlando, United States, 18th – 22th November, 2024

Agenda item:	7.8.3.2

Source: 	Xiaomi

Title: 	Discussion on UE RF requirements for NTN less than 5MHz

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 had some discussion on UE RF requirements for NTN less than 5MHz, and the related WF was approved in [1]. This contribution discusses the remaining issues for UE RF requirements for NTN less than 5MHz and provides our proposals.

Discussion

General spurious emission

Issue 2-1-9: General spurious emission - Way forward - FFS

The general spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) from the edge of the channel bandwidth. And it is assumed that the waveforms for NTN and TN are the same and the challenges to meet the emission requirements remain the same. Thus, it is proposed to reuse 3MHz spurious emission requirement defined in TS38.101-1 for NTN 3MHz CBW.

Proposal 1: Reuse 3MHz spurious emission requirement defined in TS38.101-1 for NTN 3MHz CBW.

Reference sensitivity

Issue 2-2-1: Reference sensitivity - Way forward - Option 1: 2.2dB improvement compared with 5 MHz CBW; - Option 2: 1.7dB improvement compared with 5 MHz CBW;

RAN4 needs to define new reference sensitivity requirements for NTN 3MHz CBW for band n254, n255 and n256. In theory, for 3MHz channel bandwidth with 15 PRBs, the total received power is reduced by 2.2dB compared with 5MHz channel bandwidth. However, for NR NTN, the maximum input power and in-band blocking interferer signal level are reduced to -40 dBm, the Rx IIP2 effects needs to be considered, and we support to consider option 2, e.g. 1.7dB improvement compared with 5 MHz CBW.

Proposal 2: For reference sensitivity, RAN4 to define new requirements for 3MHz for band n256, n255 and n254, and 1.7dB offset is applied compared with existing 5MHz requirements.

Maximum input level

Issue 2-2-2: Maximum input level - Way forward - FFS on the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm)

In the current 38.101-5, the requirement of maximum input level for 5/10/15/20MHz channel bandwidth is common, e.g. -40dBm. We propose to reuse the same maximum input level for NTN UE 3 MHz.

Proposal 3: The maximum input level for NTN 3MHz is defined as -40dBm.

Issue 2-2-3: ACS - Way forward - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE. - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

For ACS requirement, the existing requirements for NTN 5MHz/10MHz is reused for NTN 3MHz.

Proposal 4: For ACS requirement, the existing requirements defined for NTN 5MHz/10MHz is reused for NTN 3MHz.

Conclusion

In this contribution, we discussed the UE RF requirements for NTN less than 5MHz and provides our proposals:

Proposal 1: Reuse 3MHz spurious emission requirement defined in TS38.101-1 for NTN 3MHz CBW.

Proposal 2: For reference sensitivity, RAN4 to define new requirements for 3MHz for band n256, n255 and n254, and 1.7dB offset is applied compared with existing 5MHz requirements.

Proposal 3: The maximum input level for NTN 3MHz is defined as -40dBm.

Proposal 4: For ACS requirement, the existing requirements defined for NTN 5MHz/10MHz is reused for NTN 3MHz.

- Reference

[1]	R4-2416587	Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF		Xiaomi