# 3GPP TSG-RAN WG4 Meeting #113	R4-2417943

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	7.23.4.2

Document for:	Discussion

# 1Introduction

In RAN4#112bis meeting, the RRM impacts on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was discussed and a WF was approved [1]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

- LP-WUR requirements at RRC_IDLE/INACTIVE state

- General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Based on the discussion in previous meetings, RRM measurement relaxation case#1 and case#3 has already be agreed.

In case 1, i.e., fully offloading case, MR stops perform any RRM measurement and LP-WUR performs serving cell measurement. There is still FFS on the condition and details for the MR neighboring cell measurement. From our perspective, this is the case where UE could perform the greatest level of relaxation measurement. We prefer not to consider any neighbor cell measurement in this case.

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case #1, except for the higher priority frequency layer search.

In case 2, MR has RRM relaxation on serving cell measurement (no neighbour cell measurement is needed) and LP-WUR performs serving cell measurement. We think this is the case UE in the cell center, when UE does not need to measure neighbor cell, as well as no higher priority inter-f or inter-RAT configured. From our perspective, this is the typical case for starting LP-WUR monitoring and no need to start MR measurement. So, we prefer to de-prioritize this case.

Proposal 2: It is suggested to de-prioritize RRM relaxation case #2.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

Based on RAN1 progress, there would be two types of LP-WUR, one is capable of OOK envelope detection and the other one is capable of OFDM sequence detection. For LP-WUR with OOK envelope detection, i.e. OOK-based LP-WUR, only time domain processing could be conducted. The measurement performance is expected to be deteriorated compared with the LP-WUR with OFDM sequence detection, i.e. OFDM-based LP-WUR when conducting LP-SS based measurement with overlaid OFDM symbols. While for the OFDM based LP-WUR, we also think the measurement performance might be different for LP-SS with overlaid OFDM sequences and PSS/SSS considering the different OFDM sequence design. From our perspective, the impact on requirements due to different LP-WUR type and different reference signal, i.e., newly designed LP-SS (with/without overlaid OFDM sequences) and existing PSS/SSS should be studied. As the design for LP-SS with overlaid OFDM sequences is still not determined in RAN1, RAN4 can suspend study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions.

Proposal 3: RAN4 to suspend study on measurement requirements for OFDM based LP-WUR based on LP-SS in idle/inactive state until more RAN1 conclusions.

Proposal 4: RAN4 to define measurement requirements for:

- OOK-based LP-WUR serving cell measurement based on LP-SS in Idle/Inactive state, and

- OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS in Idle/Inactive state.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

In last meeting, companies showed their opinions on the entry/exist conditions for LP-WUR related operations, including LP-WUR measurement, LP-WUS monitoring and MR RRM measurement relaxation.

For the entry/exist conditions for LP-WUS monitoring, we understand that this issue is under discussion in RAN1. We think RAN4 should wait for RAN1 progress. RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered RAN1 if needed.

Proposal 5: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1/2 if needed.

As we understand that the LP-WUR measurement is initially introduced to guarantee the performance of LP-WUS monitoring, we think the measurement is not expected to start later than the LP-WUS monitoring. Also, we see little advantage in early starting of the LR measurement. If UE still use MR for paging reception, it is reasonable to use MR for RRM measurements as MR also need measurement for T/F synchronization. In this case, the LR measuremnt results seems useless. So, we prefer to define the same entry/exit condition of LP-WUR measurement and LP-WUS monitoring.

Proposal 6: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring.

RAN4 also discuss the entry/exit conditions for MR RRM measurement relaxation. Based on our analysis in the previous section, we think it is more reasonable that the MR RRM relaxation is executed when LR is used for serving cell measurement. If the LR is not doing measurement, the evaluation of measurement relaxation can only depend on serving cell measurement in MR, which is aligned with Rel-16 power saving functionality. If the LR is doing RRM measurement while MR is also doing normal RRM measurement, there seems little benefit but more power consumption. So, from our perspective, the entry/exit conditions for MR RRM measurement relaxation should also be the same as LP-WUR serving cell measurement. However, we understand RAN2 is currently working on the criteria design, it is suggest RAN4 to focus on the relaxation cases.

Proposal 7: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Issue 1-1-8: LP-WUR operating carrier frequency

One LS[2] was sent by RAN1 asking RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs based on the common understanding that UE may not support LP-WUS reception on all the bands supported by the UE. In last RAN4#112bis meeting, the below agreements were achieved.

Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

It is still FFS if there is any technical issue for the case MR and LR working on different carrier frequencies if it is supported in RAN1/2. In our understanding, this issue is about whether the measurement results obtained from LP-WUR can be used to evaluate the measurement results from MR when LR and MR working in different carrier frequencies. We think it can be achieved under certain condition. For example, when the MR and LR work on different carrier frequencies but in the same band and the LP-WUR and MR share the antenna, we think the measurement results obtained from LP-WUR can be used to evaluate the criteria for MR relaxation. However, as this is related to the NW development and UE implementation, we are open to further study.

Observation 1: Considering antenna sharing architecture, the measurement results obtained from LP-WUR can be used to evaluate the criteria for MR relaxation for the case of MR and LR working on different carrier frequencies but in same band.

Proposal 8: The discussion on RRM requirements for the case of MR and LR working on different carrier frequencies can be pending until further conclusions from RAN1/2 are available.

Issue 1-1-9: LP-WUR status

LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring is under discussion. In our understanding, this issue is related to the enter/exist conditions for LP-WUS entry/exit condition. Here below, we attached RAN2’s latest agreements:

- Agreements in RAN2#127bis - Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

RAN2 agreed that NW can configure thresholds for both MR and LR measurements. And the entry condition is met when all the measured results are above the configured threshold(s). In this case, we understand the LP-WUR status should be ON as UE need to obtain LR measurement results to evaluate the entry condition.

Proposal 9: LR status is ON before entering LP-WUS monitoring or after exiting LP-WUS monitoring, based on RAN2 agreements on LP-WUS monitoring entry condition.

Issue 1-1-11: Considerations on higher priority frequency layer

In accordance with the legacy measurement procedure for MR only UE, measurements for neighbor cell could be divided into two parts. One is for intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority, and the decision for MR to conduct these measurements hinges on the results from the serving cell measurements. When the signal quality of the serving cell reduce to certain threshold, the intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority will be triggered. The other one is for NR inter-frequency or inter-RAT frequencies with higher priority, and the decision for MR to conduct the measurements depend on NW configuration. In this case, as long as NW configuration indicates the priority information, UE needs to perform measurements of higher priority NR inter-frequency or inter-RAT frequencies.

Observation 2: According to existing cell selection/reselection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

We think the handling on NR inter-frequency or inter-RAT frequencies measurements with higher priority should also follow the legacy.

- Proposal 10: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case#1 and case#3.

- Detail LP-WUR requirements

Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering - Proposals - P1: LP-SS measurement delay requirement in IDLE/Inactive mode shall be defined based on LP-SS periodicity. (Apple) - P2: RAN4 to define measurement filtering in LP-WUR with at least min(DRX/2, 320)ms (Ericsson) Recommendations: Agreement from Coffee break discussion: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. Issue X: Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on Further discuss in the next meeting: Case 1: If LP-SS periodicity is configured, Option A: follow the same periodicity of LP-SS. (Apple, vivo, QC, HW, LGE, MTK, Xiaomi, CMCC) Option B: Based on DRX configured for MR (Samsung) Option C: based on LP-SS periodicity (E////) Other options are not precluded. Case 2: If LP-SS periodicity is not configured Option A: Based on DRX configured for MR (vivo) Option B: A fix value (like 320ms), FFS on the value (Apple, HW, LGE, vivo) Other options are not precluded.

During last meeting discussion, the measurement delay requirement for LP-WUR was divided into two cases. One is for LP-WUR based on LP-SS, the other one is for LP-WUR based on SSB. For the former case, agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. For the later case, i.e. LP-WUR based on SSB, the measurement delay requirements are still open.

The below attached agreements for the LP-SS periodicity was achieved in the latest RAN1#118bis meeting.

Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Initially, we support to define the measurement delay requirements based on the periodicity of LP-SS if LP-SS periodicity is configured, no matter LP-WUR is OOK-based or OFDM-based. However, based on the latest RAN1 progress, we can see that there is still possibility that the periodicity of LP-SS can be configured to 80ms or 160ms, which is comparable to the periodicity of SSB. We prefer to set a lower limit for the measurement delay requirements in case the periodicity of LP-SS can be configured to small value like 80ms or 160ms.

Proposal 11: RAN4 to define the measurement delay requirements based on the periodicity of LP-SS if LP-SS periodicity is configured, no matter LP-WUR is OOK-based or OFDM-based.

Proposal 12: RAN4 to define the lower limit for measurement delay requirements in case the periodicity of LP-SS can be configured to small value like 80ms or 160ms.

For the case LP-SS is not configured, we understand this is only for OFDM-based LP-WUR. Then, the OFDM-based LP-WUR obtains synchronization based on SSB. Basically, we think it is quite similar as MR. So, we prefer that the measurement delay requirements can be defined based on DRX cycle as the measurement delay requirements defined for MR.

Proposal 13: RAN4 to define the measurement delay requirements for LP-WUR based on DRX cycle, if LP-SS periodicity is not configured.

- MR RRM relaxation

- Agreements in RAN2#127bis - RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Based on the conclusion reached in study phrase, UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbor cells are applied and UE MR enters in ultra-deep sleep during LP-WUS monitoring. To obtain enough power saving gain, one of the objectives in the WI is to “specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR”. In Rel-16 power saving WI, relaxed RRM measurement requirements for neighbor cell in RRC_IDLE/INACTIVE mode are introduced. UE meeting the RRM relaxation criteria is allowed to conduct measurements with longer intervals. Generally, we agree that the Rel-16 relaxation method could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR.

Proposal 14: The Rel-16 relaxation method, i.e., extending measurement interval, could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR.

- Accuracy

During discussion in previous meeting, it was agreed to wait RAN1 progress in target SNR/SINR and use the RAN1 agreed target SNR/SINR as baseline for RRM simulation assumption. Meanwhile, companies still have different understanding on whether/how to define the accuracy. From our perspective, we support the idea that the accuracy discussed here is for simulation assumption and no need to define dedicated accuracy requirement in performance section. As per the existing serving cell measurement requirements defined in RRC_IDLE/INACTIVE state, no corresponding measurement accuracy requirement introduced. Since we are discussing RRM requirements for LP-WUR in RRC_IDLE/INACTIVE state, we prefer to follow the same logic as for MR.

Proposal 15: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

- LP-WUR requirements at RRC_CONNECTED mode

In RRC_CONNECTED mode, as clearly described in the objectives, UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think there should be no RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode expected. However, we are fine not to jumps to conclusions in this early stage.

Proposal 16: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Conclusion

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case #1, except for the higher priority frequency layer search.

Proposal 2: It is suggested to de-prioritize RRM relaxation case #2.

Proposal 3: RAN4 to suspend study on measurement requirements for OFDM based LP-WUR based on LP-SS in idle/inactive state until more RAN1 conclusions.

Proposal 4: RAN4 to define measurement requirements for:

- OOK-based LP-WUR serving cell measurement based on LP-SS in Idle/Inactive state, and

- OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS in Idle/Inactive state.

Proposal 5: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1/2 if needed.

Proposal 6: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring.

Proposal 7: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Observation 1: Considering antenna sharing architecture, the measurement results obtained from LP-WUR can be used to evaluate the criteria for MR relaxation for the case of MR and LR working on different carrier frequencies but in same band.

Proposal 8: The discussion on RRM requirements for the case of MR and LR working on different carrier frequencies can be pending until further conclusions from RAN1/2 are available.

Proposal 9: LR status is ON before entering LP-WUS monitoring or after exiting LP-WUS monitoring, based on RAN2 agreements on LP-WUS monitoring entry condition.

Observation 2: According to existing cell selection/reselection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

- Proposal 10: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case#1 and case#3.

Proposal 11: RAN4 to define the measurement delay requirements based on the periodicity of LP-SS if LP-SS periodicity is configured, no matter LP-WUR is OOK-based or OFDM-based.

Proposal 12: RAN4 to define the lower limit for measurement delay requirements in case the periodicity of LP-SS can be configured to small value like 80ms or 160ms.

Proposal 13: RAN4 to define the measurement delay requirements for LP-WUR based on DRX cycle, if LP-SS periodicity is not configured.

Proposal 14: The Rel-16 relaxation method, i.e., extending measurement interval, could be taken as baseline when considering the MR RRM relaxation for UE supporting LP-WUR.

Proposal 15: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Proposal 16: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Reference

- R4-2416861, WF for RRM core requirements for LP-WUS/WUR, Vivo

- R4-2414909, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1