# 3GPP TSG-RAN WG4 Meeting #113	R4-2417944

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on Conditional Intra-CU LTM for NR mobility enhancements Phase 4

Agenda Item:	7.24.2.2

Document for:	Discussion

# 1Introduction

In the last RAN4#112bis meeting, an initial discussion on Conditional Intra-CU LTM for NR mobility enhancements occurred and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further consideration on the related RRM core requirements and provide our proposals.

- Discussion

In Rel-18, LTM was introduced, offering the advantage of reduced interruption time. In Rel-19 NR mobility enhancement, conditional intra-CU LTM is agreed to be supported. We understand that this conditional intra-CU LTM may be akin to CHO or CPAC, as the procedure can be executed without the need for a signaling exchange with the source cell beforehand. We think related RRM requirements should be specified in RAN4.

In last RAN2 meeting, the following agreements on C-LTM were achieved:

Agreements on C-LTM in RAN2#127bis

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Below we list the approved WF related to Conditional LTM delay:

Issue 1-2-1: General aspects on Conditional LTM delay < Way Forward > FFS the following options: - Option 1: CHO requirements framework can be used as baseline. - Option 1a: Rel-18 LTM requirements framework can be used as references. - Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions. - Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM. - Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario. - Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM. Issue 1-2-2: The starting point of conditional LTM delay < Way Forward > FFS the following options: - Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. - Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message. - Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2. Issue 1-2-3: The ending point of conditional LTM delay < Way Forward > FFS the following options: - Option 1: - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell. - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement. - It can be revisited if any technical issue will be identified considering the progress in other WGs. - Other option is not precluded. Issue 1-2-4: The components of conditional LTM delay < Way Forward > FFS the following options: - Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt. - Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution. - Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU. - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design. - Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay.

Based on RAN2 agreements, the functionality of Conditional LTM basically follow CHO. We agree that CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM.

Legacy conditional handover delay is defined as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. Also, RAN4 define the the starting point is from the end of the last TTI containing the RRC command implying conditional handover. We support to define the starting point as for Conditional LTM as legacy CHO.

Proposal 2: The starting point of conditional LTM delay is from the end of the last TTI containing the RRC command implying conditional handover.

Similarly, the conditional LTM delay requirement could be defined with the similar components: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCLTM_execution. For each components:

- TRRC: The conditional LTM configuration is configured to UE via RRC. So, TRRC is the RRC procedure delay.

- TEvent_DU: RAN2 agreed that Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

- Tmeasure: RAN2 also discussed whether to use L3 measurement. So we think the definition of Tmeasure can wait for more RAN2 progress.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell. Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU , where the TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU can refer to clause 6.3.1.3.

- TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, including both RACH based and RACH less cases, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined.

Proposal 3: RAN4 to define the conditional LTM delay as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution, where

- TRRC is the RRC procedure delay,

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch,

- Tmeasure can wait for more RAN2 progress,

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell. Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU , where the TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU can refer to clause 6.3.1.3,

- TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, including both RACH based and RACH less cases, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined.

- Conclusion

Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM.

Proposal 2: The starting point of conditional LTM delay is from the end of the last TTI containing the RRC command implying conditional handover.

Proposal 3: RAN4 to define the conditional LTM delay as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution, where

- TRRC is the RRC procedure delay,

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch,

- Tmeasure can wait for more RAN2 progress,

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell. Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU , where the TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU can refer to clause 6.3.1.3,

- TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, including both RACH-based and RACH-less cases, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined.

- Reference

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom