# 3GPP TSG-RAN WG4 Meeting #113	R4-2417945

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on (e)RedCap RRM requirements in NTN for NR Phase 3

Agenda Item:	7.26.4.1

Document for:	Discussion

# 1Introduction

In the last RAN4#112bis meeting, further discussion on RRM requirements for Rel-19 NR NTN phase3 took place and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on (e)RedCap RRM requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

General consideration on RRM requirements

Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Cell reselection requirement for (e)RedCap UE with FR1-NTN is still under discussion. The open issue is whether to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement. In R18 NTN, inter-RAT cell resection requirements from NTN in FR1-NTN to LTE TN was introduced. In R17 RedCap, inter-RAT E-UTRAN measurement requirements was also introduced. So, from our perspective, we think it is reasonable to define the measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE with FR1-NTN.

Issue 2-3-1: RedCap R17 relaxed measurement Online agreement <Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

RRM measurement relaxation for neighbour cells is introduced as an optional feature for RedCap UEs that can be enabled by the network in both RRC idle/inactive states and RRC connected state. From RRM perspective, the spec impact only exists in RRC idle/inactive states. The UE is allowed to further relax neighbour-cell RRM measurements (compared to existing Rel-16 relaxation functionality) when an RSRP/RSRQ-based stationarity criterion is met for a period of time, or when both the stationarity criterion and a not-at-cell-edge criterion are met.

Based on the current TS 38.133, NTN has already supported Rel-16 relaxation functionality, i.e. relaxed measurement criterion for UE with low mobility and relaxed measurement criterion for UE not at cell edge.

#### 4.2C.2.7 Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.9 apply provided that target cell’s satellite is GEO. #### 4.2C.2.8 Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion The requirements in clause 4.2.2.10 apply provided that target cell’s satellite is GEO.

If we follow the same logic as in Rel-17 NTN discussion, the requirements in clause 4.2B.2.9 and 4.2B.2.10 for UE fulfilling Rel-17 relaxation criteria, i.e. relaxed measurement criterion for a stationary (e)RedCap UE and relaxed measurement criterion for a stationary (e)RedCap UE not at cell edge, could apply provided that target cell’s satellite is GEO.

Proposal 2: RAN4 to define relaxed measurement requirements for (e)RedCap UE with FR1-NTN when it fulfilling Rel-17 relaxation criteria, i.e. relaxed measurement criterion for a stationary (e)RedCap UE and relaxed measurement criterion for a stationary (e)RedCap UE not at cell edge, provided that target cell is GSO.

Issue 2-3-2: eDRX configuration Online agreement <Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

The Rel-17 RedCap WI introduced extended DRX cycles for RRC Idle state (up to 10485.76 seconds) and RRC Inactive state (up to 10.24 seconds). Furthermore, the Rel-18 eRedCap WI introduced enhanced eDRX cycle length in RRC Inactive state beyond 10.24 seconds, up to the same eDRX cycle length as for RRC Idle state (up to 10485.76 seconds). During discussion in the (e)RedCap WI, the eDRX feature is introduced as an optional feature for both RedCap and non-RedCap UEs. As the eDRX feature is not specific for (e)RedCap UE, we think RAN4 should discuss whether to support eDRX feature for NR NTN.

In last meeting, it was agreed to define measurement requirements for eDRX feature in GSO scenario and all DRX and eDRX related requirements for RedCap UE in TN are re-used. For LEO scenario, further discussion is needed.

Based on current spec, the UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment, considering the limited serving time in earth-moving LEO scenario. So, we think the eDRX feature is not applicable for earth-moving LEO development.

For quasi-earth fixed LEO deployment, we understand the serving time can be up to several minutes. Considering that the number of samples needed for Tdetect/Tmeasure/Tevaluate should be completed within the serving time, we think the upper limit for the applicable eDRX cycle can be 10.24s.

Proposal 3: The eDRX feature is not applicable for earth-moving LEO development.

Proposal 4: RAN4 to define measurement requirements for eDRX cycle length = 2.56s, 5.12s and 10.24s for quasi-earth fixed LEO deployment.

Issue 2-3-3: Multiple SMTCs <Way Forward>: - Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands. - Option 1: make it optional for RedCap UE to support 2 SMTC per MO. - Option 2: change the requirements so that measurement for different SMTCs are not parallel. - Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario. - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC. - Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands. - Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE. Issue 2-3-4: Concurrent gap <Way Forward>: - Proposal 1 (CMCC, HW): Consider concurrent gap for Redcap over NTN. - Proposal 1a (HW): Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline. - Proposal 2 (CATT, QC): NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE. - Proposal 3 (Ericsson): RedCap for NTN supports (at the least) 1 MG. Issue 2-3-5: The number of searchers and CSSF <Way Forward>: - Proposal 1 (CMCC, CATT): Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

In NTN Rel-17, a NTN UE shall at the least be capable of 2 SMTCs with same periodicity but different offsets to measure neighbour satellites and the serving satellites. However, considering the reduced complexity of (e)RedCap, it is challenging for RedCap UE to support simultaneous measurements. So we think it is more reasonable not to define requirements for the case of parallel measurements for (e)RedCap UE with FR1-NTN. Similarly, we also support not to consider concurrent gap for RedCap NTN UE.

- Proposal 5: NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

- Proposal 6: NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE.

- Proposal 7: Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Reduced UE complexity

#### Issue 6-3-1: The principle for defining the requirements for 2Rx/1Rx (e)Redcap UE with FR1-NTN Online agreement <Agreement> - For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx. - RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx (e)Redcap UE, if needed. - As baseline, follow the same principle for (e)RedCap UE in TN network. - For 2Rx (e)RedCap UEs with FR1-NTN: Reuse the existing requirements for NTN as a baseline. - Meanwhile, consider the following difference between 2Rx (e)RedCap and non-RedCap UE: - The number of searchers - Others if any #### Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ? <Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

Based on Rel-17 and Rel-18 progress, the main characteristic for RedCap UE is the reduced UE complexity. RedCap UE supports 1 or 2 UE Rx branches and HD-FDD or FD-FDD operation. Based on last meeting progress, it was agreed to follow the same principle for (e)RedCap UE in TN network for (e)RedCap UEs with FR1-NTN.

Besides some additional clarification on available samples for HD-FDD operation, the same RRM requirements as for normal NR UE are reused for Rel-17 RedCap with 2Rx, while the relaxed measurement requirements are defined for Rel-17 RedCap with 1Rx. The relaxation method including extending the number of measurement samples and relaxing the measurement accuracy. RAN4 has also agreed to reuse the existing requirements for NTN as a baseline for 2Rx (e)RedCap UEs with FR1-NTN. So, from our perspective, when defining RRM requirements for 1RX (e)RedCap UEs with FR1-NTN, similar principle as 1RX RedCap in TN network could be taken.

- Proposal 8: RAN4 to define RRM requirements for 1 RX (e)RedCap with FR1-NTN by relaxing requirements of 2 RX RedCap with FR1-NTN:

- Cell reselection and Random access: 1 dB offset to the cell-specific RSRP thresholds

- Handover: sample relaxation for Tsearch

- Re-establishment and Re-direction: sample relaxation for Tidentify

- RLM/BFD: sample relaxation for evaluation period and hypothetical PDCCH parameters update

- L3 and L1 measurement: accuracy relaxation

HD-FDD

For HD-FDD, it was agreed that the TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point. Currently, RAN1 is still working on evaluating the applicability of the existing handling rules for HD collisions cases, considering the TA mismatch between actual TA used by UE and assumed TA at the gNB. RAN4 can waiting RAN1’s final decision to check whether there is new RRM impact.

Proposal 9: RAN4 to check whether existing HD-FDD related requirements can be reused or not after RAN1 reaching further conclusions.

- Conclusion

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE with FR1-NTN.

Proposal 2: RAN4 to define relaxed measurement requirements for (e)RedCap UE with FR1-NTN when it fulfilling Rel-17 relaxation criteria, i.e. relaxed measurement criterion for a stationary (e)RedCap UE and relaxed measurement criterion for a stationary (e)RedCap UE not at cell edge, provided that target cell is GSO.

Proposal 3: The eDRX feature is not applicable for earth-moving LEO development.

Proposal 4: RAN4 to define measurement requirements for eDRX cycle length = 2.56s, 5.12s and 10.24s for quasi-earth fixed LEO deployment.

- Proposal 5: NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

- Proposal 6: NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE.

- Proposal 7: Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

- Proposal 8: RAN4 to define RRM requirements for 1 RX (e)RedCap with FR1-NTN by relaxing requirements of 2 RX RedCap with FR1-NTN:

- Cell reselection and Random access: 1 dB offset to the cell-specific RSRP thresholds

- Handover: sample relaxation for Tsearch

- Re-establishment and Re-direction: sample relaxation for Tidentify

- RLM/BFD: sample relaxation for evaluation period and hypothetical PDCCH parameters update

- L3 and L1 measurement: accuracy relaxation

Proposal 9: RAN4 to check whether existing HD-FDD related requirements can be reused or not after RAN1 reaching further conclusions.

- Reference

- R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT