# 3GPP TSG-RAN WG4 Meeting #113	    R4-2417946

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on other RRM core requirements in NTN for NR Phase 3

Agenda Item:	7.26.4.2

Document for:	Discussion

# 1Introduction

In the last RAN4#112bis meeting, discussion on RRM requirements for Rel-19 NR NTN phase3 occurred and a WF was approved as detailed in [1]. In this contribution, we would like to provide our further discussion on other RRM core requirements in NTN for NR Phase 3 and provide our proposals.

- Discussion

NR-NTN downlink coverage enhancement

###### Issue 1-1: SSB periodicity enhancement Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc. ###### Issue 1-3: RRM impacts due to Satellite Tx Beam Hopping Agreement: - RAN4 to wait for more progress from RAN1/2 on the detailed solutions to support a satellite T/Rx beam sweeping pattern, e.g. to adopt the concept of Rel-18 cell DTX/DRX, before discussing whether and how to modify the relevant RRM requirements.

Based on the latest revised WID, SSB channel enhancement focusing on SSB periodicity extension will be considered in RAN1, while the final decision will be given by RAN1 at the latest by RAN#106. The concrete method for SSB periodicity extension depends on the subsequent RAN1 discussion. Based on current RAN1 progress, the following agreements were reached.

Agreements in RAN1#112 As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint Agreements in RAN1#112bis For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

Based on the progress, RAN1 agreed to support the extended SSB periodicity during initial access. Currently, it is agreed the maximum additional default value is at least 160ms, while the legacy assumption is 20ms. From our side, at least the below requirements may be impacted:

- Cell reselection:

- Current cell reselection measurement requirements are defined based on DRX cycle, of which the smallest value is 320ms. RAN4 need to check the potential impact when SSB periodicity is extended to larger than 160ms.

- RRC Re-establishment and RRC Connection Release with Redirection:

- The current requirements involve description of UE’s assumption on SSB periodicity. As the default SSB periodicity can be larger than 160ms, the related requirements need to be revisited.

- Timing accuracy:

- Te are defined based on the assumption that SSB is available at least every 160ms. If the default value is larger than 160ms, there may be no SSB available during 160ms. The related requirements need to be revisited.

- L3 and L1 measurement:

- L3 and L1 measurements are defined based on MGRP/ SMTC period/ DRX cycle. We understand the extension of SSB periodicity would very likely lead to the extension of MGRP/ SMTC period/ DRX cycle. The requirements impact need to be further study.

Proposal 1: RAN4 to study the RRM impact of SSB periodicity extension on Cell reselection, RRC Re-establishment and RRC Connection Release with Redirection, Timing accuracy, L3 and L1 measurements.

- Conclusion

Proposal 1: RAN4 to study the RRM impact of SSB periodicity extension on Cell reselection, RRC Re-establishment and RRC Connection Release with Redirection, Timing accuracy, L3 and L1 measurements.

- Reference

- R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm