3GPP TSG-RAN WG4 Meeting # 113	R4-2417950

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for CSI compression

Agenda Item:	7.17.2.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability and interoperability issues for CSI compression.

- Discussion

      - 2.1 Simulation assumption and results

Issue 4-1: Simulation results discussion for Option 3 Agreement: Next step: Interested companies to share decoder and encoder ML models and dataset in next meeting (RAN4#113) - Model and dataset used to generate the data submitted to RAN4#112-bis Additional data to be submitted: - CDF of SGCS per subband - CDF of the largest eigen value of the channel matrix of training data for each subbands (preferably to be submitted in an xls format) Other agreements Interested companies are invited to bring further proposals for the next steps for the feasibility study of Option 4(for any of the sub-options of Option 4) - Analysis of commonality with option 3(whether any steps or data generated for Option 3 can be reused) would be useful

Here, we provide the CDF of SGCS per subband as below.

Fig.1 SGCS per subband

      - 2.2 Standardization process for option 3

In last meeting, companies discussed standardization process for option 3[1].

Issue 4-1: Option 3 next steps Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed Select one or more decoder for further analysis - selection criteria to be discussed Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Several steps are involved and needs further discussion.

Simulation alignment

In last meeting, simulation results for averaged SGCS cross all sub-band are submitted. The results among companies reach good alignment.

When CDF curves for sub-band are provided, there are 13 CDF curves. For each CDF curve, there are 5%, 50%,95% point. RAN4 needs to discuss the criteria for simulation results alignment.

Proposal 1: RAN4 should discuss and establish the criteria for simulation alignment.

Choosing a Reference Decoder

If the simulation results can be aligned, the next step is to select a reference decoder. It’s hard to choose the reference decoder for the first round. Therefore, we suggest that multiple decoders can be chosen as candidate first and then the reference decoder can be selected from these candidate decoders with some conditions. Company can use company-specific data to train decoder-encoder jointly.

We propose the steps as below:

- Candidate Decoders: Multiple reference decoders can be set as candidates.

- Training and Evaluation: Each company trains its own encoders to fit multiple candidate decoders and calculates the prediction accuracy for each pair based on company-specific data.

- Selection: Companies select decoders with SGCS values larger than a threshold and provide these decoders to RAN4.

- Final Choice: RAN4 chooses the decoder selected by most companies, indicating better generalization and compatibility with more encoders to achieve good performance.

Proposal 2: Multiple decoders can be chosen as candidates first and then the final decoder can be selected from these candidates with good generalization.

Proposal 3: If alignment can be achieved, company can train own encoder with multiple candidate reference decoders by company-specific data.

Result misalignment

If the simulation results using company-specific data cannot be aligned, RAN4 can consider setting up a common data set by mixing data from different companies. Companies can then train their encoder/decoder models based on this common data set to further align the results.

Then each company can provide encoder and decoder. Multiple decoders can be chosen as candidate reference decoder. Companies can then train their own encoder with multiple candidate reference decoders based on common data set.

Proposal 4: If alignment cannot be achieved using company-specific data, RAN4 should discuss how to combine data from different companies to create a common data set for training models.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: RAN4 should discuss and establish the criteria for simulation alignment.

Proposal 2: Multiple decoders can be chosen as candidates first and then the final decoder can be selected from these candidates with good generalization.

Proposal 3: If alignment can be achieved, company can train own encoder with multiple candidate reference decoders by company-specific data.

Proposal 4: If alignment cannot be achieved using company-specific data, RAN4 should discuss how to combine data from different companies to create a common data set for training models.

- Reference

- R4-2417212, WF on NR ALML air interface, Qualcomm