3GPP TSG-RAN WG4 Meeting # 113	R4-2417953

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on CSI-RS based L1 measurement for mobility

Agenda Item:	7.24.2.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 Mobility.

- Known cell condition - Definition of intra-frequency and inter-frequency - Supported measurement types - Pre-requisite of CSI-RS based L1 measurement

- Discussion

2.1 Known cell condition

Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Online Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

For the second and third bullet, many conditions are needed. The issue is whether all these conditions will be included in known cell condition or some of them are in applicability requirement. Take requirement applicability of SSB based LTM as example:

### 9.14.2 Requirements Applicability The requirements in the clause 9.14 are applicable to FR1 and FR2-1 for LTM. The requirements in clause 9.14 apply, provided for the SSB from the neighbor cell configured for intra-frequency L1-RSRP measurement, the following conditions are met: - The cell is known - The SSB configured for intra-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for L3 intra-frequency measurement. - The SSB resources configured for L1-RSRP measurements are measurable. An SSB resource configured for L1-RSRP for neigbor cell shall be considered measurable when for each relevant SSB the following conditions are met: - L1-RSRP related side conditions given in clause [10.1.19] for FR1 and [10.1.20] for FR2-1, respectively, for a corresponding band, - SSB_RP and SSB Ês/Iot according to Annex [B.2.4.1] for a corresponding band.

As shown above, for requirement applicability, there are several parallel conditions. Cell known condition and SSB for L1-RSRP measurement measurable condition are two independent conditions. Cell known condition is further defined later.

Observation 1: For SSB based LTM, known cell only refers that SSB of L3 measurement is detectable. L1 related conditions are defined in requirement applicability.

Therefore, we also suggest to use the same framework of SSB based LTM for CSI based LTM, e.g. known cell only refers that L3 measurement is detectable. L1 related condition are defined in requirement applicability.

Known cell condition for requirement applicability

For known cell condition for SSB based LTM is defined below:

The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2.ment remains detectable according to the cell identification requirements specified in clause 9.2.

For the first bullet, only L3 measurement but not SSB based L3 measurement is defined. From our understanding, there are two types of L3 measurement: SSB based L3 measurement and CSI-RS based L3 measurement. However, the 2nd bullet implicitly mentioned that SSB is configured for L3 measurement. Therefore, we don’t have strong view whether to only consider SSB based L3 measurement for CSI-RS based LTM. We slightly prefer to keep the current wording as “UE has performed L3 measurement”, similar as SSB based LTM.

L1 measurable/detectable condition for requirement applicability

We are not sure whether associated SSB will be configured for CSI-RS based LTM. There are some discussions in RAN1 how to link the CSI-RS resource with cell. For example, Association for the CSI-RS resource(s) and candidate cell is linked by implicit association (QCL source SSB). Or is linked by explicit association (by addition candidate cell ID). At least, CSI-RS for L1 measurement will be configured. Therefore, the CSI-RS from the target cell configured for L1 measurement needs to be remain measurable. Besides, it’s related to issue 2-4 about whether there will be SSB based L1 measurement in advance.

Proposal 1: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The SSB from the target cell configured for L3 measurement remains detectable according to the cell identification requirements in clause 9.2.

Proposal 2: Include the following conditions for requirement applicability:

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

  - FFS for SSB

2.2 Definition of intra-frequency and inter-frequency

In last meeting, there is some discussion about how to define intra-frequency and inter-frequency measurement for CSI-RS L1-RSRP.

Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session) - Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell. - Recommended WF - Continue discussion.

RAN1 agreements are as below:

Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement.

Observation 2: From RAN1, there is no restriction for frequency location of CSI-RS used for L1-measurement.

From RAN1 aspect, there is no restriction for frequency location of CSI-RS, for one cell, multiple CSI-RS resources can be configured with different center frequency, BW. However, it’s challenging to re-use the current intra-frequency definition if there is no any limitation for CSI-RS configuration.

If CSI-RS resources are configured without any restriction：

- For CSI-RS resource configuration of serving cell, multiple CSI-RS resource set can be configured. For each CSI-RS resource set, there can be many CSI-RS resources. For each CSI-RS resource, different frequency location, SCS can be configured.

- For CSI-RS resource configuration of one neighbor cell, similarly, multiple CSI-RS resources can be configured with different frequency location and SCS.

There are several possible issues:

- For serving cell, which CSI-RS resource can be used as center-frequency and SCS for serving cell.

- For one neighbor cell, some CSI-RS measurement can be intra-frequency measurement, while some others are inter-frequency measurement. The reason is that if there are many CSI-RS resources, it’s possible that the center frequencies and SCS of some CSI-RS resources are same as that of serving cell, while some center frequencies and SCS of CSI-RS resources are not. It’s quite different from legacy. In legacy, intra-frequency or inter-frequency measurement is defined per cell.

Therefore, if legacy intra-frequency definition will be re-used, the above issues need to be solved first. The simplest way is to limit all the CSI-RS resources configured for the same cell are with the same frequency location and SCS, similar as legacy CSI-RS based L3 configuration. Another alternative is to consider associated SSB as reference for center frequency.

Proposal 3: If legacy definition of intra-frequency and inter-frequency will be re-used, all the CSI-RS resources configured for the same cell should be with the same frequency location and SCS. RAN4 to discuss whether to limit the CSI-RS configuration.

2.3 Supported measurement types

- Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap - Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE) - Recommended WF - Continue discussion.

The main difference about measurement requirement is whether the measurement is performed inside or outside active BWP. For intra-f without gap and inter-f without gap, if both are within active BWP, the requirement is similar. Considering the complexity of performing CSI-RS based measurement, we suggest to prioritize CSI-RS based L1-RSRP measurement inside active BWP.

Proposal 4: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

2.4 Pre-requisite of CSI-RS based L1 measurement

- Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

From our understanding, the above two Alternatives are not conflicted. For Alternative 1, SSB-based L1 measurements and reports may be skipped under certain conditions. For Alternatives 2, SSB-based L1 measurements are skipped. Since Alternative 1 includes Alternative 2, we support Alternatives 1.

The main difference between Alternative 1 and Alternative 2 is step 2. The key issue is to analyse the benefit and drawback when step 2 is introduced.

When SSB based L1 measurement is performed before step 3:

- Possible Benefit:

  - UE can perform fine RX beam sweeping based on SSB. Reduce CSI-RS number configured for RX beam sweeping in step 3.

    - It’s FFS whether RX beam obtained during step 2 can be directly used in step 3. Since SSB based TX beam is wider than CSI-RS based TX beam.

  - Narrow down TX beam spatial coverage and reduce CSI-RS number configured for spatial coverage in step 3.

    - If step 1 can also provide certain TX beam information, then spatial coverage can also be reduced based on step1. This benefit is FFS.

- Drawback:

  - Extended L1 measurement time for neighbour cell. Since SSB sharing between different cells may be needed. Periodicity of SSB may be longer than CSI-RS resource.

  - Limit the NW configuration flexibility.

RAN4 to further discuss the possible benefit and drawback of step 2. As there may be many candidate cells, the total number of CSI-RS resource number may be quite large. While SSB is already there, if step 2 can help to reduce the CSI-RS number configured in step 3, then it’s beneficially not to skip step 2.

Proposal 5: RAN4 to discuss Alternative 1 and analyse the possible benefit and drawback of step 2.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: For SSB based LTM, known cell only refers that SSB of L3 measurement is detectable. L1 related conditions are defined in requirement applicability.

Proposal 1: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The SSB from the target cell configured for L3 measurement remains detectable according to the cell identification requirements in clause 9.2.

Proposal 2: Include the following conditions for requirement applicability:

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

  - FFS for SSB

Observation 2: From RAN1, there is no restriction for frequency location of CSI-RS used for L1-measurement.

Proposal 3: If legacy definition of intra-frequency and inter-frequency will be re-used, all the CSI-RS resources configured for the same cell should be with the same frequency location and SCS. RAN4 to discuss whether to limit the CSI-RS configuration.

Proposal 4: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

Proposal 5: RAN4 to discuss Alternative 1 and analyse the possible benefit and drawback of step 2.

- Reference