3GPP TSG-RAN WG4 Meeting # 113	R4-2417956

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on core part maintenance for Multi-RX

Agenda Item:	5.10.1

Document for:	Discussion

# 1 Introduction

In RAN4#110bis meeting, the WF about fast beam sweeping is agreed. However, there are still some issues.

- Discussion

- 2.1 fast beam sweeping

Issue 1-1-1: When is UE considered to be in multi-rx operation - Proposals - Proposal 1: Keep the agreed definition - Given UE indication of its preference of multi-RX/single-RX operation has been allowed, it is OK to take the current definition of multi-RX operation - Proposal 2: Add following additional condition - P2a: - UE sent a recent valid Rel-17 group-based beam reporting (GBBR). - If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair. - P2b: Rel-17 group-based beam reporting (GBBR) is activated/triggered by the network. - Recommended WF: - Further discuss. Issue 1-1-2: End point for fast beam sweeping application. - Proposals - Proposal 1: When multiple PDSCHs are not scheduled within 300s since group-based beam reporting is configured, the UE is allowed to exit fast beam sweeping. - Recommended WF: - Discuss whether need to define end point.

In RAN4 #110bis meeting, RAN4 agreed on following

- For fast beam sweeping, the UE is in multi-Rx operation if following condition is met:

  - UE is configured with group-based beam reporting (GBBR) report.

We understand the agreement is based on the intension that NW would like to control the starting point of fast beam sweeping, however, we also think that the current condition for multi-Rx operation is too loose which will lead to UE power consumption issue. we prefer to add some additional conditions for the starting of fast beam sweeping. For example, UE is activated with dual TCI state.

Proposal 1：Starting point of fast beam sweeping is that UE is configured with group-based beam reporting (GBBR) report and UE is activated with dual TCI states.

For the ending point of fast beam sweeping, it can be initiated by NW or UE.

If NW stops fast beam sweeping functionality, NW will switch dual TCI state activation to single TCI state activation.

If UE would like to stop fast beam sweeping when UE is in low status, UE can send indication in GBBR report. For example, the UE will report only one beam index or two beam indexes with one L1-RSRP value set to NULL.

Proposal 2: Ending point of fast beam sweeping can be initiated by NW or UE.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: Starting point of fast beam sweeping is that UE is configured with group-based beam reporting (GBBR) report and UE is activated with dual TCI states.

Proposal 2: Ending point of fast beam sweeping can be initiated by NW or UE.

- Reference