3GPP TSG-RAN WG4 Meeting #113																				R4-2417958

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.1

Title: 	Discussion on OD-SSB requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and the objectives are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

And in the last RAN4 meeting, the agreements for NES RRM were achieved in [4, WF R4-2416916]. Therefore, in this discussion, we provided more preliminary analysis on the potential RRM impacts due to these aspects in Rel19 NES.

- Discussion

  - General requirements

In the RAN4#112 meeting, the general consideration on OD-SSB scenario in which the RRM requirements impact shall be studied in RAN4 was agreed.

Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement.

Meanwhile, according to previous RAN1 discussion below, these OD-SSB operation scenarios in RAN1 are illustrated in the figures below.

R1#116bis Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

For Scenario 3A/3B as illustrated in Figure 1, it is possible that there is not any activated OD-SSB available after UE reception on SCell activation command. And since the current RAN4 RRM requirements on SCell activation delay are rely on SCell activation command reception as a start point, the significantly impacts on the existing Scell activation delay requirements can be expected. Both SCell activation command reception and OD-SSB trigger command will be taken count into the start point of Scell activation. For an example, the later one can be taken as the start point of Scell activation delay or the uncertainty margin between them shall be added to the whole delay.

Figure 1.

TS38.133 v18.5.0 8.3.2 SCell Activation Delay Requirement for Deactivated SCell The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length ,

Observation 1: For OD-SSB scenario 3A/3B, the significant RRM requirement impacts on current RAN4 spec [2, TS38.133] are expected.

Meanwhile, up to now in RAN1 the on-demand SSB SCell operation scenarios 3A/3B are still open.

Observation 2: The on-demand SSB SCell operation scenarios 3A/3B are still open.

That is RAN4 can also wait RAN1’s progress to avoid unnecessary working loads. Thus, we can propose that:

Proposal 1: RAN4 can start the discussion on RRM requirements for the scenarios 3A/3B of on-demand SSB upon RAN1’s further agreements.

  - RRM requirements impacts

    - OD-SSB L1 measurements

In the last RAN4 meeting, there was not discussion on whether OD-SSB based L1 neighbor cell measurements should be supported due to the limited time.

R1#118 discussion Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement: Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4

According to the further RAN1 agreement above, it is clear that L1 measurement requirement based on OD-SSB needs to be defined in RAN4. Otherwise RAN1 design can’t be tested. But from RAN4 itself it is unclear L1 measurement based on OD-SSB shall be performed before or after SCell activation.

In our views, for OD-SSB scenario 2/2A agreed RAN4 study the measurement on OD-SSB is possible after OD-SSB triggered. And according to TS38.133 requirements for SSB based SCell activation, L1 measurement based on SSB is needed also for SCell activation.

8.3.2 SCell Activation Delay Requirement for Deactivated SCell ……….. If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. ……… TL1-RSRP, enhanced_measure is - SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in reduceForSSB-L1-RSRP-Meas. Otherwise, if reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or, - CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0. - If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

Also the requirements of L1 measurement for other purpose beside SCell activation (e.g. for RLM) was also defined separately in TS38.133 clause 9.5.

9.5 L1-RSRP measurements for Reporting 9.5.1 Introduction When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. For UE supporting bwpOperationMeasWithoutInterrupt-r18, the measurements shall also be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements outside the active BWP. For UE supporting ncd-SSB-BWP-Wor-r18, the SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified. If SSB in the active DL BWP of serving cell i is NCD-SSB, for serving cell i the requirements in clause 8.1 apply provided that serving cell i is PCell or activated PSCell.

Observation 3: L1 measurement based on OD-SSB is needed also for the other measurements beside SCell activation.

That is the OD-SSB L1 measurement is needed during both SCell activation procedure and other RRM procedure. Thus, we can propose that:

Proposal 2: RAN4 shall define L1 measurement requirements based on OD-SSB in both clause 8.3 （for SCell activation delay） and Clause 9.5 （for other measurements procedures）in TS38.133. And the detailed requirements can be FFS upon further RAN1 agreements.

    - Case 1- OD-SSB based deactivated SCell measurement

In the previous RAN4 meetings, there was extensive discussion on which periodicity shall be applied for OD-SSB based deactivated SCell measurements and the following agreements was achieved in R4#112bis [4, R4-2416916].

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

When the OD-SSB was triggered for SCell activation, the periodicity of OD-SSB is expected to be short as possible to enable UE can complete the measurement quickly which can save the network power. That is the exact meaning of “fast L3 measurement” in RAN4 agreements above [4]. However, for the deactivated SCell, the periodicity of OD-SSB can be much sparser than these for the SCell activated or to be activated. As a result, UE need not to measure the OD-SSB so frequently to minimize UE’s power consumption. This is what “slow measurement” mean in comparison with the former one.

Thus, in the last RAN4 meeting, the harmonized solution to balance the power consumption and the reliable measurement results via OD-SSB occasions is to restrict the periodicity of measurement based OD-SSB in time window “Tfast” and during the time window “Tslow” UE can relax the measurement with “measCycleSCell” or stop the measurement as illustrated in Figure 2 below.

Figure 2.

Meanwhile in RAN1, there are extensive discussion on how to indicate the OD-SSB periodicity and the agreements below were achieved.

R1#117 discussion Proposal #3-1b (Signalling container): - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Proposal #3-2b (Contents): - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not R1#118 discussion Agreement (contents) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB FFS: Any other relevant parameters

Observation 4: The multiple periodicities for OD-SSB can be configured to UE and the exact applicable one will be indicated timely by MAC-CE.

Therefore, we believe when UE completed the SCell activation via OD-SSB NW should update the more proper OD-SSB periodicity to UE or deactivate OD-SSB for the power saving purpose. Otherwise, the motivation of NES OD-SSB will be vanished.

Observation 5: The more proper OD-SSB periodicity or deactivation of ODSSB shall be indicated by NW to obtain NES power saving gain.

Thus, it is naturally to assume the time window (Tfast) can start from the time point of SCell activation command (t0) and end at the MAC CE to update the ODSSB periodicity (t1). Also, the time interval (Tslow) can start from MAC CE to update OD-SSB (t2) and end by ODSSB deactivation (t3).

Figure 3.

Proposal 3: The time window in which UE shall perform the fast measurement once UE receives the OD-SSB activation indication can be defined as the duration from the SCell activation command received to the first MAC CE to update OD-SSB.

In order to further reduce UE’s power consumption, UE needs not perform such fast measurements if NW did not update the OD-SSB periodicity for UE in long time but the successful measurement results are obtained and reported over [>5] measurement samples. Under such circumstances, the total length of this time window Tfast can be also limited by other conditions (e.g. “max5*Tc1, t0,t1). Thus we also slightly prefer the alternative proposal below.

Proposal 3a: The time window in which UE shall perform the fast measurements once UE receives the OD-SSB activation indication can be defined as  max⁡(5*Tc1, [t0,t1] ). Where in

- t1 is the time point of SCell activation command

- t2 is the time point of first MAC CE to update OD-SSB after UE receiving Scell activation command,

- Tc1 is the OD-SSB periodicity when OD-SSB is triggered for SCell activation purpose.

On the other hand, it is straightforward that during Tslow UE can follow the measurement cycle depending on the updated OD-SSB periodicity. If there are multiple MAC-CEs to update the OD-SSB periodicity, the maximum one will be taken as the baseline measurement cycle for RAN4 measurement requirements specifying.

Proposal 4: The time window in which UE shall perform the slow measurements can be defined as the duration between t1 and t3 (denoted as[t2,t3]). Where in,

- t2 is time point when receiving MAC CE to update OD-SSB periodicity parameter

- t3 is time point when NW deactivated OD-SSB

The other question raised in the last RAN4 meeting needs be confirmed is which measurement cycle shall be followed during the Tslow.

Observation 6: It is unclear that which measurement cycle shall be followed during the Tslow.

For in case of there is only single updated MAC CE during Tslow, UE can follow the unique updated measurement cycle depending on the updated OD-SSB periodicity. If there are multiple MAC-CEs to update the OD-SSB periodicity, the maximum one will be taken as the measurement cycle during this Tint.

Proposal 5: The measurement cycle UE shall follow during the Tslow is

- Tc2, if only single MAC CE to update OD-SSB received or

- max⁡( Tc2, Tc3, …..), if multiple single MAC CE to update OD-SSB received

Alternatively, in order to further guarantee UE’s power consumption in comparison with the legacy deactivated SCell measurement behaviour [TS38.133], the measurement cycle applied can be also defined jointly with legacy deactivated SCell measurement cycle. Thus we can propose that:

Table 9.2.5.1-4: Time period for PSS/SSS detection, deactivated SCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Proposal 5a: From RAN4 measurement perspective, the measurement cycle which shall be followed by UE during the Tslow is

- min⁡(Tc2,  measCycleScell)  if

  - only single MAC CE to update OD-SSB received and

  - measCycleSCell indicated

- min⁡(Tc2,  [160ms] )  if

  - only single MAC CE to update OD-SSB received and

  - measCycleSCell absent

- min⁡(max⁡(Tc2,Tc3, ….),  measCycleScell)  if

  - multiple single MAC CE to update OD-SSB received and

  - measCycleSCell indicated

- minmax⁡(Tc2,Tc3, ….,[160ms])  if

  - multiple single MAC CE to update OD-SSB received and

  - measCycleSCell absent

    - SCell (de)activation with OD-SSB

In the last RAN4 meeting, on how to define the necessary requirement on SCell (de)activation with OD-SSB was discussed also with some high level agreements achieved.

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

For the detailed RRM requirements updates, in our view it needs more discussion under the different conditions and scenarios. For an example, as illustrated in Figure 2 below, for the case of “Scenario #2 and Case #1“ and known Scell activation case, if the first SSB transmitted after OD SSB trigger command can located before the time “n + THARQ+5ms+TfirstSSB +margin”, UE can follow the current behavior for Scell activation and corresponding requirements defined in TS38.133 for known cases can be reused. Otherwise, the requirements in current spec needs to be updated. However, for the unknown case since the more than one SSB needed for the SCell timing and frequency synchronization, the current requirements could not be reused with the condition above for known cases indeed.

Observation 7: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be updated.

Figure 4.

Proposal 6: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined.

Also, if there are more than one SSB within the time window of [ t0,t0 + n + THARQ+5ms+TfirstSSB +margin] , in order to save more power up to NW implementation, gNB needs only to forward the SSB of Scell which is closer to the end of this window.

Observation 8: RAN4 can also consider whether further power saving optimization method needs to be accounted when specifying SCell activation/deactivation requirements with on-demand SSB.

Nevertheless, beside the scenarios, RAN1 also discussed the always-on SSB is cell based or not (case 2).

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

That is it is possible UE can receive both non-cell-defined on-demand SSB and legacy SSB together for case 2. If this is implemented by gNB. From UE perspective, it is also up to UE to perform the measurement based either of them.

Observation 9: In order to allow UE’s implement flexibility (e.g. reduce UE’s power consumption with longer measurement cycle), it needs not require UE performing the SCell measurement with OD-SSB if both of OD-SSB and legacy SSB are available.

Especially, within a successful measurement reporting period in which at least 4 measurement samples required, if there are different types of SSB used, this will significantly impact RAN4 current requirements since there is not any requirements defined by the hybrid measurement cycle in TS38.133.

Figure 5.

Observation 10: In order to simplify RAN4 discussion, RAN4 can define the SCell activation measurement requirements base on a single SSB period cycle instead of hybrid ones.

In summary, we can suggest that:

Proposal 7: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon

- further power saving

- simplified specification works. E.g. no hybrid measurement SSBs for case 2.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: For OD-SSB scenario 3A/3B, the significant RRM requirement impacts on current RAN4 spec [2, TS38.133] are expected.

Observation 2: The on-demand SSB SCell operation scenarios 3A/3B are still open.

Proposal 1: RAN4 can start the discussion on RRM requirements for the scenarios 3A/3B of on-demand SSB upon RAN1’s further agreements.

Observation 3: L1 measurement based on OD-SSB is needed also for the other UE behaviors beside SCell activation.

Proposal 2: RAN4 shall define L1 measurement requirements based on OD-SSB in both clause 8.3 （for SCell activation delay） and Clause 9.5 （for other measurements procedures）in TS38.133. And the detailed requirements can be FFS upon further RAN1 agreements.

Observation 4: The multiple periodicities for OD-SSB can be configured to UE and the exact applicable one will be indicated timely by MAC-CE.

Observation 5: The more proper OD-SSB periodicity or deactivation of ODSSB shall be indicated by NW to obtain NES power saving gain.

Proposal 3: The time window in which UE shall perform the fast measurement once UE receives the OD-SSB activation indication can be defined as the duration from the SCell activation command received to the first MAC CE to update OD-SSB.

Proposal 3a: The time window in which UE shall perform the fast measurements once UE receives the OD-SSB activation indication can be defined as  max⁡(5*Tc1, [t0,t1] ). Where in

- t1 is the time point of SCell activation command

- t2 is the time point of first MAC CE to update OD-SSB after UE receiving Scell activation command,

- Tc1 is the OD-SSB periodicity when OD-SSB is triggered for SCell activation purpose

Proposal 4: The time window in which UE shall perform the slow measurements can be defined as the duration between t1 and t3 (denoted as[t2,t3]). Where in,

- t2 is time point when receiving MAC CE to update OD-SSB periodicity parameter

- t3 is time point when NW deactivated OD-SSB

Observation 6: It is unclear that which measurement cycle shall be followed during the Tslow.

Proposal 5: The measurement cycle UE shall follow during the Tslow is

- Tc2, if only single MAC CE to update OD-SSB received or

- max⁡( Tc2, Tc3, …..), if multiple single MAC CE to update OD-SSB received

Proposal 5a: From RAN4 measurement perspective, the measurement cycle which shall be followed by UE during the Tslow is

- min⁡(Tc2,  measCycleScell)  if

  - only single MAC CE to update OD-SSB received and

  - measCycleSCell indicated

- min⁡(Tc2,  [160ms] )  if

  - only single MAC CE to update OD-SSB received and

  - measCycleSCell absent

- min⁡(max⁡(Tc2,Tc3, ….),  measCycleScell)  if

  - multiple single MAC CE to update OD-SSB received and

  - measCycleSCell indicated

- minmax⁡(Tc2,Tc3, ….,[160ms])  if

  - multiple single MAC CE to update OD-SSB received and

  - measCycleSCell absent

Observation 7: When leveraging the current requirements in TS38.133 to the SCell activation/deactivation requirement with on-demanding SSB, whether SCell is known or unknown needs to be updated.

Proposal 6: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions shall be redefined.

Observation 8: RAN4 can also consider whether further power saving optimization method needs to be accounted when specifying SCell activation/deactivation requirements with on-demand SSB.

Observation 9: In order to allow UE’s implement flexibility (e.g. reduce UE’s power consumption with longer measurement cycle), it needs not require UE performing the SCell measurement with OD-SSB if both of OD-SSB and legacy SSB are available.

Observation 10: In order to simplify RAN4 discussion, RAN4 can define the SCell activation measurement requirements base on a single SSB period cycle instead of hybrid ones.

Proposal 7: RAN4 can further discuss the SCell activation/deactivation requirements with OD SSB upon

- further power saving

- simplified specification works. E.g. no hybrid measurement SSBs for case 2.

- References

- RP-240830

- TS38.133 v18.5.0

- TS38.331 v17.1.0

- R4-2416916