3GPP TSG-RAN WG4 Meeting #113	R4-2417973

Orlando, United States, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.16.2

Document for:	Discussion

Introduction

In RAN4#112bis the topic of intercell and intracell inter user interference for 8Rx configuration were further discussed with focus on enabling companies to provide simulation results for alignment:

- It was agreed to introduce CQI requirements with inter-cell interference, however the configuration for simulation alignment were not discussed due to available time in RAN4#112bis.

- Agreements were made enabling companies to provide initial simulation results for

  - PDSCH with inter-cell interference

  - PDSCH with intra-cell inter-user interference.

In the following we will discuss the remaining open issues and provide our view with observations and proposals to enable further progress on the topic

Discussion

General

8Rx UE MMSE-IRC receiver coverage

In RAN4#112bis it was discussed if one or two sets of requirements were to be introduced according to the baseline and simplified 8Rx receiver (see [1]):

8Rx UE MMSE-IRC receiver coverage Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for the initial simulation analysis. - TBD: Criteria used to decide if one set of requirements can be introduced covering both receiver types. - Note: Test coverage shall cover both receivers

We would re-iterate our view from RAN4#112bis where we prefer to define one set of 8Rx requirements for MMSE-IRC receiver, that is not related to a specific receiver type. Though, alternatively, we can be open to define two sets of requirements (baseline and simplified), if simulation results indicate that a simplified receiver may not be able to pass interference requirements for baseline receiver.

As agreed in RAN4#112bis companies can provide simulation results for baseline and/or simplified receivers and it can then be discussed in RAN4#113 if one or two sets of requirements shall be defined based on provided results and possible future agreement on criteria to decide if one or two sets of requirements are to be defined.

Normal criteria to define requirements is if the span across simulation results are <=2.5dB. Hence if total span across all provided simulations results for a configuration are <=2.5dB when reasonable number of outliers have been removed, only one set of requirements will be needed for said configuration.

In some cases, with higher than 2.5dB span even after a reasonable number of outliers has been removed, agreements have been made in earlier topics to increase the margin added to the provided impaired simulation results to adjust for the higher span. We are not in favour of using such an approach to secure only one set of requirements as it will result in less restrictive requirements for the baseline receiver.

This leads us to the following observations and proposals:

If the span after removing a reasonable number of outliers is <=2.5dB, only one set of requirements can be defined to cover both receivers. If the span after removing a reasonable number of outliers is >2.5dB it shall be considered to define two sets of requirements.

Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition.

In case it is decided to define two set of requirements, the current definition of baseline and simplified receiver in 38.101-4 [3] cannot be used as it is written for SU-MIMO configuration only:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

We therefor propose that in Rel.19 the definition shall be changed to be more generic to also cover the interference cases discussed in this WID.

The existing definition of baseline and simplified receivers in 38.101-4 is focused on SU-MIMO configuration. This needs to be changed to also cover the discussed interference cases.

Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

PDSCH requirements with inter cell interference

Propagation conditions

In RAN4#112bis a starting point was agreed for the propagation conditions (see [1]):

Propagation conditions Agreement: As a starting point, use: - TDLC300-100 for 2 interfering cells with rank 1 (HomNet) - TDLA30-10 for all other configurations.

As all our initial simulation results (see [5]) show it is feasible to define requirements with the agreed configuration above, hence we support to continue the evaluation with the agreed configuration.

Our simulation results show it is feasible to define requirements using the agreed starting point from RAN4#112bis.

Define PDSCH requirements with inter-cell interference using the following:
- TDLC300-100 for 2 interfering cells with rank 1 (HomNet)
- TDLA30-10 for all other configurations.

Rank of serving cell

In RAN4#112bis it was agreed to evaluate rank 1,2 and 4 in the initial analysis (see [1]):

Rank of serving cell Agreement - Evaluate rank 1,2 & 4 in initial simulation analysis. - TBD: Down-selection for requirement definition.

Based on our simulation results (see [5]), we see it feasible to define requirements for rank 1,2 and 4. As it is possible for a 8Rx only UE to exist, we believe RAN4 should not down-select from the current evaluated ranks for defining requirements.

Our simulation results show it feasible to define requirements for rank 1,2 and 4. To cover 8Rx only UEs all 3 rank configurations should be used for requirement definition (i.e. no down-selection are to be done)

Define PDSCH requirements with inter-cell interference using rank 1, 2 and 4.

MCS of serving cell

In RAN4#112bis the MCS for different rank configuration was agreed to be evaluated (see [1]):

MCS of serving cell Agreement: Evaluate the following configurations for initial analysis: - Rank 1: MCS17 - Rank 2: MCS17 - Rank 4: MCS13 In case one of the above configurations is found not feasible consider alternative MCS. Interrested companies can provide simulations results with: - Rank 1: MCS19 - Rank 2: MCS13 / MCS19

Based on our simulation results (see [5]) we have seen that rank 1 with MCS17 results in SNR values which are 2.5dB and 4.9dB for MMCS-IRC with 1 and 2 interference cell cases respectively, however we do not see this as a limiting factor for requirement definition as impairment and margin will be added.

Our simulations have shown SNR values for the initial analysis configurations which we see as feasible for requirement definition.

Define PDSCH requirements with inter-cell interference using the following configurations:
- Rank 1: MCS17
- Rank 2: MCS17
- Rank 4: MCS13

Other parameters

In RAN4#112bis it was discussed how to configure parameters not specifically discussed in the WID. A number of agreements were made but a few proposals were still open (see [1]):

Other parameters Agreement - Use 2Tx for interreference cell. - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration as Rel-17 - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells Remaining proposals - Option 1: - For PDSCH configuration, DMRS configuration, TRS configuration, HARQ, precoding, Tx EVM, Test metric and other parameters, reuse the configuration from 8Rx requirement as the baseline. - For PCI and SSB configuration of serving cell and interference cell(s), Rank/MCS, precoding, timing and frequency offset, and other interference cell modeling parameters, reuse the configuration from R17 2/4Rx inter-cell interference suppressing requirement as the baseline. - Option 2: For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

As we already mentioned in RAN4#112bis we support that for not specifically agreed parameters in this WID to re-use the same configuration for Rel-18 2/4Rx for MMSE-IRC.

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as baseline.

PDSCH requirements with intra cell inter user interference

Propagation condition

In RAN4#112bis the following was agreed for the propagation conditions (see [1]) with the option to reconsider TDLC300-100 for rank 1+1:

Propagation condition Agreement: - 1+1: TDLA30-10. Reconsider TDLC300-100 if TDLA30-10 found not feasible - 2+2: TDLA30-10 - 4+4: TDLA30-10

In our simulation results [5] we the lowest SNR is 5.2dB, hence we see requirements with TDLA30-10 as feasible.

Our simulation results show that requirements with TDLA30-10 is feasible, hence no need to reconsider TDLC300-100 for requirement definition.

Use the already agreed propagation condition of TDLA30-10 for all rank combinations (1+1, 2+2 and 4+4)

Precoding method for the co-scheduled UE

In RAN4#112bis the precoding method for the co-scheduled UE was discussed. Agreements were made for rank 2+2 and 4+4, however it is still pending which precoding method to use for Rank 1+1 (see [1]):

Precoding method for the co-scheduled UE Agreement: For initial simulation alignment use: - Rank 1+1: random and orthogonal. - Rank 2+2: orthogonal. - Rank 4+4: orthogonal. For orthogonal precoding, select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

In our simulation results [5] we see using orthogonal precoding for rank 1+1 to have a gain with MMSE-IRC compared to MMSE of only 0.5dB, which is not enough for requirement definition. We have not yet provided results for MMSE with random precoder, however we expect the gain to be higher, hence we prefer to use random precoder with Rank 1+1.

Our simulations show the gain from MMSE to MMSE-IRC is 0.5 dB only for orthogonal precoder with Rank 1+1, hence we prefer to use random precoder for Rank 1+1 requirements.

For requirement definition use the following precoding methods:
- Rank 1+1: random.
- Rank 2+2: orthogonal.
- Rank 4+4: orthogonal.

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

In RAN4#112bis agreements were made for Antenna, rank configuration and MIMO correlation for initial analysis. It was kept open if down-selection of the agreed configurations shall be made (see [1]):

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation Agreement: For initial simulation analysis: - Rank 1+1 for 2T8R ULA Low - Rank 2+2 for 4T8R ULA Low - Rank 4+4 for 8T8R ULA Low - Note: Will require DMRS len=2, hence optional feature, which means if introduced requirements would also be needed for one mandatory configuration (i.e. either 1+1 or 2+2) - Note: TBD: Down-selection of configurations.

In existing 4Rx PDSCH requirements with intra-cell inter-user interference, requirements have been defined for 1+1 and 2+2 rank configurations. As it is possible for 8Rx only UEs to exist, we see the need to define requirements for 8Rx with 1+1, 2+2 and 4+4 without doing down-selection.

In existing requirements for 4Rx requirements are defined for rank 1+1 and 2+2. For 8Rx, rank 4+4 is now possible, hence requirements should in addition be defined with rank 4+4.

Define PDSCH requirements with intra-cell inter user interference with:
- Rank 1+1 for 2T8R ULA Low
- Rank 2+2 for 4T8R ULA Low
- Rank 4+4 for 8T8R ULA Low

MCS

In RAN4#112bis the MCS for different rank configuration was agreed to be evaluated (see [1]):

MCS Agreement: Evaluate: - Rank 1+1: MCS13 - Rank 2+2: MCS13 - Rank 4+4: MCS13 as starting point. Interrested companies can provide results for - Rank 1+1: MCS17 - Rank 2+2: MCS19

Based on our simulation results (see [5]) we have seen that rank 1+1 with MCS13 results in SNR values which are 5.2dB, however we do not see this as a limiting factor for requirement definition as impairment and margin will be added.

Our simulations have shown SNR values for the initial analysis configurations which we see as feasible for requirement definition.

For PDSCH requirement definition with intra-cell inter-user interference use the following configurations:
- Rank 1+1: MCS13
- Rank 2+2: MCS13
- Rank 4+4: MCS13

Other parameters

In RAN4#112bis it was discussed how to configure parameters not specifically discussed in the WID. (see [1]):

Other parameters Proposals - Option 1: For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation. - Other options not precluded

As we already mentioned in RAN4#112bis we support that for not specifically agreed parameters in this WID to re-use the same configuration for Rel-18 2/4Rx for MMSE-IRC.

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as baseline.

Wideband CQI reporting requirements with inter-cell interference

In RAN4#112bis it was agreed to introduce CQI reporting requirements with inter-cell interference (see [1]), however further discussion on simulation parameters was postponed to RAN4#113 due to limited time:

Introduction of CQI requirements Agreement: - Cover CQI reporting requirements under ICI for 8Rx.

Propagation condition

In RAN4#112bis the propagation conditions were brought up. Especially if fading channel conditions should be included for requirement definition (see [1]):

Propagation condition Proposals - Option 1: Reuse the same configuration for Rel-17 2/4Rx: - TDLA30-5 for the serving cell - AWGN for the interference cell. - Option 2: Further study whether to also include fading channel conditions. - Option 3: CQI requirements under fading channel (If time allows) - Option 4: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline - Other options not precluded

Fading channel was used in Rel-17 CQI requirements for inter-cell interference. No CQI requirements were defined for inter-cell interference for 8Rx in Rel-18 and we could not fully re-constrict the reason for including AWGN and not including fading channel conditions.

We could not fully re-construct the reason for not including fading channel conditions and it does not seem to be captured in the meeting notes.

Further study whether to also include fading channel conditions. In case it is decided to not include fading channel conditions then reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

CQI calculation algorithm

In RAN4#112bis the CQI calculation algorithm was brought up (see [1])

CQI calculation algorithm Proposals - Option 1: Define CQI reporting requirements as below: - Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation - Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation. - Option 2: RAN4 to define CQI requirements for two receiver types: - Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. - Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. - Option 3: Only define one CQI calculation assumption to match both receiver types. - Option 4: Introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed. - Option 5: RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for one requirement. Otherwise, RAN4 can define two sets of requirements for different type of receiver. - Other options not precluded.

In Rel-18 CQI requirements were only defined for 8Rx AWGN case without any distinction between simplified or baseline receiver, hence there was found to be no need to distinguish between CQI calculation assumptions.

We support only introducing one CQI calculation assumption independent of the receiver types unless it is found that introduction of two CQI calculation assumptions is needed possible if agreed to introduce CQI requirements for fading conditions.

Introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed. (Option 2)

Test metric

In RAN4#112bis the test metric was brought up (see [1])

Test metric Proposals - Option 1: Reuse the same test metric for Rel-17 2/4Rx: a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.. - Other options no precluded

For 8Rx we do not see any reason to change the test metric from Rel-17, hence we are fine with reusing the same test metric from Rel-17 2/4Rx.

As a starting point for initial evaluation, reuse the same test metric as for Rel-17 2/4Rx (Option 1)

Tested SINR value, throughput ratio γ and BLER X%

In RAN4#112bis the SINR value, throughput ratio γ and BLER X% was brought up (see [1])

Tested SINR value, throughput ratio γ and BLER X% Proposals - Option 1: The tested SINR value shall be decided based on simulation results with reasonable performance gain. - Other options not precluded.

It has been the usual practice in previous requirement definitions to decide the SINR, throughput ration and BLER based on companies’ simulation results.

Decide the tested SINR value, throughput ration and BLER based on simulation results.

Rank for the serving cell

In RAN4#112bis the rank was brought up (see [1])

Rank for the serving cell Proposals - Option 1: Rank 1 - Option 2: Rank 2 - Option 3: Rank 4 - Option 4: Cover the following for initial study and possible down select based on simulation results: - Rank 1, 2 & 4 - Other options not precluded.

Existing CQI requirements for 8Rx is defined using Rank 4 (Table A.4-3, TBS.3-2 in [3]), hence we see no reason to not use rank 4 as a starting point also for 8Rx with intercell interference.

Overall rank2 is minimum to not trivialize the CQI selection (i.e. we need two different streams to be captured in one value).

We see Rank2 as minimum to not trivialize the CQI selection to only capture one stream.

If 4T8R configuration is agreed, use Rank 4 as a starting point for initial evaluation of CQI requirements. If 2T8R configuration is agreed, use Rank 2.

If decided to define requirements for Rank 4 then also define requirements for Rank 2.

Antenna configuration and MIMO correlation for the serving cell

In RAN4#112bis the Antenna configuration and MIMO correlation for the serving cell was brought up (see [1])

Antenna configuration and MIMO correlation for the serving cell Proposals - Option 1: 2T8R - Option 1a: - 2T8R with ULA Low - Option 1b: - 2T8R with rank 1 - Option 2: 4T8R - Option 2a: 4T8R with ULA Low, Rank 4 - Option 2b: 4T8R with Rank 4 - Other options not precluded.

Since existing 8Rx CQI requirements are defined with 4T8R, the definition of CQI requirements for 8Rx with inter-cell interference shall also be using antenna configuration 4T8R for the serving cell unless initial evaluation shows it not feasible to define requirements for 4T8R. As such, we see no reason to change the baseline from the prior 8Rx CQI requirements.

Existing CQI requirements for 8Rx is defined in a 4T8R antenna configuration and we see no reason changing the baseline from the prior 8Rx requirements.

Use 4T8R ULA Low antenna configuration and rank 4 for the serving cell (Option 2a) as a starting point

CSI-IM and NZP CSI-RS configurations

In RAN4#112bis the CSI-IM and NZP CSI-RS configurations was brought up (see [1])

CSI-IM and NZP CSI-RS configurations Proposals - Option 1: Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation: - CSI-IM on serving cell overlapping with PDSCH from interference cell - NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell - Other options not precluded

We are fine to re-use the Rel-17 configurations for further simulation alignment.

Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation (option 1): 
- CSI-IM on serving cell overlapping with PDSCH from interference cell
- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell.

Other parameters

In RAN4#112bis it was brought up what to use for parameters not specifically defined (see [1])

Issue 1-4-10: Other parameters Proposals - Option 1: Re-use the same configuration for Rel-17 2/4Rx - Option 2: For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation. - Other options not precluded.

For 8Rx we do not see an issue using the existing Rel-17 configurations as baseline with modifications needed to support 8Rx (i.e. configuration parameters not specifically dictated by this WID can be re-used from existing Rel-17 configurations)

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation. (Option 2)

Interference power level

In RAN4#112bis the interference power level was brought up (see [1])

Issue 1-4-11: Interference power level Proposals - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1 - Other options not precluded.

For initial simulation analysis, we are fine with using the legacy interference power level which was introduced for the Rel-17 2/4Rx CQI intercell interference requirements (i.e. INR = 10.04dB)

Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB

Applicability rule and release independent

Test applicability

The question of test applicability was brought up in RAN4#112bis (see [1]):

Test applicability Proposals - Option 1: The new requirements for 8Rx MMSE-IRC receiver shall be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined. - Option 4: - The new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs - If agreed to define two set of requirements (baseline and simplified receiver), introduce new applicability rules based on Table 5.1.1.3-1 of TS38.101-4. - Option 5: Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed regardless the concrete test setup. - Other options not precluded.

At this point it is not possible to determine if future 8Rx requirements will fully cover the same UE functionality as existing 2Rx/4Rx requirements, hence we do not at this time see it reasonable to discuss test applicability. Our view at this point is that the new requirements should be mandatory requirements for all 8Rx UEs without additional test applicability rules. However, the discussion on test applicability can be postponed until it is more clear, which requirements are to be defined.

Baseline assumption: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule (option 1)

Release independency

The question of release independency was brought up in RAN4#112bis (see [1]):

Release independency Proposals - Option 1: The new requirements for 8Rx should be release independent from Rel-18 - Option 2: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics. - Option 4: Discuss release independence requirements at a later stage in the WI. - Other options not precluded.

Since there are already 8Rx requirements defined in Rel.18 and it is expected that a Rel.18 UE will perform equal in interference scenarios to a Rel-19 UE, we see no issue for new requirements for 8Rx to also be mandatory from Rel-18. Still we would like to postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

Conclusion

In this paper we have provided our view on the topic of requirement definition for 8Rx inter-cell and intra-cell inter-user requirements.

The following Observations and Proposals were made:

General

- If the span after removing a reasonable number of outliers is <=2.5dB, only one set of requirements can be defined to cover both receivers. If the span after removing a reasonable number of outliers is >2.5dB it shall be considered to define two sets of requirements.

- Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition.

The existing definition of baseline and simplified receivers in 38.101-4 is focused on SU-MIMO configuration. This needs to be changed to also cover the discussed interference cases.

Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

PDSCH requirements with inter cell interference

Propagation conditions

Our simulation results show it is feasible to define requirements using the agreed starting point from RAN4#112bis.

Define PDSCH requirements with inter-cell interference using the following:
- TDLC300-100 for 2 interfering cells with rank 1 (HomNet)
- TDLA30-10 for all other configurations.

Rank of serving cell

Our simulation results show it feasible to define requirements for rank 1,2 and 4. To cover 8Rx only UEs all 3 rank configurations should be used for requirement definition (i.e. no down-selection are to be done)

Define PDSCH requirements with inter-cell interference using rank 1, 2 and 4.

MCS of serving cell

Our simulations have shown SNR values for the initial analysis configurations which we see as feasible for requirement definition.

Define PDSCH requirements with inter-cell interference using the following configurations:
- Rank 1: MCS17
- Rank 2: MCS17
- Rank 4: MCS13

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as baseline.

PDSCH requirements with intra cell inter user interference

Propagation condition

Our simulation results show that requirements with TDLA30-10 is feasible, hence no need to reconsider TDLC300-100 for requirement definition.

Use the already agreed propagation condition of TDLA30-10 for all rank combinations (1+1, 2+2 and 4+4)

Precoding method for the co-scheduled UE

Our simulations show the gain from MMSE to MMSE-IRC is 0.5 dB only for orthogonal precoder with Rank 1+1, hence we prefer to use random precoder for Rank 1+1 requirements.

For requirement definition use the following precoding methods:
- Rank 1+1: random.
- Rank 2+2: orthogonal.
- Rank 4+4: orthogonal.

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

In existing requirements for 4Rx requirements are defined for rank 1+1 and 2+2. For 8Rx, rank 4+4 is now possible, hence requirements should in addition be defined with rank 4+4.

Define PDSCH requirements with intra-cell inter user interference with:
- Rank 1+1 for 2T8R ULA Low
- Rank 2+2 for 4T8R ULA Low
- Rank 4+4 for 8T8R ULA Low

MCS

Our simulations have shown SNR values for the initial analysis configurations which we see as feasible for requirement definition.

For PDSCH requirement definition with intra-cell inter-user interference use the following configurations:
- Rank 1+1: MCS13
- Rank 2+2: MCS13
- Rank 4+4: MCS13

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as baseline.

Wideband CQI reporting requirements with inter-cell interference

Propagation condition

We could not fully re-construct the reason for not including fading channel conditions and it does not seem to be captured in the meeting notes.

Further study whether to also include fading channel conditions. In case it is decided to not include fading channel conditions then reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

CQI calculation algorithm

Introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed. (Option 2)

Test metric

As a starting point for initial evaluation, reuse the same test metric as for Rel-17 2/4Rx (Option 1)

Tested SINR value, throughput ratio γ and BLER X%

Decide the tested SINR value, throughput ration and BLER based on simulation results.

Rank for the serving cell

We see Rank2 as minimum to not trivialize the CQI selection to only capture one stream.

If 4T8R configuration is agreed, use Rank 4 as a starting point for initial evaluation of CQI requirements. If 2T8R configuration is agreed, use Rank 2.

If decided to define requirements for Rank 4 then also define requirements for Rank 2.

Antenna configuration and MIMO correlation for the serving cell

Existing CQI requirements for 8Rx is defined in a 4T8R antenna configuration and we see no reason changing the baseline from the prior 8Rx requirements.

Use 4T8R ULA Low antenna configuration and rank 4 for the serving cell (Option 2a) as a starting point

CSI-IM and NZP CSI-RS configurations

Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation (option 1): 
- CSI-IM on serving cell overlapping with PDSCH from interference cell
- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell.

Other parameters

For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC (Option 1) as a starting point for initial evaluation. (Option 2)

Interference power level

Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB

Applicability rule and release independent

Test applicability

Baseline assumption: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule (option 1)

Release independency

Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

- References

- R4-2416555 - WF for [112bis][320] NR_demod_Ph5_Part2_UE

- TS38.212 - Multiplexing and channel coding (Release 18)

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- RP-241297, “NR demodulation performance: Phase 5”, RAN#104, June 2024

- R4-2417974 - Simulations for UE demodulation performance req. for 8Rx with MMSE-IRC