3GPP TSG-RAN WG4 Meeting #113	R4-2418001

Orlando, USA, 18-22 Nov 2024

Title: 	On MPR applicability for FR1 intra-band UL CA

Agenda Item:	7.1.1.2.2

Source: 	Nokia

Document for:	Approval

# 1				Introduction

In previous RAN4 meeting power domain enhancements topic approved WF[1], this contribution discussed open issues of Topic #2: MPR applicability for FR1 intra-band UL CA.

# 2	Discussion

## 2.1	Open issues in WF

For the open issue

#### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE

- Proposals

  - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication.

  - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated.

- WF

  - FFS in next meeting

We support proposal 1, which is depicted in Figure 1 as we understand it.

Figure 1: Bandwidth for determining OOB emission requirements

RAN4 has agreed that no interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intra-band contiguous UL CA which means that LO retuning is not possible. Hence in case of UE supporting dualPA-Architecture then incase of single activated CC the LO stays in the middle of remaining CC and incase UE doesn’t indicate support for dualPA-Architecture then LO remains between single activated CC and de-activated CC.

The placement of LO will determine the correct way of defining the OOB requirements, hence we are proposing

Proposal 1: If UE indicates support for dualPA-Architecture then when only one CC is activated the OOB emission requirements are based on single carrier requirements. If UE doesn’t indicate support for dualPA-Architecture then when only one CC is activated the OOB emission requirements are based on UL CA requirements, as in Figure 1.

Another open issue was

#### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell

- Proposals

  - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell.

  - Proposal 2:

    - For UE supporting contiguous ULCA with the dualPA IE:

      - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively.

    - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support:

      - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA

  - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported

  - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

- WF

  - FFS in next meeting

Which is very much related to Issue 2-1-2. From these options for issue 2-1-3 we support Proposal 2, which is part of our proposal 1 in this TDoc.

## 2.2	MPR applicability for FR1 intra-band UL CA with one or more than activated cell

In RAN4#112 meeting [1], the applicable MPR for intra-band contiguous CA with single activated cell is discussed.

In case of single CC with activated cell, the following MPR requirements should be applied for PC3/PC2 intra-band contiguous carrier aggregation:

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

However, more than 1 cell may be activated and thus the single carrier MPR in Table 6.2.2-1 and Table 6.2.2-2 won’t be applied in all possible cases based on the proposals in [1]. For example, if more than 1 cell are active but UE doesn’t have time-overlapping UL transmissions among the activated cells, UE can also use lower MPR, e.g., based on the aforementioned MPR tables and conditions. So, these MPR applicability can be further enhanced by considering the deterministic and/or actual time non-overlapping UL transmissions.

Proposal 2: MPR applicability and enhancement in the cases of more than one active cell with time non-overlapping UL transmission should be considered also in Rel-19 if time allows.

In all cases of CA MPR definition based on one or more activated cells, the impact on power headroom report (PHR) should be discussed to provide the network the required assistance information. Without further enhancement of PHR, NW would need to analyse achievable UL transmit power or MPR gains by (de)activating secondary cells with the related slow MAC-CE. Hence, a ping-pong issue due to (de)activation of secondary cells and a large latency would occur before identifying the suitable cell(s) activation status and UL configuration, and these limitations should be avoided.

Observation: For MPR applicability based on active cells for intra-band UL CA, RAN4 to discuss if there is a need for PHR enhancements as assistance information for the network.

# 3	Conclusion

In this contribution we have made following proposal

Proposal 1: If UE indicates support for dualPA-Architecture then when only one CC is activated the OOB emission requirements are based on single carrier requirements. If UE doesn’t indicate support for dualPA-Architecture then when only one CC is activated the OOB emission requirements are based on UL CA requirements, as in Figure 1.

Proposal 2: MPR applicability and enhancement in the cases of more than one active cell with time non-overlapping UL transmission should be considered also in Rel-19 if time allows.

Observation: For MPR applicability based on active cells for intra-band UL CA, RAN4 to discuss if there is a need for PHR enhancements as assistance information for the network.

# 4	References

[1] R4-2417204, WF on power domain enhancements, RAN4#112bis